3GPP TSG-RAN WG4 #82bis Meeting	R4-1703559

Spokane, USA, 3 - 7 April 2017

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Conducted UE ACLR for frequency range 1

Agenda item:		10.4.1.1

Document for:		Discussion

# 1	Introduction

In RAN#75 meeting NR WI was approved [1] and the work guideline for RAN4 was presented in [2] to indicate the urgent issues sorted out in RAN4 such as ACLR/ACS.

During the NR study item phase, the following agreement was made for UE ACLR for the frequency range 1, i.e., below 6GHz [3], though the threshold frequency has not been finalized yet.

- For Range 1

  - It was agreed that NR ACLR requirements for UTRA, E-UTRA and NR need to be specified in the WI phase.

In this paper, we discuss the conducted ACLR requirement on for Range 1.

# 2	Discussion

## 2.1 NR ACLR

E-UTRA UE ACLR is specified in TS36.101 in the following.

Table 2.1-1: TS36.101 Table 6.6.2.3.1-1: General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +1.4 / -1.4 | +3.0 / -3.0 | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |

Although the coexistence of NR-NR has not been studied in the NR study phase for the frequency range 1, the similar conducted requirement to E-UTRA ACLR should be applied assuming the deployment scenario of the NR and E-UTRA are similar to the analysis in TR36.942 for the existing frequency bands (in Range 1). It is also assumed that there is no advanced beam forming technology for the conducted requirement. In that case, the coexistence requirement in TS36.101 should be mostly reused for the NR. For the same channel bandwidth as E-UTRA, the ACLR can be specified 30 dB. For the wider channel bandwidth than 20MHz, CA ACLR is specified in TS36.101, which is also 30 dB. Even though the UE uplink maximum contiguous bandwidth is at most 40 MHz in the current version of TS36.101, the same ACLR for the wider channel bandwidth can be assumed in the NR.

Observation 1: The conducted NR ACLR level can be specified 30 dB for Range 1.

Note that different deployment scenarios from TR36.942 such as advanced beam forming based system may be covered in OTA requirement, where the required ACLR level needs to be further investigated.

Another difference of the NR is the spectrum utilization better than E-UTRA utilization of 90%. Thus, to properly secure the adjacent channel coexistence for the worst-case NR-NR scenario, i.e., two channels are next to each other, the measurement bandwidth should be based on the maximum allowed transmission bandwidth of the NR for that channel bandwidth. The transmission bandwidth depends on the subcarrier spacing and the number of physical resource blocks in the uplink, which can be more than 90% of the NR channel bandwidth. The maximum transmission bandwidth among all the possible numerologies should be selected for the worst case.

Observation 2: NR ACLR can be defined with the measurement channel bandwidth aligned with the maximum transmission bandwidth of the NR uplink.

In summary, the NR ACLR possible specification is represented in Table 2.1-2 for the channel bandwidth between 5MHz to 100MHz.

Note that the minimum and maximum channel bandwidth for the NR is not agreed yet. This table is only for the purpose to illustrate the possible specifications.

Table 2.1-2: Example requirements for NRACLR

|  | NR Channel bandwidth / NRACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz |
| NRACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| NR channel Measurement bandwidth | Max Tx BW for 5MHz | Max Tx BW for 10MHz | Max Tx BW for 15MHz | Max Tx BW for 20MHz | Max Tx BW for 50MHz | Max Tx BW for 100MHz |
| Adjacent channel centre frequency offset [MHz] | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 | +50 / -50 | +100 / -100 |
| Max Tx BW is the maximum transmission bandwidth allowed for each NR channel bandwidth. |  |  |  |  |  |  |

For the flexible channel bandwidth [4], it is possible interpolate the requirement, i.e., ACLR is still 30dB for any channel bandwidth. The channel measurement bandwidth shall be the maximum transmission bandwidth allowed for each channel bandwidth among all possible numerologies. The frequency offset shall be the same as the channel bandwidth.

The ACLR requirement so far guarantees only the basic channel deployment scenarios that the channels next to each other is of the same channel bandwidth. So far in E-UTRA, it is not the intention of ACLR requirement to support all different deployment like asymmetric channel bandwidth. Thus, it is not so essential in our view to introduce ACLR requirement for the flexible channel bandwidth. In any case for the flexible channel bandwidth, the adjacent channel would be also flexible and may not be the same in most deployment. Therefore, it would be sufficient to introduce the selected set of channel bandwidths for the basic channel deployment scenarios.

Considering the possible regulatory requirement and transmitter RF characterization, we may need to specify the spectrum emission mask and spurious emission for the flexible channel bandwidth. This is further discussed in [5]. On the contrary, ACLR is to characterize the coexistence and may not be so crucial to introduce it for flexible channel bandwidth.

Observation 3: Although it is possible to introduce ACLR requirement for flexible channel bandwidth based on the maximum transmission bandwidth allowed for each channel bandwidth, it would not be essential to introduce ACLR requirement for flexible channel bandwidth.

## 2.2 E-UTRA ACLR

To secure the NR coexistence with E-UTRA, the E-UTRA ACLR shall be used in the same way as Table 2.1-1 for the same channel bandwidth of NR and E-UTRA.

NR channel bandwidth can be much wider than E-UTRA. However, E-UTRA can already support wider bandwidth than 20MHz through the carrier aggregation. In case of E-UTRA carrier aggregation, the ACLR is defined for the same aggregated channel bandwidth with 30 dB. Although the current TS36.101 spec only supports at most bandwidth class C for uplink, i.e., 40MHz, 30 dB would be reused for wider aggregated bandwidth than 40MHz. Thus, ACLR requirement for the same bandwidth between NR and E-UTRA can be specified according to Table 2.1-2. For simplicity, 90% channel bandwidth is assumed as a measurement bandwidth rather than using the CA bandwidth.

Note that the minimum and maximum bandwidth for the NR is not agreed yet. This table is only for the purpose to illustrate the possible specifications.

Table 2.1-2: Example requirements for E-UTRAACLR

|  | NR Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz | 45 MHz | 90 MHz |
| Adjacent channel centre frequency offset [MHz] | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 | +50 / -50 | +100 / -100 |

This requirement is more relaxed than Table 2.1-1, thus it is unnecessary to verify.

Observation 4: For the same channel bandwidth, NR ACLR is always tighter than E-UTRA ACLR, therefore it is not necessary to verify E-UTRA ACLR.

For the coexistence of NR channel with a narrower E-UTRA channel, although such scenario is not defined in E-UTRA UE requirement among various E-UTRA bandwidths, it is possible to consider the coexistence of wider NR channel and narrower E-UTRA channel next each other. However, it is questionable to introduce such requirement for every E-UTRA channel bandwidth because the wide NR channel is only possible if the large block is allocated to operators. It is not a likely scenario or recommended scenario that 100MHz NR channel is right next to 1.4MHz E-UTRA channel. Most likely it is enough to include only wide E-UTRA channels (like 10MHz or wider) for the coexistence of unequal channel bandwidth between NR and E-UTRA.

Observation 5: Necessity of ACLR requirement for the coexistence with a narrower E-UTRA channel bandwidth should be discussed further.

## 2.2 UTRA ACLR

UTRA UE ACLR is specified in TS36.101 in the following.

Table 2.2-1: TS36.101 Table 6.6.2.3.2-1: Requirements for UTRAACLR1/2

|  | Channel bandwidth / UTRAACLR1/2 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB |
| Adjacent channel centre frequency offset [MHz] | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | +2.5+BWUTRA/2 / -2.5-BWUTRA/2 | +5+BWUTRA/2 / -5-BWUTRA/2 | +7.5+BWUTRA/2 / -7.5-BWUTRA/2 | +10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 36 dB | 36 dB | 36 dB | 36 dB |
| Adjacent channel centre frequency offset [MHz] | - | - | +2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | +5+3*BWUTRA/2 / -5-3*BWUTRA/2 | +7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | +10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth (NOTE 1) | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth (NOTE 2) | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28MHz | 1.28MHz | 1.28MHz |
| NOTE 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. NOTE 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |  |  |  |  |  |

This requirement secures the coexistence with two consecutive UTRA channels right next to the E-UTRA channel. To introduce the similar requirement for the NR next to UTRA channels, the table can be revised in the following Table 2.2-2 for the NR channel bandwidth between 5 MHz and 100 MHz.

Note that the minimum and maximum bandwidth for the NR is not agreed yet. This table is only for the purpose to illustrate the possible specifications.

Table 2.2-2: Example requirements for UTRAACLR1/2

|  | NR Channel bandwidth / UTRAACLR1/2 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz |
| UTRAACLR1 | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB |
| Adjacent channel centre frequency offset [MHz] | +2.5+BWUTRA/2 / -2.5-BWUTRA/2 | +5+BWUTRA/2 / -5-BWUTRA/2 | +7.5+BWUTRA/2 / -7.5-BWUTRA/2 | +10+BWUTRA/2 / -10-BWUTRA/2 | +25+BWUTRA/2 / -25-BWUTRA/2 | +50+BWUTRA/2 / -50-BWUTRA/2 |
| UTRAACLR2 | 36 dB | 36 dB | 36 dB | 36 dB | 36 dB | 36 dB |
| Adjacent channel centre frequency offset [MHz] | +2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | +5+3*BWUTRA/2 / -5-3*BWUTRA/2 | +7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | +10+3*BWUTRA/2 / -10-3*BWUTRA/2 | +25+3*BWUTRA/2 / -7.5-25*BWUTRA/2 | +50+3*BWUTRA/2 / -50-3*BWUTRA/2 |
| NR channel Measurement bandwidth | Based on Tx BW config | Based on Tx BW config | Based on Tx BW config | Based on Tx BW config | Based on Tx BW config | Based on Tx BW config |
| UTRA 5MHz channel Measurement bandwidth (NOTE 1) | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth (NOTE 2) | 1.28 MHz | 1.28MHz | 1.28MHz | 1.28MHz | 1.28MHz | 1.28MHz |
| NOTE 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. NOTE 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. NOTE 3: NR channel measurement shall be based on the transmission bandwidth configuration of the NR channel. |  |  |  |  |  |  |

The requirement for 50MHz or 100MHz channel in the above table will not be applicable for the bands with narrower pass band bandwidth. Furthermore, such a band with large pass band, UTRA channel deployment is not expected. Thus, it will be sufficient to introduce UTRA ACLR for the bands if UTRA operating band is specified in the same frequency range in TS25.101.

Observation 6: The UTRA ACLR is not necessary for the band if UTRA operating band is not specified in TS25.101.

# 3	Conclusions

In this contribution, we have discussed the conducted ACLR requirement for Range 1, i.e., the frequency below [6] GHz. We concluded the following obvervations.

Observation 1: The conducted NR ACLR level can be specified 30 dB for Range 1.

Observation 2: NR ACLR can be defined with the measurement channel bandwidth aligned with the maximum transmission bandwidth of the NR uplink.

Observation 3: Although it is possible to introduce ACLR requirement for flexible channel bandwidth based on the maximum transmission bandwidth allowed for each channel bandwidth, it would not be essential to introduce ACLR requirement for flexible channel bandwidth.

Observation 4: For the same channel bandwidth, NR ACLR is always tighter than E-UTRA ACLR, therefore it is not necessary to verify E-UTRA ACLR.

Observation 5: Necessity of ACLR requirement for the coexistence with a narrower E-UTRA channel bandwidth should be discussed further.

Observation 6: The UTRA ACLR is not necessary for the band if UTRA operating band is not specified in TS25.101.

# 4	References

- RP-170847		New WID on New Radio Access Technology, NTT DOCOMO, INC.

- RP-170754		Guideline for RAN4 work for NR bands/band combinations, NTT DOCOMO, INC., Qualcomm, Intel

- TR38.803		Study on New Radio Access Technology; RF and co-existence aspects (Release 14)

- R4-1702091	WF on UE RF requirements scalability for flexible channel bandwidth consideration

- R4-1703562	UE conducted transmitter requirement for flexible channel bandwidth for frequency range 1, Nokia, Alcatel-Lucent Shanghai Bell