3GPP TSG-RAN WG4 Meeting #82bis	R4-1703994

Spokane, WA, USA, 3rd – 7th April, 2017

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Hybrid requirements set(s)

Agenda item:	9.12.2

Document for:	Approval

# 1	Introduction

During the RAN4 #82 meeting, a WF on hybrid requirements [1] was approved. In the WF, the following open issues were captured:

- To consider fixed list and/or flexible additional Hybrid requirements set with mapping of requirements in either the conducted domain or the OTA domain but not both

  - Example of a fixed Hybrid requirement is a set which include all conducted and radiated requirements as is currently specified in Rel-13 AAS specifications

  - A flexible Hybrid requirement will include a minimum sub-set of OTA requirements (FFS) and vendors can then define their own remaining requirements in either the conducted or radiated domains

- For some requirements, either OTA or conducted, have dependency on other requirements. As an example, the OTA blocking requirements depends on OTA sensitivity.

  - In such case if a requirement is selected to be included in a domain, e.g. either OTA or Conducted, all dependent requirements for that requirements shall be maintained in its corresponding requirement domain.

This document addresses the aforementioned open issue for FFS and concludes with our observation and proposal.

# 2	Discussion

The agreements captured in the WF [1] are:

- It is agreed to include in AAS Rel-14 specifications a set of OTA requirements as per the current Scope of the WID and additional set of one or more Hybrid requirements.

- The OTA requirements set shall apply to OTA AAS BS which does not have conducted interface (i.e. access to connectors are not available for conducted testing).

- A hybrid AAS BS, i.e. an AAS BS with conducted interface and OTA interface (i.e. access to connectors available for conducted testing), either the OTA requirements set or any (if more than one is specified) of the hybrid requirements set(s) can be applied.

- All the requirements (conducted and the two OTA requirements) in AAS Rel-13 specifications (TS 37.105-1 and -2) are considered as one of the hybrid requirements sets.

- Additional hybrid requirements set(s) either fixed, e.g. Rel-13 AAS requirements as indicated above, or flexible set in which mapping of requirements to either conducted or OTA domains are not fixed but rather declared by vendors will be further investigated with open issues as identified in the following slide.

At the minimum, the hybrid requirements set comprises the Rel-13 AAS requirements set only. As one or more of the existing conducted requirements in the Rel-13 AAS group are converted to OTA requirement, new hybrid requirements sets are derived as shown in Figure 1. The last hybrid requirements set consists of all OTA requirements.

It is important to note that the Rel-13 AAS requirements set provides the minimum number of OTA requirements as compared with other hybrid requirements sets.

Observation 1: the existing Rel-13 AAS requirements set provides the minimum number of OTA requirements as compared with other hybrid requirement sets.

All OTA AAS requirements set

Additional hybrid AAS requirements set

…

…

Rel-13 AAS requirements set

Increasing number of OTA requirements

Figure 1: Hybrid requirements set(s)

Therefore, a flexible hybrid requirements set can be derived from the Rel-13 AAS OTA requirements by including its “Radiated transmit power (EIRP accuracy)” and “OTA sensitivity” as the minimum subset of OTA requirements. The remaining requirements are defined as either conducted or radiated by manufacturers.

Proposal 1: the Rel-13 AAS OTA requirements, namely “Radiated transmit power (EIRP accuracy)” and “OTA sensitivity”, constitute the minimum subset of OTA requirements in any flexible hybrid requirements sets.

# 3	Conclusions

In this document, we have addressed the open issues of hybrid requirements and we can make the following observation and proposal:

Observation 1: the existing Rel-13 AAS requirements set provides the minimum number of OTA requirements as compared with other hybrid requirement sets.

Proposal 1: the Rel-13 AAS OTA requirements, namely “Radiated transmit power (EIRP accuracy)” and “OTA sensitivity”, constitute the minimum subset of OTA requirements in any flexible hybrid requirements sets; the remaining requirements are defined as either conducted or radiated by manufacturers.

# References

- R4-1702263, Way Forward on hybrid requirements, NEC, Huawei