3GPP TSG-RAN WG4 Meeting #82bis	R4-1703996

Spokane, USA, April 3rd - 7th, 2017

Agenda item:	10.4.2.2 – UE Transmitter characteristics

Source: 	Qualcomm Incorporated

Title: 	Minimum TRP and dynamic range of mm-wave UE transmitters

Document for:	Discussion

# Introduction

Minimum power is an issue that needs to be defined in the context of phased-array NR radios. This paper analyses minimum total radiated power (TRP) for mm-wave UE’s. TRP is needed for emissions analysis and specification of the performance of PA performance. However, the interpretation of minimum power will be broader than these two criteria.  The purpose of this paper is to start that discussion in RAN4.

For the purpose of illustration, the link parameters in this paper are taken from the sharing studies (cf. [1], [2], and [3]).  Since the parameters are not normative, the objective is to discuss tentative power budgets with an example of QAM-64 in a single CC scenario. More analysis needs to be done in different scenarios.

As a corollary, we also analyze the dynamic power range that is implied by minimum power. Based on the analysis in this paper, the minimum output TRP for a dual-polarized configuration is found to be -13.6 dBm to support QAM-64.  If we assume a peak TRP of 23 dBm as in the sharing studies, this implies a dynamic range of 36.6 dB.

# Discussion

The table below summarizes the analysis. To compute minimum TRP, the obvious method is to select just one antenna of the phased-array, resulting in a wide beam. Hence, the array gain at the UE will be 0 dB.

|  | Value | Comments |
| --- | --- | --- |
| BS noise figure | 9 dB |  |
| Signal BW | 100 MHz | Single CC |
| Thermal noise floor @ BS | -85 dBm | -174 dBm/Hz + 80 dB + 9 dB |
| SNR for QAM-64 + implementation loss | 25 dB |  |
| Beamforming and gain @ BS (with 16x8 element sub-array) | 29 dB | 10log10(128) + 8 dBi per-antenna gain |
| Needed conducted RX power at LNA input | -89 dBm | 25 dB - 85 dBm - 29 dB |
| Power control margin | 10 dB | To equalize RX power at BS from UE’s at different distances |
| Required RX power at BS | -79 dBm |  |
| Friis path loss over 2m @ 28 GHz | 67.4 dB |  |
| Required EIRP @ UE | -11.6 dBm | - 79 dBm + 67.4 dB |
| Single patch antenna gain @ UE | 5 dBi |  |
| Required TRP @ UE for 1 polarization | -16.6 dBm | EIRP – antenna gain |
| Min TRP @ UE for dual pol | -13.6 dBm | + 3dB from above |
| Maximum total conducted power @ UE | 23 dBm | 14 dBm + 10log10(4) + 3dB |
| Dynamic range of total conducted power @ UE | 36.6 dB | 23 – (-13.6) dB |

# Conclusion

With an example of QAM-64, we analyse TRP from the UE perspective. Based on the assumptions, we estimate a minimum power of -13.6 dBm for a dual-polarized UE transmitter and QAM-64 constellation. The associated dynamic range is 36.6 dB. The larger objective of the paper is to start the discussion on minimum power, and its very definition.

# References

- R4-1700042,	“Simulation results for NR coexistence study: indoor deployment at 30GHz, 45GHz and 70GHz”, Qualcomm Incorporated, RAN4-NR#1 Ad Hoc, Spokane, USA, Jan 17th-19th, 2017.

- R4-1700043,	“Simulation results for NR coexistence study: urban macro deployment at 30GHz”, Qualcomm Incorporated, RAN4-NR#1 Ad Hoc, Spokane, USA, Jan 17th-19th, 2017.

- R4-1700044, “Simulation results for NR coexistence study: dense urban deployment at 30GHz, 45GHz and 70GHz”, Qualcomm Incorporated, RAN4-NR#1 Ad Hoc, Spokane, USA, Jan 17th-19th, 2017.

- TS 36.101, “LTE; E-UTRA; User Equipment (UE) radio transmission and reception”, ETSI, Release 14, 2016