3GPP TR 36.714-00-00 V0.6.0 (2017-04)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

LTE Advanced intra-band Carrier Aggregation

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	8

2	References	8

3	Definitions and abbreviations	9

3.1	Definitions	9

3.2	Abbreviations	9

4	Background	9

4.1	TR Maintenance	9

5	Intra-band Carrier Aggregation including contiguous and non-contiguous spectrum: General Part	9

6	Intra-band Carrier Aggregation including contiguous and non-contiguous spectrum: Specific Band Combination Part	10

6.1	CA_66D_BCS0	10

6.1.1	Channel bandwidths per operating band for CA	10

6.1.2	Co-existence studies	10

6.1.3	∆TIB and ∆RIB values	10

6.1.4	REFSENS requirements	10

6.1.5	Blocking requirements	10

6.2	CA_66A-66C_BCS0	11

6.2.1	Channel bandwidths per operating band for CA	11

6.2.2	Co-existence studies	11

6.2.3	∆TIB and ∆RIB values	11

6.2.4	REFSENS requirements	11

6.3	CA_7A-7A_BCS2	12

6.3.1	 Channel bandwidths per operating band for CA	12

6.3.2	 Co-existence studies	12

6.3.3	 ∆TIB and ∆RIB values	12

6.3.4	 REFSENS requirements	12

6.4	CA_42F_BCS0	12

6.4.1	Channel bandwidths per operating band for CA	12

6.4.2	Co-existence studies	12

6.4.3	∆TIB and ∆RIB values	13

6.4.4	REFSENS requirements	13

6.4.5	Maximum input level	13

6.4.6	Adjacent channel selectivity	13

6.4.7	Blocking characteristics	14

6.4.7.1	In-band blocking	14

6.4.7.2	Out-of-band blocking	15

6.4.7.3	Narrow band blocking	15

6.4.8	Spurious response	16

6.4.9	Intermodulation characteristics	16

6.4.10	Blocking characteristics	17

6.5	CA_46C_BCS1	17

6.5.1	Operating bands for CA	17

6.5.2	Channel bandwidths per operating band for CA	18

6.6	CA_46D_BCS1	18

6.6.1	Operating bands for CA	18

6.6.2	Channel bandwidths per operating band for CA	18

6.7	CA_12A-12A_BCS0	18

6.7.1	 Channel bandwidths per operating band for CA	18

6.7.2	 Co-existence studies	19

6.7.3	 ∆TIB and ∆RIB values	20

6.7.4	UL configuration for reference sensitivity	20

6.8	CA_7A-7A_BCS3	20

6.8.1	 Channel bandwidths per operating band for CA	20

6.8.2	 Co-existence studies	20

6.8.3	 ∆TIB and ∆RIB values	20

6.8.4	 REFSENS requirements	21

6.9	CA_40D_3UL_BCS0	21

6.9.1	Channel bandwidths per operating band	21

6.9.2	UE maximum output power for	21

6.9.3	UE Maximum Output power for modulation / channel bandwidth	21

6.9.4	Power control	21

6.9.5	Transmit modulation quality	21

6.9.6	Spectrum emission mask	21

6.9.7	UTRA and E-UTRA ACLR requirements	22

6.9.8	Spurious emission band UE co-existence	22

6.9.9	Transmit intermodulation	22

6.9.10	MPR for uplink CA bandwidth class D	22

6.10	CA_41D_3UL_BCS0	24

6.10.1	Channel bandwidths per operating band	24

6.10.2	UE maximum output power	24

6.10.3	Spurious emission band UE co-existence	24

6.10.4	UE maximum output power with additional requirements	24

6.10.5	Additional Spectrum emission mask for CA_41D	24

6.11	CA_3A-3A_BCS2	25

6.11.1	 Channel bandwidths per operating band for CA	25

6.11.2 	Co-existence studies	25

6.11.3	∆TIB and ∆RIB values	25

No TIB and RIB are required for CA_3A-3A intra-band non-contiguous CA.6.11.4	REFSENS requirements	25

6.12	CA_3B_BCS0	26

6.12.1	Channel bandwidths per operating band for CA	26

6.12.2 	Co-existence studies	26

6.12.3	∆TIB and ∆RIB values	27

6.12.4	REFSENS requirements	27

6.12.5	Blocking characteristics	27

6.12.5.1	In-band blocking	27

6.12.5.2	Out-of-band blocking	27

6.13	CA_3DL_40D_1UL_BCS1 and CA_4DL_40E_1UL_BCS0	28

6.13.1	Channel bandwidths per operating band for CA	28

6.13.2	Blocking characteristics	28

6.13.2.1	In-band blocking	28

6.13.2.2	Out-of-band blocking	28

6.14	CA_2DL_40A-40A_1UL_BCS1, CA_3DL_40A-40C_1UL_BCS0 and CA_4DL_40C-40C_1UL_BCS0	29

6.14.1	Channel bandwidths per operating band for CA	29

6.15	CA_3DL_66A-66B_1UL_BCS0	30

6.15.1	Channel bandwidths per operating band for CA	30

6.15.2	REFSENS requirements	30

6.16	CA_2DL_ CA_46A-46A_0UL_BCS0, CA_3DL_ CA_ 46A-46C_0UL_BCS0 and CA_4DL_ CA_ 46A-46D_0UL_BCS0	31

6.16.1	Channel bandwidths per operating band for CA	31

6.17	CA_2DL_5A-5A_1UL_BCS1	31

6.17.1	Channel bandwidths per operating band for CA	31

6.18	CA_2DL_5B_1UL_BCS1	32

6.18.1	Channel bandwidths per operating band for CA	32

6.19	CA_2DL_48C_1UL_BCS0	32

6.19.1	Channel bandwidths per operating band for CA	32

6.19.2	Co-existence studies	32

6.19.3	∆TIB and ∆RIB values	33

6.19.4	REFSENS requirements	33

6.19.5	Blocking requirements	33

6.20	CA_3DL_48D_1UL_BCS0	34

6.20.1	Channel bandwidths per operating band for CA	34

6.20.2	Co-existence studies	34

6.20.3	∆TIB and ∆RIB values	34

6.20.4	REFSENS requirements	34

6.20.5	Blocking requirements	35

6.21	CA_4DL_48E_1UL_BCS0	35

6.21.1	Channel bandwidths per operating band for CA	35

6.21.2	Co-existence studies	35

6.21.3	∆TIB and ∆RIB values	36

6.21.4	REFSENS requirements	36

6.21.5	Blocking requirements	36

6.22	CA_5DL_48F_1UL_BCS0	37

6.22.1	Channel bandwidths per operating band for CA	37

6.22.2	Co-existence studies	37

6.22.3	∆TIB and ∆RIB values	38

6.22.4	REFSENS requirements	38

6.22.5	Blocking requirements	38

6.23	CA_4DL_46E_0UL_BCS1	39

6.23.1	Operating bands for CA	39

6.23.2	Channel bandwidths per operating band for CA	39

6.24	CA_3DL_41A-41C_2UL_41C_BCS0	40

6.24.1	Channel bandwidths per operating band for CA	40

6.24.2	 Co-existence studies	40

6.24.3	 ∆TIB and ∆RIB values	40

6.24.4	 REFSENS requirements	40

6.25	CA_4DL_41E_2UL_41C_BCS0	41

6.25.1	Channel bandwidths per operating band for CA	41

6.25.2	 Co-existence studies	41

6.25.3	 ∆TIB and ∆RIB values	41

6.25.4	 REFSENS requirements	41

6.25.5	 Blocking requirements	41

6.26	CA_5DL_41C-41D_2UL_41C_BCS0	42

6.26.1	Channel bandwidths per operating band for CA	42

6.26.2	 Co-existence studies	42

6.26.3	 ∆TIB and ∆RIB values	42

6.26.4	 REFSENS requirements	42

6.27	CA_48A-48A_1UL_BCS0	43

6.27.1	Channel bandwidths for CA_48A-48A	43

6.27.2	Co-existence studies	43

6.27.3	REFSENS requirements	43

6.28	CA_48A-48C_1UL_BCS0	43

6.28.1	Channel bandwidths for CA_48A-48C	43

6.28.2	Co-existence studies	44

6.28.3	REFSENS requirements	44

6.29	CA_48A-48D_1UL_BCS0	44

6.29.1	Channel bandwidths for CA_48A-48D	44

6.29.2	Co-existence studies	45

6.29.3	REFSENS requirements	45

6.30	CA_48C-48C_1UL_BCS0	45

6.30.1	Channel bandwidths for CA_48C-48C	45

6.30.2	Co-existence studies	46

6.30.3	REFSENS requirements	46

6.31	CA_48C-48D_1UL_BCS0	46

6.31.1	Channel bandwidths for CA_48C-48D	46

6.31.2	Co-existence studies	47

6.31.3	REFSENS requirements	47

6.32	CA_43C_BCS0	48

6.32.1 Operating bands for CA	48

6.32.2	Channel bandwidths per operating band for CA	48

6.32.3	Co-existence studies	48

6.32.4	∆TIB and ∆RIB values	48

6.32.5	REFSENS requirements	48

6.32.6	Other receiver RF requirements	49

6.32.6.1 In-band blocking	49

6.32.6.2 Out-of-band blocking	49

6.33	CA_1A-1A_BCS0	50

6.33.1 Operating bands for CA	50

6.33.2	Channel bandwidths per operating band for CA	50

6.33.3	Co-existence studies	50

6.33.4	∆TIB and ∆RIB values	51

Annex A: Change history	52

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for Intra-band Carrier Aggregation under Rel-14 time frame according to the New CA WI approach in Release 14 [1]. The purpose is to gather the relevant background information and studies in order to address Intra-band Carrier Aggregation requirements for the Rel-14 band combinations in Table 1-1.

Table 1-1: Release 14 intra-band carrier aggregation combinations

| CA combination | Release independent from |
| --- | --- |
| CA_2DL_3B_1UL_BCS0 | Rel-10 |
| CA_2DL_3A-3A_1UL_BCS2 | Rel-11 |
| CA_2DL_5B_2UL_BCS0 | Rel-11 |
| CA_2DL_5B_1UL_BCS1 | Rel-11 |
| CA_2DL_5A_5A_1UL_BCS1 | Rel-11 |
| CA_2DL_7A-7A_1UL_BCS2 | Rel-11 |
| CA_2DL_7A-7A_2UL_7A-7A_BCS1 | Rel-11 |
| CA_2DL_7A-7A_1UL_BCS3 | Rel-11 |
| CA_2DL_7A-7A_2UL_7A-7A_BCS3 | Rel-11 |
| CA_2DL_12A-12A_1UL_BCS0 | Rel-11 |
| CA_3DL_40D_1UL_BCS1 | Rel-11 |
| CA_3DL_40D_3UL_BCS0 | Rel-12 |
| CA_4DL_40E_1UL_BCS0 | Rel-11 |
| CA_2DL_40A-40A_1UL_BCS1 | Rel-11 |
| CA_3DL_40A-40C_1UL_BCS0 | Rel-11 |
| CA_4DL_40C-40C_1UL_BCS0 | Rel-11 |
| CA_3DL_41D_3UL_BCS0 | Rel-12 |
| CA_3DL_41A-41C_2UL_BCS0 | Rel-11 |
| CA_4DL_41E_2UL_41C_BCS0 | Rel-11 |
| CA_5DL_41C-41D_2UL_41C_BCS0 | Rel-12 |
| CA_2DL_42C_2UL_BCS1 | Rel-11 |
| CA_3DL_42D_2UL_BCS1 | Rel-11 |
| CA_5DL_42F_1UL_BCS0 | Rel-11 |
| CA_5DL_42F_2UL_BCS0 | Rel-11 |
| CA_2DL_46C_0UL_BCS0 | Rel-13 |
| CA_2DL_46C_0UL_BCS1 | Rel-13 |
| CA_3DL_46D_0UL_BCS0 | Rel-13 |
| CA_3DL_46D_0UL_BCS1 | Rel-13 |
| CA_4DL_46E_0UL_BCS0 | Rel-13 |
| CA_4DL_46E_0UL_BCS1 | Rel-13 |
| CA_5DL_42F_0UL_BCS0 | Rel-11 |
| CA_2DL_CA_46A-46A_0UL_BCS0 | Rel-13 |
| CA_3DL_CA_46A-46C_0UL_BCS0 | Rel-13 |
| CA_4DL_CA_46A-46D_0UL_BCS0 | Rel-13 |
| CA_3DL_66D_1UL_BCS0 | Rel-10 |
| CA_3DL_66A-66B_1UL_BCS0 | Rel-11 |
| CA_3DL_66A-66C_1UL_BCS0 | Rel-11 |
| CA_2DL_66A-66A_2UL_66A-66A_BCS0 | Rel-13 |
| CA_2DL_66B_2UL_66B_BCS0 | Rel-13 |
| CA_2DL_66C_2UL_66C_BCS0 | Rel-13 |

The present document contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	RP-151862, “New CA WI approach”, RAN#70.

[2]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[3]	 3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[4]	3GPP TR 36.833-2-07: “Evolved Universal Terrestrial Radio Access (E-UTRA); LTE-Advanced intra-band non-contiguous Carrier Aggregation (CA) in Band 7 (Release 12)”

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [2] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [2].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [2] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [2].

# 4	Background

The present document is a technical report for intra-band Carrier Aggregation including contiguous and non-contiguous spectrum under Rel-14 time frame. It covers both the UE and BS side. The document is divided in two different parts:

General part: this part covers BS and UE specific which is band combination independent.

Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs.

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the present document, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	Intra-band Carrier Aggregation including contiguous and non-contiguous spectrum: General Part

As Intra-band Carrier Aggregation combinations including contiguous and non-contiguous spectrum have been studied in previous releases, the identified general issues identified in previous releases also apply to Rel-14. The present document only captured the new general issues if identified in Rel-14.

# 6	Intra-band Carrier Aggregation including contiguous and non-contiguous spectrum: Specific Band Combination Part

## 6.1	CA_66D_BCS0

### 6.1.1	Channel bandwidths per operating band for CA

Table 6.1.1-1

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66D | - | 5 | 20 | 20 | 60 | 0 |
|  |  | 20 | 5 | 20 |  |  |
|  |  | 20 | 20 | 5 |  |  |
|  |  | 10 | 20 | 15 |  |  |
|  |  | 15 | 20 | 10 |  |  |
|  |  | 10, 15, 20 | 15, 20 | 20 |  |  |
|  |  | 15, 20 | 10 | 20 |  |  |
|  |  | 15 | 15, 20 | 15 |  |  |
|  |  | 20 | 15, 20 | 10, 15 |  |  |
|  |  | 20 | 10 | 15 |  |  |

For CA_66D, the specifications should support fallback to CA_66C in the event that one of the outer component carriers is released by deactivation or deconfiguration.  However, the bandwidth combination set 0 for CA_66C does not currently support 5+20 MHz, so it is proposed to add the 5+20 MHz bandwidth combination to CA_66C for fallback compatibility.

### 6.1.2	Co-existence studies

<Text will be added.>

### 6.1.3	∆TIB and ∆RIB values

No TIB and RIB are required for CA_66D intra-band CA.

### 6.1.4	REFSENS requirements

No reference sensitivity adjustments are needed for CA_66D intra-band CA with single UL.

### 6.1.5	Blocking requirements

In-band blocking and out-of-band blocking requirements for intra-band contiguous CA with 1UL should apply to CA_66D.

Table 6.1.5-1: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_66D | FInterferer (Range) | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 |  |  |  |  |

Table 6.1.5-2: Out of band blocking

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_66D | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |

## 6.2	CA_66A-66C_BCS0

### 6.2.1	Channel bandwidths per operating band for CA

Table 6.2.1-1

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66A-66C | - | 5, 10, 15, 20 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A-1 of 36.101 |  | 60 | 0 |
|  |  | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  | 5, 10, 15, 20 |  |  |

### 6.2.2	Co-existence studies

<Text will be added.>

### 6.2.3	∆TIB and ∆RIB values

No TIB and RIB are required for CA_66D intra-band CA.

### 6.2.4	REFSENS requirements

Reference sensitivty for CA_66A-66C is not anticipated to be degraded due to the large Tx-Rx separation and single uplink carrier. Annex A: Change history

## 6.3	CA_7A-7A_BCS2

CA-7A-7A is already specified in Release 12. See clause 5.1 in [4] for CA operating bands.

### 6.3.1		Channel bandwidths per operating band for CA

A new bandwidth combination set 2 is introduced for CA_7A-7A in Release 14.

Table 6.3.1-1: Supported E-UTRA bandwidths for intra-band non-contiguous CA in Band 7

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | 5, 10, 15, 20 | 5, 10 | 30 | 2 |

See clause 5.2 in [4] for the already specified bandwidth combination set 0 and set 1.

### 6.3.2		Co-existence studies

The same analysis as Release 12 is applied. See clause 7.1.1 in [4].

### 6.3.3		∆TIB and ∆RIB values

No TIB and RIB are required for CA_7A-7A intra-band CA.

### 6.3.4		REFSENS requirements

The same REFSENS requirements as Release 12 are applied, due to larger duplex gap (e.g. compared to band 25), the ΔRIBNC values for CA_7A-7A are all set to zero. See clause 6.2.1 in [4].

## 6.4	CA_42F_BCS0

### 6.4.1	Channel bandwidths per operating band for CA

Table 6.4.1-1 defines the supported E-UTRA CA configurations and bandwidth combination set for LTE-advanced intra-band contiguous CA in Band 42. The 5MHz, 10MHz and 15MHz carriers are only allowed at the edges of aggregated channel bandwidth so that potential co-existence issue with adjacent systems can be handled with some flexibility.

Table 6.4.1-1: E-UTRA CA configurations and bandwidth combination sets for CA_42F

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_42F | CA_42C | 5,10,15,20 | 20 | 20 | 20 | 20 | 100 | 0 |
|  |  | 20 | 20 | 20 | 20 | 5,10, 15, 20 |  |  |

### 6.4.2	Co-existence studies

No co-existence issue identified for CA_42F.

### 6.4.3	∆TIB and ∆RIB values

No TIB and RIB are required for CA_42F intra-band contiguous CA.

### 6.4.4	REFSENS requirements

In general, TDD intra-band carrier aggregation does not change the UE reference sensitivity requirement as it does not introduce any impact to the UE front end RF architecture. Further the REFSENS requirement for intra-band contiguous carrier aggregation is measured on a per CC basis and the same reference sensitivity level as that for single CC is reused on a per CC basis. Hence the introduction of CA_42F with 5 downlink CCs and maximum 2 uplink CCs does not impact the current REFSENS requirement.

### 6.4.5	Maximum input level

The maximum input level for CA_42F is defined to keep the total input power level unchanged compared to CA_42E,  -20dBm (64QAM) for 64QAM and -22dBm for 256QAM respectively. Correspondingly the maximum input level per CC for CA_42F should be -27dBm for 64QAM and -29dBm for 256QAM.

Table 6.4.5-1: Maximum input level for intra-band contiguous CA

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E | F |
| Power in largest Transmission Bandwidth Configuration CC | dBm |  |  |  |  |  | -272 |
|  |  |  |  |  |  |  | [-29]3 |
| Power in each other CC | dBm |  |  |  |  |  | -27 + 10log(NRB,c /NRB,largest BW) 2 |
|  |  |  |  |  |  |  | [-29] + 10log(NRB,c /NRB,largest BW) 3 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A of TS 36.101. NOTE 2: Reference measurement channel is Annex A.3.2: 64QAM, R=3/4 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: Reference measurement channel is Annex A.3.2: 256QAM, R=4/5 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |

### 6.4.6	Adjacent channel selectivity

To account for the added difficulty of the baseband filter impact, the ACS requirements are scaled for the number of aggregated component carriers.  The ACS requirements for CA_42F UE is defined as 20dB as shown in Table 6.4.6-1. Corresponding power settings for the ACS requirements are shown in Table 6.4.6-2 and Table 6.4.6-3.

Table 6.4.6-1: Adjacent channel selectivity

|  |  | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C | D | E | F |
| ACS | dB |  |  |  |  | 20 |

Table 6.4.6-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC |  |  |  |  |  | REFSENS + 14 dB |
| PInterferer | dBm |  |  |  |  | Aggregated power + 18.5 dB |
| BWInterferer | MHz |  |  |  |  | 5 |
| FInterferer (offset) | MHz |  |  |  |  | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A in TS 36.101. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 NOTE 3: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |

Table 6.4.6-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm |  |  |  |  | -43.5 +10log10(NRB,c/NRB agg) |
| PInterferer | dBm | -25 |  |  |  |  |
| BWInterferer | MHz |  |  |  |  | 5 |
| FInterferer (offset) | MHz |  |  |  |  | 2.5+ Foffset / -2.5- Foffset |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A in TS36.101. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 NOTE 3: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |

### 6.4.7	Blocking characteristics

#### 6.4.7.1	In-band blocking

To account for the added difficulty of the baseband filter impact, the in-band requirement is scaled for the number of aggregated component carriers.  The wanted signal level per CC for CA_42F UE is defined as REFSENS+16dBm as shown in Table 6.4.7.1-1 and CA configuration is listed in Table 6.4.7.1-2.

Table 6.4.7.1-1: In band blocking parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  |  |  |  |  | 16 |
| BWInterferer | MHz |  |  |  |  | 5 |
| FIoffset, case 1 | MHz |  |  |  |  | 7.5 |
| FIoffset, case 2 | MHz |  |  |  |  | 12.5 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A in TS 36.101 NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 in TS 36.101 |  |  |  |  |  |  |

Table 6.4.7.1-2: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_1C, CA_2C, CA_3C, CA_5B, CA_7C, CA_8B, CA_12B, CA_23B, CA_27B, CA_38C, CA_39C, CA_40C, CA_41C, CA_40D, CA_41D, CA_42C, CA_42D, CA_42E, CA_42F, CA_66B, CA_66C | FInterferer (Range) | MHz | (Note 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 NOTE 3: Foffset is the frequency offset from the center frequency of the CC being tested to the edge of aggregated channel bandwidth. NOTE 4: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

#### 6.4.7.2	Out-of-band blocking

There is no aggregated bandwidth dependency for out-of-band blocking requirement.  The same requirement is reused for CA_42F with no changes from the requirements for CA Class B/C/D/E.

Table 6.4.7.2-1: Out-of-band blocking parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  |  |  |  |  | 9 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2. |  |  |  |  |  |  |

Table 6.4.7.2-2: Out-of-band blocking

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_1C, CA_2C, CA_3C, CA_5B, CA_7C , CA_8B, CA_12B, CA_23B, CA_27B, CA_38C, CA_40C, CA_41C, CA_40D, CA_42C1, CA_42D1, CA_42E1, CA_42F1,CA_66B, CA_66C | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for this CA configuration for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

#### 6.4.7.3	Narrow band blocking

There is no aggregated bandwidth dependency for narrow blocking requirement.  The same requirement is reused for CA bandwidth Class F with no changes from the requirements for CA Class B/C/D/E.

Table 6.4.7.3-1: Narrow band blocking

| Parameter | Unit | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Power per CC in Aggregated Transmission Bandwidth Configuration | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  |  |  |  |  | 16 |
| Puw (CW) | dBm |  |  |  |  | -55 |
| Fuw (offset for f = 15 kHz) | MHz |  |  |  |  | - Foffset – 0.2 / + Foffset + 0.2 |
| Fuw (offset for f = 7.5 kHz) | MHz |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The Fuw (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. NOTE 4: The requirement is applied for the band combinations whose component carriers’ BW≥5 MHz. |  |  |  |  |  |  |

### 6.4.8	Spurious response

There is no aggregated bandwidth dependency for spurious response requirement.  The same requirement is reused for CA bandwidth Class F with no changes from the requirements for CA Class B/C/D/E.

Table 6.4.8-1: Spurious response parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  |  |  |  |  | 9 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

Table 6.4.8-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

### 6.4.9	Intermodulation characteristics

When the aggregated bandwidth is wider, the intermodulation requirements need to account for the added difficulty of the baseband filter impact. The wanted signal power per CC is scaled up as REFSENS+16dB for CA_42F.

Table 6.4.9-1: Wide band intermodulation

| Rx parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Power per CC in Aggregated Transmission Bandwidth Configuration | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  |  |  |  |  | 16 |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |
| BWInterferer 2 | MHz |  |  |  |  | 5 |
| FInterferer 1 (Offset) | MHz |  |  |  |  | –Foffset-7.5 / + Foffset+7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 with set-up according to Annex C.3.1. NOTE 4: The interfering modulated signal is 5MHz E-UTRA signal as described in Annex D for channel bandwidth 5MHz; NOTE 5: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |  |  |

### 6.4.10	Blocking characteristics

There is no aggregated bandwidth dependency for Rx image requirement. It is proposed to reuse Rx Image requirements for CA_42F with no changes from Class B/C/D/E.

Table 6.4.10-1: Receiver image rejection

|  | CA bandwidth class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rx parameter | Units | A | B | C | D | E | F |
| Receiver image rejection | dB |  |  |  |  |  | 25 |

## 6.5	CA_46C_BCS1

### 6.5.1	Operating bands for CA

Table 6.5.1-1: Intra-band contiguous CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high | FDL_low – FDL_high |  |  |  |
| CA_46 | 46 | N/A | 5150 MHz | – | 5925 MHz | TDD |

### 6.5.2	Channel bandwidths per operating band for CA

Table 6.5.2-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_46C | - | 20 | 10, 20 |  |  | 40 | 1 |
|  |  | 10, 20 | 20 |  |  |  |  |

NOTE: 	Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell.

## 6.6	CA_46D_BCS1

### 6.6.1	Operating bands for CA

Table 6.6.1-1: Intra-band contiguous CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high | FDL_low – FDL_high |  |  |  |
| CA_46 | 46 | N/A | 5150 MHz | – | 5925 MHz | TDD |

### 6.6.2	Channel bandwidths per operating band for CA

Table 6.6.2-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_46D | - | 20 | 20 | 10, 20 |  | 60 | 1 |
|  |  | 10, 20 | 20 | 20 |  |  |  |

NOTE: 	Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell.

## 6.7	CA_12A-12A_BCS0

### 6.7.1		Channel bandwidths per operating band for CA

Bandwidth combination set 0 for CA_12A-12A introduced as in table 6.8.1-1.

Table 6.8.1-1: Supported E-UTRA bandwidths for intra-band non-contiguous CA in Band 12

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_12A-12A | 5 | 5 | 10 | 0 |

### 6.7.2		Co-existence studies

Table 7.2-1 shows harmonics frequency limits and Table 8.1-2 inter-modulation products frequency limits for CA of Band 12, respectively.

Table 7.2-1: DL harmonics frequency limits for CA of Band 12

| BS DL carriers | f_low | f_high |
| --- | --- | --- |
| DL frequency (MHz) | 729 | 746 |
| 2nd order harmonics frequency range (MHz) | 1458 to 1492 |  |
| 3rd order harmonics frequency range (MHz) | 2187 to 2238 |  |

As shown in Table 7.2-1:

- second harmonics may fall into Band 21 or Band 45 UL frequencies

- no third harmonics will fall to any 3GPP UL frequencies.

Table 7.2-2: DL inter-modulation products frequency limits for CA of Band 12

| BS DL carriers | f_low | f_high |
| --- | --- | --- |
| DL frequency (MHz) | 729 | 746 |
| 2nd order IMD products | f_low - f_high | f_high - f_low |
| IMD frequency range (MHz) | -17 to 17 |  |
| 3rd order IMD products | 2*f_low – f_high | 2* f_high – f_low |
| IMD frequency range (MHz) | 712 to 763 |  |

As shown in Table 7.2-2:

- no second order inter-modulation products will fall to any 3GPP UL frequencies.

- third order inter-modulation products may fall into Band 12, 17, 28, 44 or 68 UL frequencies

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 12, 17, 21, 28, 44, 45 or 68 BS receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 12 BS transmitters do not share the same antenna with Band 17, 21, 28, 44, 45 or 68 BS receiver, or the own Band 12 BS receiver if the aforementioned BS transmit configurations are used.

Therefore, in order to prevent BS receiver desensitization, it is recommended that Band 12 BS transmitters should not share the same antenna with Band 17, 21, 28, 44, 45 or 68 BS receiver (or the own Band 12 BS receiver if the aforementioned BS transmit configurations are used) unless the antenna path meets very stringent 2nd and 3rd order PIM specification so that the PIM will not cause Band 17, 21, 28, 44, 45 or 68 (or the own Band 12 BS) receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

### 6.7.3		∆TIB and ∆RIB values

No TIB and RIB are required for CA_12A-12A intra-band CA.

### 6.7.4	UL configuration for reference sensitivity

Table 6.7.5-1: Intra-band non-contiguous CA_12A-12A uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_12A-12A | 25RB+25RB | 0.0 < Wgap ≤ 7.0 | 51 | 3 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located at RBstart=12. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The carrier center frequency of PCC in the UL operating band is configured closer to the DL operating band. |  |  |  |  |  |

## 6.8	CA_7A-7A_BCS3

CA-7A-7A is already specified in Release 12. See clause 5.1 in [4] for CA operating bands.

### 6.8.1		Channel bandwidths per operating band for CA

A new bandwidth combination set 3 is introduced for CA_7A-7A in Release 14.

Table 6.8.1-1: Intra-band non-contiguous CA operating band definition in Band 7

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_7-7 | 7 | 2500 MHz | – | 2570 MHz | 10, 15, 20 | 2620 MHz | – | 2690 MHz | 10, 15, 20 | FDD |

Table 6.8.1-2: Supported E-UTRA bandwidths for intra-band non-contiguous CA in Band 7

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | 10, 15, 20 | 10, 15, 20 | 40 | 3 |

### 6.8.2		Co-existence studies

The same analysis as Release 12 is applied. See clause 7.1.1 in [4].

### 6.8.3		∆TIB and ∆RIB values

No TIB and RIB are required for CA_7A-7A intra-band CA.

### 6.8.4		REFSENS requirements

The same REFSENS requirements as Release 12 are applied, due to larger duplex gap (e.g. compared to band 25), the ΔRIBNC values for CA_7A-7A are all set to zero. See clause 6.2.1 in [4].

## 6.9	CA_40D_3UL_BCS0

### 6.9.1	Channel bandwidths per operating band

Table 6.9.1-1 defines the supported E-UTRA CA configurations and bandwidth combination set for LTE-advanced 3UL intra-band contiguous CA in Band 40.

Table 6.9.1-1: E-UTRA CA configurations and bandwidth combination sets for CA_40D

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] Channel bandwidths for carrier [MHz] |  |  |
| CA_40D | CA_40D | 10, 15, 20 | 20 | 20 | 60 | 0 |
|  |  | 20 | 10, 15 | 20 |  |  |
|  |  | 20 | 20 | 10, 15 |  |  |

### 6.9.2	UE maximum output power for

The UE maximum output power for CA_40D is proposed to reuse requirement for bandwidth class C.

Table 6.9.2-1 CA UE Power Class for intra-band contiguous CA_40D

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_40D |  |  |  |  | 23 | +2/-2 |  |  |

### 6.9.3	UE Maximum Output power for modulation / channel bandwidth

The requirements are FFS.

### 6.9.4	Power control

Requirements for bandwidth class B and C can be reused.

### 6.9.5	Transmit modulation quality

Requirements for bandwidth class B and C can be reused.

### 6.9.6	Spectrum emission mask

Table 6.9.6-1 shows the SEM requirement for bandwidth class D.

Table 6.9.6-1: General E-UTRA CA spectrum emission mask for Bandwidth Class D

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB | 50RB+75RB+100RB | 75RB+75RB+75RB | 50RB+100RB+100 RB | 75RB+75RB+100RB | 75RB+100RB+100RB | 100RB+100RB+100 RB | Measurement bandwidth |
| (MHz) | (44.6MHz) | (45MHz) | (49.7MHz) | (49.85MHz) | (54.65MHz) | (59.6MHz) |  |
|  0-1 | -22 | -22 | -22.5 | -22.5 | -23.5 | -24 | 30 kHz |
|  1-5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
|  5-44.6 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
|  44.6-45 | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
|  45-49.6 | -25 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
|  49.6-49.7 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
|  49.7-49.85 |  | -25 | -25 | -13 | -13 | -13 | 1 MHz |
|  49.85-50 |  | -25 | -25 | -25 | -13 | -13 | 1 MHz |
|  50-54.65 |  |  | -25 | -25 | -13 | -13 | 1 MHz |
|  54.65-54.7 |  |  | -25 | -25 | -25 | -13 | 1 MHz |
|  54.7-54.85 |  |  |  | -25 | -25 | -13 | 1 MHz |
|  54.85-59.6 |  |  |  |  | -25 | -13 | 1 MHz |
|  59.6-59.65 |  |  |  |  | -25 | -25 | 1 MHz |
|  59.65-64.6 |  |  |  |  |  | -25 | 1 MHz |

### 6.9.7	UTRA and E-UTRA ACLR requirements

Requirements for bandwidth class B and C can be reused.

### 6.9.8	Spurious emission band UE co-existence

Table 6.9.8-1 shows the spurious emission band UE co-existence for CA_40D.

Table 6.9.8-1: Spurious emission band UE co-existence Requirements

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_40D | E-UTRA Band 1, 3, 7, 8, 20, 22, 26, 27, 32, 33, 34, 38, 39, 41, 42, 43, 44, 65, 67 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.9.9	Transmit intermodulation

Requirements for bandwidth class B and C can be reused.

### 6.9.10	MPR for uplink CA bandwidth class D

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power due to higher order modulation and contiguously aggregated transmit bandwidth configuration (resource blocks) is specified in Table 1. In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 1: MPR for bandwidth class D

| Modulation | CA bandwidth Class D |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 RB + 75 RB + 100RB | 50 RB + 100 RB + 100 RB | 75 RB + 75 RB + 100 RB | 75 RB + 100 RB + 100 RB | 100 RB + 100 RB + 100 RB |  |
| QPSK | > 12 and ≤ 50 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 1 |
| QPSK | > 50 and ≤ 125 | > 50 and ≤ 150 | > 75 and ≤ 150 | > 75 and ≤ 175 | > 100 and ≤ 200 | ≤ 2 |
| QPSK | > 125 | > 150 | > 150 | > 150 | > 200 | ≤ 3 |
| 16 QAM | ≤ 12 | ≤ 12 | ≤ 16 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16 QAM | > 12 and ≤ 50 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 2 |
| 16 QAM | > 50 and ≤ 125 | > 50 and ≤ 150 | > 75 and ≤ 150 | > 75 and ≤ 175 | > 100 and ≤ 200 | ≤ 3 |
| 16 QAM | > 125 | > 150 | > 150 | >175 | > 200 | ≤ 3.5 |
| 64 QAM | ≤ 12 allocation wholly contained within a single CC | ≤ 12 and allocation wholly contained within a single CC | ≤ 16 and allocation wholly contained within a single CC | ≤ 16 and allocation wholly contained within a single CC | ≤ 18 and allocation wholly contained within a single CC | ≤ 2 |
| 64 QAM | > 12 allocation wholly contained within a single CC or allocation extends across two CC’s | > 12 allocation wholly contained within a single CC or allocation extends across two CC’s | > 16 allocation wholly contained within a single CC or allocation extends across two CC’s | > 16 allocation wholly contained within a single CC or allocation extends across two CC’s | > 18 allocation wholly contained within a single CC or allocation extends across two CC’s | ≤ 3 |
| 64 QAM | allocation extends across three CC’s | allocation extends across three CC’s | allocation extends across three CC’s | allocation extends across three CC’s | allocation extends across three CC’s | ≤ 4.0 |

For intra-band contiguous carrier aggregation bandwidth class D with non-contiguous resource allocation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A-1 is specified as follows

MPR = CEIL { MA, 0.5 }

Where MA is defined as follows

MA = 	8.2						; 0 ≤ A < 0.025

9.2 - 40A 				; 0.025	 ≤ A < 0.05

8 – 16A				; 0.05	≤ A < 0.25

4.0						; 0.25 ≤ A < 1

Intra-band contiguous carrier aggregation bandwidth class D 256 QAM MPR is FFS.

## 6.10	CA_41D_3UL_BCS0

### 6.10.1	Channel bandwidths per operating band

Table 6.10.1-1 defines the supported E-UTRA CA configurations and bandwidth combination set for LTE-advanced 3UL intra-band contiguous CA in Band 41.

Table 6.10.1-1: E-UTRA CA configurations and bandwidth combination sets for CA_41D

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] Channel bandwidths for carrier [MHz] |  |  |
| CA_41D | CA_41D | 10 | 20 | 15 | 60 | 0 |
|  |  | 10 | 15, 20 | 20 |  |  |
|  |  | 15 | 20 | 10, 15 |  |  |
|  |  | 15 | 10, 15, 20 | 20 |  |  |
|  |  | 20 | 15, 20 | 10 |  |  |
|  |  | 20 | 10, 15, 20 | 15, 20 |  |  |

### 6.10.2	UE maximum output power

The UE maximum output power for CA_41D is proposed to reuse requirement for bandwidth class C.

Table 6.10.2-1 CA UE Power Class for intra-band contiguous CA_41D

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_41D |  |  |  |  | 23 | +2/-22 |  |  |
| NOTE 2: If all transmitted resource blocks (Figure 5.6A-1) over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

### 6.10.3	Spurious emission band UE co-existence

Table 6.10.3-1 shows the spurious emission band UE co-existence for CA_41D.

Table 6.10.3-1: Spurious emission band UE co-existence for CA_41D

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_41D | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.10.4	UE maximum output power with additional requirements

The requirements are FFS.

### 6.10.5	Additional Spectrum emission mask for CA_41D

The requirements are FFS.

## 6.11	CA_3A-3A_BCS2

### 6.11.1	 Channel bandwidths per operating band for CA

Table 6.11.1-1: Supported E-UTRA bandwidths for intra-band non-contiguous CA in Band 3

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_3A-3A | 5 | 3 | 10 | 2 |
|  | 3, 5 | 5 |  |  |

### 6.11.2 	Co-existence studies

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of operating band 3 non-contiguous DL carriers can be calculated as shown in Table 6.11.2-1 below:

Table 6.11.2-1: Operating Band 3 non-contiguous DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 1805 | 1880 |
| 2nd harmonics frequency limits (MHz) | 3610 | 3760 |
| 3rd harmonics frequency limits (MHz) | 5415 | 5640 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -75 | 75 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 1730 | 1955 |

It can be seen from table 6.11.2-1 that the 2nd order harmonics from operating band 3 DL transmit will fall inside operating band 43, and 3rd order harmonics from operating band 3 DL transmit will fall inside operating band 46. It can also be seen that the third order inter-modulation products from a BS supporting NC_CA in operating band 3 may fall inside the receive part of operating bands 1, 2, 3, 4, 9, 10, 25, 33, 35, 36, 37, 39, 65 and 66. Note that the limits in table 6.11.2-1 are calculated assuming maximum possible spacing between the aggregated carriers in the operating band.

Table 6.11.2-2 gives the harmonic products for band 3 non-contiguous CA with 1UL. None of the harmonic products fall into the own receive band for UE, so it is concluded that there is no issue on UL harmonic interference.

Table 6.11.2-2: 1UL Band 3 non-contiguous CA harmonic products

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 3 | 1710 | 1785 | 3420 | 3570 | 5130 | 5355 |

### 6.11.3	∆TIB and ∆RIB values

### No TIB and RIB are required for CA_3A-3A intra-band non-contiguous CA.6.11.4	REFSENS requirements

For intra-band non-contiguous carrier aggregation with one uplink carrier of Band 3 the RIBNC has already been defined in Table 7.3.1A-3 of TS36.101 for the SCC(s) for BCS0 and BCS1. For the new BCS2 the legacy requirement of 25RB+25RB could be reused as shown in below table. For the 15RB+25RB and 25RB+15RB corresponding requirements are also provided in Table 6.11.4-1.

Table 6.11.4-1: Intra-band non-contiguous CA in Band 3 with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_3A-3A | 15RB+25RB | 47.0 < Wgap ≤ 67.0 | 121 | 4.5 | FDD |
|  |  | 0.0 < Wgap ≤ 47.0 | 151 | 0 |  |
|  | 25RB+15RB | 47.0 < Wgap ≤ 67.0 | 121 | 4.9 |  |
|  |  | 0.0 < Wgap ≤ 47.0 | 251 | 0 |  |
|  | 25RB+25RB | 45.0 < Wgap ≤ 65.0 | 121 | 4.7 |  |
|  |  | 0.0 < Wgap ≤ 45.0 | 251 | 0 |  |
| NOTE 1: 1refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. |  |  |  |  |  |

## 6.12	CA_3B_BCS0

### 6.12.1	Channel bandwidths per operating band for CA

Table 6.12.1-1: Supported E-UTRA bandwidths for intra-band contiguous CA in Band 3

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_3B | 5 | 3 | 10 | 0 |
|  | 3, 5 | 5 |  |  |

### 6.12.2 	Co-existence studies

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of operating band 3 contiguous DL carriers can be calculated as shown in Table 6.12.2-1 below:

Table 6.12.2-1: Operating Band 3 contiguous DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 1805 | 1880 |
| 2nd harmonics frequency limits (MHz) | 3610 | 3760 |
| 3rd harmonics frequency limits (MHz) | 5415 | 5640 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -75 | 75 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 1730 | 1955 |

It can be seen from table 6.12.2-1 that the 2nd order harmonics from operating band 3 DL transmit will fall inside operating band 43, and 3rd order harmonics from operating band 3 DL transmit will fall inside operating band 46. It can also be seen that the third order inter-modulation products from a BS supporting C_CA in operating band 3 may fall inside the receive part of operating bands 1, 2, 3, 4, 9, 10, 25, 33, 35, 36, 37, 39, 65 and 66. Note that the limits in table 6.12.2-1 are calculated assuming maximum possible spacing between the aggregated carriers in the operating band.

Table 6.12.2-2 gives the harmonic products for band 3 contiguous CA with 1UL. None of the harmonic products fall into the own receive band for UE, so it is concluded that there is no issue on UL harmonic interference.

Table 6.12.2-2: 1UL Band3 contiguous CA harmonic products

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 3 | 1710 | 1785 | 3420 | 3570 | 5130 | 5355 |

### 6.12.3	∆TIB and ∆RIB values

No ∆TIB and ∆RIB are required for CA_3B intra-band contiguous CA.

### 6.12.4	REFSENS requirements

No reference sensitivity adjustments are needed for CA_3B intra-band CA with single UL.

### 6.12.5	Blocking characteristics

#### 6.12.5.1	In-band blocking

The additions for CA_3B_BCS0 in Table 6.12.5.1-1 need be added to Table 7.6.1.1A-2 in TS 36.101.

Table 6.12.5.1-1: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_3B | FInterferer (Range) | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 NOTE 3: Foffset is the frequency offset from the center frequency of the CC being tested to the edge of aggregated channel bandwidth. NOTE 4: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

#### 6.12.5.2	Out-of-band blocking

The additions for CA_3B_BCS0 in Table 6.12.5.2-1 need be added to Table 7.6.2.1A-2 in TS 36.101.

Table 6.12.5.2-1: Out of band blocking

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_3B | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for this CA configuration for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

## 6.13	CA_3DL_40D_1UL_BCS1 and CA_4DL_40E_1UL_BCS0

### 6.13.1	Channel bandwidths per operating band for CA

The additions for CA_40D_BCS1 and CA_40E_BCS0 in Table 6.13.1-1 need to be made to Table 5.6A.1-1 in TS 36.101.

Table 6.13.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_40D |  | 15,20 | 15,20 | 15,20 |  | 60 | 1 |
| CA_40E |  | 15, 20 | 15, 20 | 15, 20 | 20 | 80 | 0 |

### 6.13.2	Blocking characteristics

#### 6.13.2.1	In-band blocking

The additions for CA_40E_BCS0 in Table 6.13.2.1-1 need to be made to Table 7.6.1.1A-2 in TS 36.101.

Table 6.13.2.1-1: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_40E | FInterferer (Range) | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 NOTE 3: Foffset is the frequency offset from the center frequency of the CC being tested to the edge of aggregated channel bandwidth. NOTE 4: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

#### 6.13.2.2	Out-of-band blocking

The additions for CA_40E_BCS0 in Table 6.13.2.2-1 need to be made to Table Table 7.6.2.1A-2 in TS 36.101.

Table 6.13.2.2-1: Out of band blocking

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_40E | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for this CA configuration for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

## 6.14	CA_2DL_40A-40A_1UL_BCS1, CA_3DL_40A-40C_1UL_BCS0 and CA_4DL_40C-40C_1UL_BCS0

### 6.14.1	Channel bandwidths per operating band for CA

The additions for CA_40A-40A_BCS1, CA_40A-40C_BCS0 and CA_40C-40C_BCS0 in Table 6.14.1-1 need to be made to Table 5.6A.1-3 in TS 36.101.

Table 6.14.1-1: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with two sub-blocks)

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_40A-40A | - | 10,15,20 | 10,15,20 |  |  | 40 | 1 |
| CA_40A-40C | - | 20 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 of TS 36.101 |  |  | 60 | 0 |
|  |  | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 of TS 36.101 |  | 20 |  |  |  |
| CA_40C-40C | - | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 of TS 36.101 |  | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 of TS 36.101 |  | 80 | 0 |

The additions for CA_40A-40C_BCS0 and CA_40C-40C_BCS0 in Table 6.14.1-2 need to be made to Table 7.3.1A-3 in TS 36.101.

Table 6.14.1-2: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_40A-40C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_40C-40C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |

## 6.15	CA_3DL_66A-66B_1UL_BCS0

### 6.15.1	Channel bandwidths per operating band for CA

The additions for CA_66A-66B_BCS0 in Table 6.15.1-1 need to be made to Table 5.6A.1-3 in TS 36.101.

Table 6.15.1-1: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with two sub-blocks)

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66A-66B | - | 5, 10, 15, 20 | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | 40 | 0 |
|  |  | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  | 5, 10, 15, 20 |  |  |  |

### 6.15.2	REFSENS requirements

The additions for CA_66A-66B_BCS0 in Table 6.15.2-1 need to be made to Table 7.3.1A-3 in TS 36.101.

Table 6.15.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_66A-66B | NOTE 6 | NOTE 7 | NOTE 8, NOTE 16 | 0.0 | FDD |

## 6.16	CA_2DL_ CA_46A-46A_0UL_BCS0, CA_3DL_ CA_ 46A-46C_0UL_BCS0 and CA_4DL_ CA_ 46A-46D_0UL_BCS0

### 6.16.1	Channel bandwidths per operating band for CA

Table 6.16.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_46A-46A 1 | - | 20 | 20 |  |  | 40 | 0 |
| CA_46A-46C 1 | - | 20 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS 36.101 |  |  | 60 | 0 |
| CA_46A-46D 1 | - | 20 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS 36.101 |  |  | 80 | 0 |
| NOTE 1: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |

## 6.17	CA_2DL_5A-5A_1UL_BCS1

### 6.17.1	Channel bandwidths per operating band for CA

The additions for CA_5A-5A_BCS1 in Table 6.17.1-1 need to be made to Table 5.6A.1-3 in TS 36.101.

Table 6.17.1-1: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with two sub-blocks)

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_5A-5A | - | 3 | 5 |  |  | 8 | 1 |

## 6.18	CA_2DL_5B_1UL_BCS1

### 6.18.1	Channel bandwidths per operating band for CA

The additions for CA_5B_BCS1 in Table 6.17.1-1 need to be made to Table 5.6A.1-1 in TS 36.101.

Table 6.18.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_5B | - | 3 | 5 |  |  | 8 | 1 |
|  |  | 5 | 3 |  |  |  |  |

## 6.19	CA_2DL_48C_1UL_BCS0

### 6.19.1	Channel bandwidths per operating band for CA

Table 6.19.1-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_48C | - | 5, 10, 15, 20 | 20 |  |  |  | 40 | 0 |
|  |  | 20 | 5, 10, 15 |  |  |  |  |  |

### 6.19.2	Co-existence studies

Table 6.19.2-1 presents DL harmonics and IMD products for Band 48 contiguous CA. As shown below, none of 2nd/3rd DL harmonics nor 2nd order IMD products will impact any 3GPP bands. 3rd order IMD products fall to UL frequency range of Band 22, 42 and 43. It should be noted Band 42 and 43 will be not used in the same region as Band 48. Furthermore, as Band 48 is a TDD band, the 3rd order IMD product will not fall to UL frequency range as BS will not transmit and receive at the same time. Note that the limits in table 6.19.2-1 are calculated assuming maximum possible spacing between the aggregated carriers in the operating band.

Table 6.19.2-1: DL harmonics and IMD products for Band 48 contiguous CA

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 3550 | 3700 |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -150 | 150 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 3400 | 3850 |

Table 6.19.2-2 gives the harmonic products for band 48 contiguous CA with 1UL. None of the harmonic products fall into the own receive band for UE, so it is concluded that there is no issue on UL harmonic interference.

Table 6.19.2-2: 1UL Band 48 contiguous CA harmonic products

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |

### 6.19.3	∆TIB and ∆RIB values

No TIB and RIB are required for CA_48C intra-band CA.

### 6.19.4	REFSENS requirements

Table 6.19.4-1: Intra-band contiguous CA uplink configuration for reference sensitivity for Bandwidth Class C

| CA configuration / CC combination / NRB_agg / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink CA configuration | 100RB+25RB |  | 100RB+50RB |  | 75RB+75RB |  | 75RB+50RB |  | 100RB+75RB |  | 100RB+100RB |  | Duplex Mode |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |
| CA_48C | 100 | 25 | 100 | 50 | N/A | N/A | N/A | N/A | 100 | 75 | N/A | N/A | TDD |

### 6.19.5	Blocking requirements

In-band blocking and out-of-band blocking requirements for intra-band contiguous CA with 1UL should apply to CA_48C.

Table 6.19.5-1: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_48C | FInterferer (Range) | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 |  |  |  |  |

Table 6.19.5-2: Out of band blocking

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_48C1 | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for this CA configuration for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

## 6.20	CA_3DL_48D_1UL_BCS0

### 6.20.1	Channel bandwidths per operating band for CA

Table 6.20.1-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_48D | - | 5,10,15,20 | 20 | 20 |  |  | 60 | 0 |
|  |  | 20 | 20 | 5,10,15 |  |  |  |  |

### 6.20.2	Co-existence studies

Table 6.20.2-1 presents DL harmonics and IMD products for Band 48 contiguous CA. As shown below, none of 2nd/3rd DL harmonics nor 2nd order IMD products will impact any 3GPP bands. 3rd order IMD products fall to UL frequency range of Band 22, 42 and 43. It should be noted Band 42 and 43 will be not used in the same region as Band 48. Furthermore, as Band 48 is a TDD band, the 3rd order IMD product will not fall to UL frequency range as BS will not transmit and receive at the same time. Note that the limits in table 6.20.2-1 are calculated assuming maximum possible spacing between the aggregated carriers in the operating band.

Table 6.20.2-1: DL harmonics and IMD products for Band 48 contiguous CA

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 3550 | 3700 |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -150 | 150 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 3400 | 3850 |

Table 6.20.2-2 gives the harmonic products for band 48 contiguous CA with 1UL. None of the harmonic products fall into the own receive band for UE, so it is concluded that there is no issue on UL harmonic interference.

Table 6.20.2-2: 1UL Band 48 contiguous CA harmonic products

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |

### 6.20.3	∆TIB and ∆RIB values

No TIB and RIB are required for CA_48D intra-band CA.

### 6.20.4	REFSENS requirements

No additional REFSENS requirements are identified.

### 6.20.5	Blocking requirements

In-band blocking and out-of-band blocking requirements for intra-band contiguous CA with 1UL should apply to CA_48D.

Table 6.20.5-1: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_48D | FInterferer (Range) | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 |  |  |  |  |

Table 6.20.5-2: Out of band blocking

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_48D1 | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for this CA configuration for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

## 6.21	CA_4DL_48E_1UL_BCS0

### 6.21.1	Channel bandwidths per operating band for CA

Table 6.21.1-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_48E | - | 5,10,15,20 | 20 | 20 | 20 |  | 80 | 0 |
|  |  | 20 | 20 | 20 | 5,10,15 |  |  |  |

### 6.21.2	Co-existence studies

Table 6.21.2-1 presents DL harmonics and IMD products for Band 48 contiguous CA. As shown below, none of 2nd/3rd DL harmonics nor 2nd order IMD products will impact any 3GPP bands. 3rd order IMD products fall to UL frequency range of Band 22, 42 and 43. It should be noted Band 42 and 43 will be not used in the same region as Band 48. Furthermore, as Band 48 is a TDD band, the 3rd order IMD product will not fall to UL frequency range as BS will not transmit and receive at the same time. Note that the limits in table 6.21.2-1 are calculated assuming maximum possible spacing between the aggregated carriers in the operating band.

Table 6.21.2-1: DL harmonics and IMD products for Band 48 contiguous CA

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 3550 | 3700 |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -150 | 150 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 3400 | 3850 |

Table 6.21.2-2 gives the harmonic products for band 48 contiguous CA with 1UL. None of the harmonic products fall into the own receive band for UE, so it is concluded that there is no issue on UL harmonic interference.

Table 6.21.2-2: 1UL Band 48 contiguous CA harmonic products

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |

### 6.21.3	∆TIB and ∆RIB values

No TIB and RIB are required for CA_48E intra-band CA.

### 6.21.4	REFSENS requirements

No additional REFSENS requirements are identified.

### 6.21.5	Blocking requirements

In-band blocking and out-of-band blocking requirements for intra-band contiguous CA with 1UL should apply to CA_48E.

Table 6.21.5-1: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_48E | FInterferer (Range) | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 |  |  |  |  |

Table 6.21.5-2: Out of band blocking

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_48E1 | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for this CA configuration for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

## 6.22	CA_5DL_48F_1UL_BCS0

### 6.22.1	Channel bandwidths per operating band for CA

Table 6.22.1-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_48F | - | 5, 10, 15, 20 | 20 | 20 | 20 | 20 | 100 | 0 |
|  |  | 20 | 20 | 20 | 20 | 5, 10, 15, 20 |  |  |

### 6.22.2	Co-existence studies

Table 6.22.2-1 presents DL harmonics and IMD products for Band 48 contiguous CA. As shown below, none of 2nd/3rd DL harmonics nor 2nd order IMD products will impact any 3GPP bands. 3rd order IMD products fall to UL frequency range of Band 22, 42 and 43. It should be noted Band 42 and 43 will be not used in the same region as Band 48. Furthermore, as Band 48 is a TDD band, the 3rd order IMD product will not fall to UL frequency range as BS will not transmit and receive at the same time. Note that the limits in table 6.22.2-1 are calculated assuming maximum possible spacing between the aggregated carriers in the operating band.

Table 6.22.2-1: DL harmonics and IMD products for Band 48 contiguous CA

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 3550 | 3700 |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -150 | 150 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 3400 | 3850 |

Table 6.22.2-2 gives the harmonic products for band 48 contiguous CA with 1UL. None of the harmonic products fall into the own receive band for UE, so it is concluded that there is no issue on UL harmonic interference.

Table 6.22.2-2: 1UL Band 48 contiguous CA harmonic products

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |

### 6.22.3	∆TIB and ∆RIB values

No TIB and RIB are required for CA_48F intra-band CA.

### 6.22.4	REFSENS requirements

No additional REFSENS requirements are identified.

### 6.22.5	Blocking requirements

In-band blocking and out-of-band blocking requirements for intra-band contiguous CA with 1UL should apply to CA_48F.

Table 6.22.5-1: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_48F | FInterferer (Range) | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 |  |  |  |  |

Table 6.22.5-2: Out of band blocking

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_48F1 | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for this CA configuration for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

## 6.23	CA_4DL_46E_0UL_BCS1

### 6.23.1	Operating bands for CA

Table 6.23.1-1: Intra-band contiguous CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high | FDL_low – FDL_high |  |  |  |
| CA_46 | 46 | N/A | 5150 MHz | – | 5925 MHz | TDD |

### 6.23.2	Channel bandwidths per operating band for CA

Table 6.23.2-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_46E | - | 20 | 20 | 20 | 10, 20 | 80 | 1 |
|  |  | 10 | 20 | 20 | 20 |  |  |

NOTE: 	Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell.

### 6.23.3  Co-existence studies

Co-existence already studied when BCS0 for CA_46E was introduced.

### 6.23.4	 ΔTIB,c and ΔRIB,c values

Insertion loss values already defined when BCS0 for CA_46E was introduced.

## 6.24	CA_3DL_41A-41C_2UL_41C_BCS0

### 6.24.1	Channel bandwidths per operating band for CA

Table 6.24.1-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_41A-41C | CA_41C | 5, 10, 15, 20 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  | 60 | 0 |
|  |  | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  | 5, 10, 15, 20 |  |  |  |

NOTE: 	Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell.

### 6.24.2		Co-existence studies

Table 6.24.2-1 presents DL harmonics and IMD products for Band 41 non-contiguous CA. As shown DL 2nd harmonics might fall into Band 46 and 3rd order IMD products might fall to UL frequency range of Band 7, 30, 38 or 40. It should however be noted that Band 7 and 38 will not be used in the same region as Band 41, and also note that the limits in table 6.24.2-1 are calculated assuming maximum possible spacing between the aggregated carriers in the operating band. With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 30 and 40 BS receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 41 BS transmitters do not share the same antenna with Band 30 or 40 BS receiver.

Table 6.24.2-1: DL harmonics and IMD products for Band 41 non-contiguous CA

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 2496 | 2690 |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -194 | 194 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 2302 | 2884 |

### 6.24.3		∆TIB and ∆RIB values

TIB and RIB does not need to be defined for this combination.

### 6.24.4		REFSENS requirements

Existing combination CA_41A-41C with single uplink has REFSENS defined. No additions are identified by adding dual uplink.

## 6.25	CA_4DL_41E_2UL_41C_BCS0

### 6.25.1	Channel bandwidths per operating band for CA

Table 6.25.1-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_41E | CA_41C | 15, 20 | 15, 20 | 15, 20 |  | 20 | 80 | 0 |

NOTE: 	Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell.

### 6.25.2		Co-existence studies

Co-existence studies are not required since this combination is similar to existing combination CA_41D/41C.

### 6.25.3		∆TIB and ∆RIB values

TIB and RIB does not need to be defined for this combination.

### 6.25.4		REFSENS requirements

REFSENS requirements don’t need to be defined for this combination.

### 6.25.5		Blocking requirements

In-band blocking and out-of-band blocking requirements need to be included according to Table 6.25.5-1 and Table 6.25.5-2.

Table 6.25.5-1: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_41E | FInterferer (Range) | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |

Table 6.25.5-2: Out of band blocking

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_41E | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for this CA configuration for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

## 6.26	CA_5DL_41C-41D_2UL_41C_BCS0

### 6.26.1	Channel bandwidths per operating band for CA

Table 6.26.1-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  |  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration |  | Uplink CA configurations (NOTE 1) |  | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  |  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_41C-41D |  | CA_41C |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | 100 | 0 |

NOTE: 	Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell.

### 6.26.2		Co-existence studies

Table 6.26.2-1 presents DL harmonics and IMD products for Band 41 non-contiguous CA. As shown DL 2nd harmonics might fall into Band 46 and 3rd order IMD products might fall to UL frequency range of Band 7, 30, 38 or 40. It should however be noted that Band 7 and 38 will not be used in the same region as Band 41, and also note that the limits in table 6.26.2-1 are calculated assuming maximum possible spacing between the aggregated carriers in the operating band. With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 30 and 40 BS receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 41 BS transmitters do not share the same antenna with Band 30 or 40 BS receiver.

Table 6.26.2-1: DL harmonics and IMD products for Band 41 non-contiguous CA

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 2496 | 2690 |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -194 | 194 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 2302 | 2884 |

### 6.26.3		∆TIB and ∆RIB values

TIB and RIB does not need to be defined for this combination.

### 6.26.4		REFSENS requirements

Existing combination CA_41C-41D with single uplink has REFSENS defined, but no additions are identified by adding dual uplink.

## 6.27	CA_48A-48A_1UL_BCS0

### 6.27.1	Channel bandwidths for CA_48A-48A

Table 6.27.1-1 presents the channel bandwidths for CA_48A-48A:

Table 6.27.1-1: Channel bandwidths for CA_48A-48A

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 1) |  | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_48A-48A | - |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  | 40 | 0 |

### 6.27.2	Co-existence studies

Table 6.27.2-1 presents DL harmonics and IMD products for Band 48 non-contiguous CA. As shown below, none of 2nd/3rd DL harmonics as well as 2nd order IMD products will impact 3GPP bands. 3rd order IMD products fall to UL frequency range of Band 42, 43 and 48. It should be noted Band 42 and 43 will be not used in the same region as Band 48. Furthermore, as Band 48 is a TDD band, the 3rd order IMD product will not fall to UL frequency range as BS will not transmit and receive at the same time. It is therefore concluded there is no issue with 3rd IMD product.

Table 6.27.2-1: DL harmonics and IMD products for Band 48 non-contiguous CA

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 3550 | 3700 |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -150 | 150 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 3400 | 3850 |

### 6.27.3	REFSENS requirements

Table 7.3.1A-3 in TS 36.101 should be updated as shown in the Table 6.27.3-1 below.

Table 6.27.3-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_48A-48A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| NOTE 6: All combinations of channel bandwidths defined in Table 5.6A.1-3. NOTE 7: All applicable sub-block gap sizes. NOTE 8: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. In case of uplink sub-block is TDD intra-band contiguous CA then the uplink PCC and SCC allocations are the same as NRB_agg defined in Table 7.3.1A-1. |  |  |  |  |  |

## 6.28	CA_48A-48C_1UL_BCS0

### 6.28.1	Channel bandwidths for CA_48A-48C

Table 6.28.1-1 presents the channel bandwidths for CA_48A-48C:

Table 6.28.1-1: Channel bandwidths for CA_48A-48C

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 1) |  | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_48A-48C | - |  | 5, 10, 15, 20 | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  | 60 | 0 |
|  |  |  | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  | 5, 10, 15, 20 |  |  |  |  |

### 6.28.2	Co-existence studies

Table 6.28.2-1 presents DL harmonics and IMD products for Band 48 non-contiguous CA. As shown below, none of 2nd/3rd DL harmonics as well as 2nd order IMD products will impact 3GPP bands. 3rd order IMD products fall to UL frequency range of Band 42, 43 and 48. It should be noted Band 42 and 43 will be not used in the same region as Band 48. Furthermore, as Band 48 is a TDD band, the 3rd order IMD product will not fall to UL frequency range as BS will not transmit and receive at the same time. It is therefore concluded there is no issue with 3rd IMD product.

Table 6.28.2-1: DL harmonics and IMD products for Band 48 non-contiguous CA

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 3550 | 3700 |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -150 | 150 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 3400 | 3850 |

### 6.28.3	REFSENS requirements

Table 7.3.1A-3 in TS 36.101 should be updated as shown in the Table 6.28.3-1 below.

Table 6.28.3-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_48A-48C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| NOTE 6: All combinations of channel bandwidths defined in Table 5.6A.1-3. NOTE 7: All applicable sub-block gap sizes. NOTE 8: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. In case of uplink sub-block is TDD intra-band contiguous CA then the uplink PCC and SCC allocations are the same as NRB_agg defined in Table 7.3.1A-1. |  |  |  |  |  |

## 6.29	CA_48A-48D_1UL_BCS0

### 6.29.1	Channel bandwidths for CA_48A-48D

Table 6.29.1-1 presents the channel bandwidths for CA_48A-48D:

Table 6.29.1-1: Channel bandwidths for CA_48A-48D

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 1) |  | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_48A-48D | - |  | 5, 10, 15, 20 | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  | 80 | 0 |
|  |  |  | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | 5, 10, 15, 20 |  |  |  |

### 6.29.2	Co-existence studies

Table 6.29.2-1 presents DL harmonics and IMD products for Band 48 non-contiguous CA. As shown below, none of 2nd/3rd DL harmonics as well as 2nd order IMD products will impact 3GPP bands. 3rd order IMD products fall to UL frequency range of Band 42, 43 and 48. It should be noted Band 42 and 43 will be not used in the same region as Band 48. Furthermore, as Band 48 is a TDD band, the 3rd order IMD product will not fall to UL frequency range as BS will not transmit and receive at the same time. It is therefore concluded there is no issue with 3rd IMD product.

Table 6.29.2-1: DL harmonics and IMD products for Band 48 non-contiguous CA

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 3550 | 3700 |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -150 | 150 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 3400 | 3850 |

### 6.29.3	REFSENS requirements

Table 7.3.1A-3 in TS 36.101 should be updated as shown in the Table 6.29.3-1 below.

Table 6.29.3-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_48A-48D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| NOTE 6: All combinations of channel bandwidths defined in Table 5.6A.1-3. NOTE 7: All applicable sub-block gap sizes. NOTE 8: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. In case of uplink sub-block is TDD intra-band contiguous CA then the uplink PCC and SCC allocations are the same as NRB_agg defined in Table 7.3.1A-1. |  |  |  |  |  |

## 6.30	CA_48C-48C_1UL_BCS0

### 6.30.1	Channel bandwidths for CA_48C-48C

Table 6.30.1-1 presents the channel bandwidths for CA_48C-48C:

Table 6.30.1-1: Channel bandwidths for CA_48C-48C

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_48C-48C | - | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | 80 | 0 |

### 6.30.2	Co-existence studies

Table 6.30.2-1 presents DL harmonics and IMD products for Band 48 non-contiguous CA. As shown below, none of 2nd/3rd DL harmonics as well as 2nd order IMD products will impact 3GPP bands. 3rd order IMD products fall to UL frequency range of Band 42, 43 and 48. It should be noted Band 42 and 43 will be not used in the same region as Band 48. Furthermore, as Band 48 is a TDD band, the 3rd order IMD product will not fall to UL frequency range as BS will not transmit and receive at the same time. It is therefore concluded there is no issue with 3rd IMD product.

Table 6.30.2-1: DL harmonics and IMD products for Band 48 non-contiguous CA

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 3550 | 3700 |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -150 | 150 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 3400 | 3850 |

### 6.30.3	REFSENS requirements

Table 7.3.1A-3 in TS 36.101 should be updated as shown in the Table 6.30.3-1 below.

Table 6.30.3-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_48C-48C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| NOTE 6: All combinations of channel bandwidths defined in Table 5.6A.1-3. NOTE 7: All applicable sub-block gap sizes. NOTE 8: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. In case of uplink sub-block is TDD intra-band contiguous CA then the uplink PCC and SCC allocations are the same as NRB_agg defined in Table 7.3.1A-1. |  |  |  |  |  |

## 6.31	CA_48C-48D_1UL_BCS0

### 6.31.1	Channel bandwidths for CA_48C-48D

Table 6.31.1-1 presents the channel bandwidths for CA_48C-48D:

Table 6.31.1-1: Channel bandwidths for CA_48C-48D

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 1) |  | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_48C-48D | - |  | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | 100 | 0 |
|  |  |  | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |

### 6.31.2	Co-existence studies

Table 6.31.2-1 presents DL harmonics and IMD products for Band 48 non-contiguous CA. As shown below, none of 2nd/3rd DL harmonics as well as 2nd order IMD products will impact 3GPP bands. 3rd order IMD products fall to UL frequency range of Band 42, 43 and 48. It should be noted Band 42 and 43 will be not used in the same region as Band 48. Furthermore, as Band 48 is a TDD band, the 3rd order IMD product will not fall to UL frequency range as BS will not transmit and receive at the same time. It is therefore concluded there is no issue with 3rd IMD product.

Table 6.31.2-1: DL harmonics and IMD products for Band 48 non-contiguous CA

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 3550 | 3700 |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -150 | 150 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 3400 | 3850 |

### 6.31.3	REFSENS requirements

Table 7.3.1A-3 in TS 36.101 should be updated as shown in the Table 6.31.3-1 below.

Table 6.31.3-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_48C-48D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| NOTE 6: All combinations of channel bandwidths defined in Table 5.6A.1-3. NOTE 7: All applicable sub-block gap sizes. NOTE 8: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. In case of uplink sub-block is TDD intra-band contiguous CA then the uplink PCC and SCC allocations are the same as NRB_agg defined in Table 7.3.1A-1. |  |  |  |  |  |

## 6.32	CA_43C_BCS0

### 6.32.1 Operating bands for CA

Table 6.32.1-1: Intra-band contiguous CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_43 | 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |

### 6.32.2	Channel bandwidths per operating band for CA

Table 6.32.2-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_43C | - | 5 | 20 |  |  | 40 | 0 |
|  |  | 10 | 15, 20 |  |  |  |  |
|  |  | 15 | 10, 15, 20 |  |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |

NOTE 1: 	Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell.

### 6.32.3	Co-existence studies

No co-existence issue identified for CA_43C.

### 6.32.4	∆TIB and ∆RIB values

No TIB and RIB are required for CA_43C intra-band contiguous CA.

### 6.32.5	REFSENS requirements

In general, TDD intra-band carrier aggregation does not change the UE reference sensitivity requirement as it does not introduce any impact to the UE front end RF architecture. Further the REFSENS requirement for intra-band contiguous carrier aggregation is measured on a per CC basis and the same reference sensitivity level as that for single CC is reused on a per CC basis. Hence the introduction of CA_43C with 1 uplink does not impact the current REFSENS requirement.

### 6.32.6	Other receiver RF requirements

The other receiver RF requirements as Maximum input level, ACS, In-band blocking, Out-of-band blocking, Narrow band blocking, Spurious response, Intermodulation characteristics and Blocking characteristics for intra-band contiguous CA are specified by the CA bandwidth classes. For CA_43C, the relevant requirements for bandwidth class C could be reused.

Where, it worth to note CA_43C is considered for the following in-band and out-of band blocking CA configurations.

#### 6.32.6.1 In-band blocking

The CA configuration for CA_43C in-band blocking is listed in the following table, the table number is kept aligned with in 36.101.

Table 7.6.1.1A-2: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_1C, CA_2C, CA_3B, CA_3C, CA_5B, CA_7C, CA_8B, CA_12B, CA_23B, CA_27B, CA_38C, CA_39C, CA_40C, CA_41C, CA_40D, CA_41D, CA_42C, CA_42D, CA_40E, CA_42E, CA_42F, CA_43C, CA_66B, CA_66C, CA_70C, CA_66D | FInterferer (Range) | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 NOTE 3: Foffset is the frequency offset from the center frequency of the CC being tested to the edge of aggregated channel bandwidth. NOTE 4: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

#### 6.32.6.2 Out-of-band blocking

The CA configuration for CA_43C Out-of-band blocking is listed in the following table, the table number is kept aligned with in 36.101.

Table 7.6.2.1A-2: Out of band blocking

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_1C, CA_2C, CA_3B, CA_3C, CA_5B, CA_7C , CA_8B, CA_12B, CA_23B, CA_27B, CA_38C, CA_40C, CA_41C, CA_40D, CA_42C1, CA_42D1, CA_40E, CA_42E1, CA_42F1, CA_43C1, CA_66B, CA_66C, CA_66D, CA_70C | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for this CA configuration for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

## 6.33	CA_1A-1A_BCS0

### 6.33.1 Operating bands for CA

Table 6.33.1-1: Intra-band contiguous CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-1 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |

### 6.33.2	Channel bandwidths per operating band for CA

Table 6.33.2-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_1A-1A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |

NOTE 1: 	Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell.

### 6.33.3	Co-existence studies

Table 6.33.3-1 summarizes frequency ranges where harmonics occur due to Band 1 for UE. It can be concluded that there is no issue on harmonic interference fall down to the DL receiver.

Table 6.33.3-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 4220 | 4340 | 6330 | 6510 |

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 1 DL non-contiguous carriers can be calculated as shown in table 6.33.3-2 below:

Table 6.33.3-2: Band 1 non-contiguous DL harmonics and IMD products

| BS DL carriers | f_low | f_high |
| --- | --- | --- |
| DL frequency (MHz) | 2110 | 2170 |
| 2nd order harmonics frequency range (MHz) | 4220 | 4340 |
| 3rd order harmonics frequency range (MHz) | 6330 | 6510 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -60 | 60 |
| 3rd order IMD products | (2*f_low –f_high) | (2*f_high –f_low) |
| IMD frequency limits (MHz) | 2050 | 2230 |

It can be seen from table 6.33.3-2 that:

- no 2nd and 3rd harmonics products may fall into the BS receive band.

- no 2nd and 3rd IMD products may fall into the BS receive band.

assuming the maximum possible spacing between the aggregated carriers in the operating band.

It can be concluded that there is no issue on harmonic and IMD interference fall down to the UL receiver for Band 1 non-contiguous CA transceiver.

### 6.33.4	∆TIB and ∆RIB values

No TIB and RIB are required for CA_1A-1A intra-band non-contiguous CA.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2016-02 | 3GPP RAN4#78 | R4-160452 |  |  | Initial TR skeleton |  | 0.0.1 |
| 2016-04 | 3GPP RAN4#78bis | R4-161931 |  |  | Implemented TP´s from RAN4 #78: R4-160495, “Introduction of Rel-14 intra-band scope as agreed in basket WID at RAN #70”, (Ericsson) R4-161091, “TP for TR 36.714-00-00: Intra-band Band 66 carrier aggregation combinations” (Qualcomm) Corrections made for CA_66D and CA_66A-66C to bandwidths tables from TP R4-161091 to align with corresponding tables in WID RP-152250. | 0.0.1 | 0.0.2 |
| 2016-05 | 3GPP RAN4#79 | R4-163717 |  |  | Implemented TP’s from RAN4 #78bis: R4-161911, ”Introduction of Rel-14 intra-band scope as agreed in basket WID at RAN #71”, (Ericsson) R4-161949, “TP to TR 36.714-00-00: Additional bandwidth combination set for CA_7A-7A”, (CHTTL) | 0.0.2 | 0.1.0 |
| 2016-08 | 3GPP RAN4#80 | R4-165630 |  |  | Comments from ETSI EditHelp have been implemented Implemented TP’s from RAN4 #79: R4-163379, “Reference sensitivity for 5CC CA in Band 42”, (CATT) R4-163380: “Maximum input level for 5DL CA in Band 42”, (CATT) R4-163381: “Adjacent channel selectivity for 5DL CA in Band 42”, (CATT) R4-163382: “Blocking Min Requirements for 5DL CA in Band 42”, (CATT) R4-163383: ”Out-of-band Blocking for 5DL CA in Band 42”, (CATT) R4-163384: “Narrow-band Blocking for 5DL CA in Band 42”, (CATT) R4-163385: “Spurious Response for 5DL CA in Band 42”, (CATT) R4-163386: “Receiver intermodulation for 5DL CA in Band 42”, (CATT) R4-163387: “Rx Image for 5DL CA in Band 42”, (CATT) R4-164154: “Channel bandwidths for intra-band contiguous CA in B42 for 5DL”, (CATT) R4-163957: “TP for TR 36.714-00-00: operating bands, channel bandwidths for CA_46C”, (Huawei, Hisilicon) R4-163958: “TP for TR 36.714-00-00: operating bands, channel bandwidths for CA_46D”, (Huawei, Hisilicon) | 0.1.0 | 0.2.0 |
| 2016-10 | 3GPP RAN4#80bis | R4-167448 |  |  | Implemented TP’s from RAN4 #80: R4-166857: “CA_12A-12A configurations for REFSENS test” (Intel, Nokia) R4-165600: “TP for TR 36.714-00-00: Additional bandwidth combination set CA_7A-7A”, (SK Telecom) R4-166858: “TP on TR 36.714-00-00 for CA_40 and CA_41”, (CMCC) | 0.2.0 | 0.3.0 |
| 2016-11 | 3GPP RAN4#81 | R4-1609185 |  |  | Implemented TP’s from RAN4 #80bis: R4-167349, “TP for TR 36.714-00-00: operating bands, channel bandwidths, co-existence studies, delta Tib/Rib values for CA_3A-3A_BCS2”, (Samsung) R4-167351, “TP for TR 36.714-00-00: Intra-band contiguous CA combination of Band 3”, (Samsung) R4-167491, “Requirements for CA_3DL_40D_1UL_BCS1 and CA_4DL_40E_1UL_BCS0”, (Ericsson) R4-167492, “Requirements for CA_2DL_40A-40A_1UL_BCS1, CA_3DL_40A-40C_1UL_BCS0 and CA_4DL_40C-40C_1UL_BCS0”, (Ericsson) R4-167436, “TP on TR 36.714-00-00 for CA_40 and CA_41”, (CMCC, Qualcomm) R4-167466, “Requirements for 66A-66B, TP to 36.714-00-00”, (Ericsson, Verizon) R4-167488, “Requirements for CA_2DL_ CA_46A-46A_0UL_BCS0, CA_3DL_ CA_ 46A-46C_0UL_BCS0 and CA_4DL_ CA_ 46A-46D_0UL_BCS0”, (Ericsson, T-Mobile USA) R4-167494, “Requirements for CA_5A-5A_BCS1 and CA_5B_BCS1”, (Ericsson) R4-167446, “TP Intra-band scope 36.714-00-00”, (Ericsson) | 0.3.0 | 0.4.0 |
| 2017-02 | 3GPP RAN4#82 | R4-1700746 |  |  | Implemented TP’s from RAN4 #81: R4-1609192, “Requirements to complete the Intra-band CA combination 12A-12A”, (Ericsson, US Cellular) R4-1610759, “TP for TR 36.714-00-00: REFSENS requirement for CA_3A-3A_BCS2”, (Samsung, Huawei, HiSilcon) R4-1609349, “TP for TR 36.714-00-00: Blocking requirements for CA_3B”, (Samsung) | 0.4.0 | 0.5.0 |
| 2017-04 | 3GPP RAN4#82 bis | R4-1702728 |  |  | Implemented TP’s from RAN4 #82: R4-1702439, “TP to TR 36.714-00-00: Uplink intraband 3CC MPR”, (Nokia, Skyworks Solutions inc., Alcatel-Lucent Shanghai Bell) R4-1702214, “TP to include CA_2DL_48C_1UL_BCS0”, (Ericsson, Nokia) R4-1702215, “TP to include CA_3DL_48D_1UL_BCS0”, (Ericsson, Nokia) R4-1702216, “TP to include CA_4DL_48E_1UL_BCS0”, (Ericsson, Nokia) R4-1702217, “TP to include CA_5DL_48F_1UL_BCS0”, (Ericsson, Nokia) R4-1700764, “TP to include CA_46E_BCS1”, (Ericsson) R4-1702218, “TP to include CA_3DL_41A-41C_2UL_BCS0”, ((Ericsson, Sprint) R4-1702219, “TP to include CA_4DL_41E_2UL_41C_BCS0”, (Ericsson, Sprint) R4-1702220, “TP to include CA_5DL_41C-41D_2UL_41C_BCS0”, (Ericsson, Sprint) R4-1700824, “TP to 36.714-00-00: Introduction of CA_48A-48A”, (Nokia, Ericsson) R4-1700825, “TP to 36.714-00-00: Introduction of CA_48A-48C”, (Nokia, Ericsson) R4-1700826, “TP to 36.714-00-00: Introduction of CA_48A-48D”, (Nokia, Ericsson) R4-1700827, “TP to 36.714-00-00: Introduction of CA_48C-48C”, (Nokia, Ericsson) R4-1700828, “TP to 36.714-00-00: Introduction of CA_48C-48D”, (Nokia, Ericsson) R4-1701286, “TP for TR 36.714-00-00: operating bands, channel bandwidths for CA_43C_BCS0”, (Huawei, Hisilicon) R4-1702221, “TP for TR 36.714-00-00: co-existence studies, UE requirements relaxation for CA_43C_BCS0”, (Huawei, Hisilicon) R4-1701288, “TP for TR 36.714-00-00: operating bands, channel bandwidths for CA_1A-1A_BCS0”, (Huawei, Hisilicon) R4-1701289, “TP for TR 36.714-00-00: co-existence studies, UE requirements relaxation for CA_1A-1A_BCS0”, (Huawei, Hisilicon) | 0.5.0 | 0.6.0 |