3GPP TSG-RAN WG4 Meeting #82bis	 R4-1702800

Spokane, US, 3 – 7 Apr, 2017

Agenda Item:	10.2.1.2

Source: 	Samsung

Title: 	Discussion on 3.5GHz band definition

Document for:	Discussion

Introduction

In RAN#75 NR WI was approved in [1] attached with NR bands and LTE-NR band combinations. And 3.3-4.2GHz is listed as a promising frequency range for NR application requested by many operators:

| Frequency range /LTE band | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- | --- |
| 3.3-4.2 GHz | REL-15 | TBD | TBD | NTT DOCOMO, KDDI, SBM, CMCC, China Unicom, China Telecom, KT, SK Telecom, LG Uplus, Etisalat, Orange, Telecom Italia, British Telecom, Deutsche Telekom | new |

3.3-4.2GHz frequency range gathers the potential spectrum allocation for 5G deployment in Japan, Korea, China and EU around 3.5GHz. It would be attractive solution to define this entire frequency range as one NR band which was also proposed during NR SI phase considering global ecosystem. Meanwhile, the impact on implementation aspect also needs to be considered. In this contribution we would like to share some initial thinking on the band plan for 3.5GHz around frequency ranges.

Discussion

There would be at least two potential alternatives proposed with the target to define a single band on frequency range of 3.3-4.2GHz as below:

- Solution 1: single band on 3.3-4.2GHz

- Solution 2: single band on 3.3-4.2GHz with two sub-blocks(3.3-3.8GHz; 3.6-4.2GHz)

The main motivation to have solution 2 on top of solution 1 would be the BWR limitation on PAE of PA. As shown by in several contributions [3][4] provided in SI phase as the typical limitation on 3.5GHz range would be around 15% BWR. Even though the simulation and measurement results shared in [5] show that may be possibility to support the entire 3.3-4.2GHz band with legacy Band 42&43 PA, there is still more aspects should be further assessed. Furthermore, according to initial survey on implementation aspect, the potential challenging on LO (integrated PLL) may need to be taken into account in further study. And whether and how it would have impact on specific RF/demodulation requirement should be considered as well.

The initial comparison between two solutions is shown as below

|  | Solution 1 | Solution 2 |
| --- | --- | --- |
| Cons | Uncertainty on Implementation feasibility Potential performance loss and cost increase | Implementation complexity Potential restriction on some intra-band CA case/limitation on contiguous transmission across two sub-blocks |
| Pros | harmonization on 3.5GHz UE ecosystem | harmonization on 3.5GHz UE ecosystem |

Furthermore, regarding RF requirement for both solutions at least following aspect should be studied especially:

- Tolerance on maximum output power, which may relate to filter assumption and performance such as insertion loss, ripple, etc.

- MPR/A-MPR, which relates to UE transmitter non-linearity(especially PA performance) and regional emission  requirement if any

- Reference sensitivity, which relates to NF(whether 9dB NF could be reused for this new band), SNR(according to physical layer design), beam forming gain(whether applicable for below 6GHz ) and RF filer performance(any additional relaxation due to IL of filter)

- Additional requirement to co-existence with legacy LTE band as mentioned in WID

Conclusion

In this contribution we provide preliminary on two solutions on how to define 3.3-4.2GHz as a single NR band. According to discussion, more analysis would be needed for RAN4 final decision on this frequency range. Furthermore, after we achieve the band definition there are still significant RF impact would be expected and should be discussed in RAN4.

Reference

[1] RP-170847, New WID on New Radio Access Technology

[2] R4-1700589, On defining NR spectrum in sub-6GHz

[3] R4-1700886, On band harmonization feasibility

[4] R4-1700957, 3.5GHz Band for NR

[5] R4-1701832, 3.5GHz band consideration for NR