3GPP TSG-RAN WG4 Meeting RAN4#82bis	            R4-1702996

Spokane, US, 3-7 April 2017

Agenda item:	10.4.1.3 - REFSENS and MSD evaluation assumption

Source: 	Intel Corporation

Title: 	UE sub-6 GHz REFSENS considerations & proposals

Document for:	Discussion

# Introduction

REFSENS is one of the critical parameters for RX chain of the user equipment. This document tries to focus on sub-6 GHz REFSENS. Although band definition has not been made by 3GPP, the way in which REFSENS should be defined is highlighted in the document.

# Discussion on sub-6 GHz REFSENS

The above definition was adapted from LTE standards. For new radio 5G sub-6 GHz, the following frequency bands are discussed with TDD as duplex mode.

| 5G Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 5G_sub6_1 | 3300MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| 5G_sub6_2 | 4400MHz | – | 4990 MHz | 4400MHz | – | 4990 MHz | TDD |

The reference sensitivity level is the minimum mean received signal strength applied to all UE antenna ports at which there is sufficient SINR for the specified modulation scheme to meet a minimum throughput requirement of 95% of the maximum possible.

REFerence SENSitivity (REFSENS) is a range of values that can be calculated using the

Formula:

REFSENS (dBm)  = kTB + NF + SNR + IM – MIMO gain ------------ (eq. 1)

- kTB is the thermal noise level

- specified bandwidth (B)

- NF is the prescribed maximum noise figure for the receiver

- SNR is the requirement for the chosen modulation and coding scheme

- IM is the implementation margin

- MIMO gain represents the diversity gain (2RX, 4RX)

By following description in equation 1, the REFSENS values are calculated as shown below for 2RX and 4RX separately. Full amount of diversity gain is not exploited because it is only in theory we get that amount of 100% diversity gain.

There may be band specific exceptions which need to be further studied. Sub-band sensitivity requirement is FFS. High SNR sensitivity requirement is FFS. Measurement reference channel definition is FFS.

# Conclusion

Proposal 1: The values in following table should be considered as baseline for 3GPP NR REFSENS spec.

Proposal 2: Sub-band sensitivity requirement is FFS. High SNR sensitivity requirement is FFS. Measurement reference channel definition is FFS.

# References

- 3GPP TS 36.101 V14.2.1 (2017-01)

- 3GPP TR 38.803 V1.1.0 (2017-01)