3GPP TSG-RAN WG4 Meeting RAN4#82bis	            R4-1703174

Spokane, US, 3-7 April 2017

Agenda item:	10.4.1.1- ACLR and ACS

Source: 	Intel Corporation

Title: 	UE sub-6 GHz ACS considerations and proposals

Document for:	Discussion

# Introduction

This document proposes methodology and initial values to define sub-6 GHz ACS for RX characterization.

# Discussion on sub-6 GHz ACS

Excerpt from [1]: Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a 5G signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s)

RAN 4 NR should consider the following tables as base line for defining sub-6 GHz ACS values. In tables 1-3, the bandwidth of interfering signal is kept equal to bandwidth of wanted RX signal.

There is a second possibility to define the ACS where BW of interfering signal can be very small than bandwidth of wanted signal. In such a case, if 5 MHz is kept as bandwidth of interfering signal for 1xcc CBW of 50 MHz and 100 MHz, then relaxation is required. So, new set of ACS values would be as follows:

- ACS (in dB) for 50 MHz channel should be  19 dB

- ACS (in dB) for 100 MHz channel should be  13 dB

# Conclusion

Proposal 1:  RAN 4 should study further the following 2 possibilities and agree on ACS definition for sub-6 GHz

- Table 1-3 in this document without higher relaxation on ACS

- ACS where wanted signal BW >> interferer signal BW, but ACS is highly relaxed

# References

- 3GPP TS 36.101 V14.2.1 (2017-01)

- 3GPP TR 38.803 V1.1.0 (2017-01)