3GPP TSG-RAN WG4 Meeting #82bis	R4-1703823

Spokane, Washington, USA, 3 - 7 April, 2017

Agenda Item:	10.4.3.3

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Proposal on mmWave NR BS Receiver Dynamic Range (urban macro scenario)

Document for:	Approval

# 1.	Introduction

mmWave NR BS receiver dynamic range was discussed in RAN4#82, and a way forward to further study the noise floor rise was agreed [1].

This contribution provides the simulation results per the agreed way forward, and a proposal for the mmWave NR BS receiver dynamic range.

# 2.	Discussion

Figure 1 shows the UL interference-over-thermal (IOT) simulation results for the 30 GHz urban macro scenario using the existing simulation assumptions of WP5D coexistence study captured in the TR 38.803 [2]. Here IOT is defined as the ratio between the total interference (with or without interfering system) and the BS noise floor. Moreover, 17 dB UE ACLR, 23.5 dB BS ACS and 10 dB BS NF are used as in the response to WP 5D [2], and 200 m ISD is used as this is a worse case compared to 300 m ISD in term of UL interference due to the shorter ISD.

(a) 0 dB UL TPC error

(b) 9 dB UL TPC error

Figure 1: CDF of UL IOT of victim BS

It can be seen in Figure 1 that adopting the 20 dB receiver dynamic range requirement (currently specified for Wide Area E-UTRA BS) will provide sufficient protection for NR BS receiver against intercell interference from own and adjacent systems. On the other hand, a more relaxed requirement would still be sufficient (e.g. 10 dB for 99.3 %-tile) and could ease implementation and reduce cost and complexity. Hence it is proposed to use the 20 dB receiver dynamic range requirement as a starting point for the mmWave urban macro scenario, and further consider a more relaxed requirement to ease implementation cost and complexity while still providing sufficient protection for NR BS receiver against intercell interference from own and adjacent systems. Note that relaxing the receiver dynamic range requirement has the disadvantage of making the test more sensitive to the BS NF.

# 3.	Conclusion and proposals

This contribution has provided the UL IOT simulation results for the mmWave urban macro scenario per the agreed way forward, and proposed to use the 20 dB receiver dynamic range requirement as a starting point for this scenario, and further consider a more relaxed requirement to ease implementation cost and complexity while still providing sufficient protection for NR BS receiver against intercell interference from own and adjacent systems.

# References

[1]	R4-1702040, “WF on noise floor rise of ICS and dynamic range requirement”, ZTE, CATT, Nokia, NTT DoCoMo, Huawei.

[2]	3GPP TR 38.803 v2.0.0, “Study on New Radio Access Technology; RF and co-existence aspects (Release 14)”.