3GPP TSG-RAN WG4 Meeting #82bis	R4-1703826

Spokane, Washington, USA, 3 - 7 April, 2017

Agenda Item:	10.4.3.3

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Proposal on mmWave NR BS Receiver In-band Blocking (urban macro scenario)

Document for:	Approval

# 1.	Introduction

mmWave NR BS receiver in-band blocking was discussed in RAN4#82, and a way forward to further study the interfering signal power level was agreed [1].

This contribution provides the simulation results per the agreed way forward, and a proposal for the interfering signal power level for the mmWave NR BS receiver in-band blocking.

# 2.	Discussion

It is stated in the approved NR WIDS that OTA RF requirements shall be specified for BS radio transmission and reception above 24GHz [2]. Hence the interfering signal power level for the mmWave NR BS receiver in-band blocking shall be specified as OTA signal power level.

Figure 1 shows the OTA received blocking signal power (i.e. no BS antenna gain or loss is applied) simulation results for the 30 GHz urban macro scenario using the existing simulation assumptions of WP5D coexistence study captured in the TR 38.803 [3]. Here 10 dB BS NF are used as in the response to WP 5D [3], and 200 m ISD is used as this is a worse case compared to 300 m ISD in term of OTA received blocking signal power due to the shorter ISD.

Figure 1: CDF of OTA received blocking signal power of victim BS

It can be seen in Figure 1 that specifying -62 dBm (-62.8363 dBm at 99.99 %-tile) OTA received blocking signal power will provide sufficient protection for NR BS receiver against in-band interference from the interfering system. Hence it is proposed to use the -62 dBm OTA interfering signal power level for the NR BS receiver in-band blocking as a starting point for the mmWave urban macro scenario.

# 3.	Conclusion and proposals

This contribution has provided the OTA received blocking signal power simulation results for the mmWave urban macro scenario per the agreed way forward, and proposed to use the -62 dBm OTA interfering signal power level as a starting point for the NR BS receiver in-band blocking for this scenario.

# References

[1]	R4-1702352, “WF on NR BS receiver requirement”, ZTE, CATT, NEC.

[2]	RP-170855, “New WID on New Radio Access Technology”, NTT DOCOMO, INC.

[3]	3GPP TR 38.803 v2.0.0, “Study on New Radio Access Technology; RF and co-existence aspects (Release 14)”.