3GPP TSG-RAN WG4 Meeting #82Bis	R4-1704092

Spokane, US, 3 - 7 April, 2017

Agenda item:	7.2.2

Source:	Dish Network

Title:	TP for TR 36.714-02-01:  ΔTIB,c  ΔRIB,c, and MSD for CA_66A-70A

Document for:	Approval

# 1	Introduction

This contribution provides a TP to capture TIB, RIB for CA_66A-70A. MSD is FFS.

# 2	Discussion

The frequency arrangements are shown in below.

B66: UL 1710-1780MHz, DL 2110-2200MHz

B70: UL 1695-1710MHz, DL 1995-2020MHz

The uplinks are adjacent with no gap, thus a triplexer instead of a quadplexer is used to combine these bands.

We contacted several filter vendors to get triplexer data. The data is collected in tables below. Please note that some of the data is in typical conditions (TYP) and some is worst case (WC).

Table 1 Insertion Loss for CA_66A-70A

Table 2 Isolation for CA_66A-70A

We note that one of the vendors provided information for both conditions, one provided IL for both conditions and isolation for typical only, and one provided information for worst case only. We have calculated averages for typical conditions and worst case conditions separately. These averages are not directly comparable with each other because they use different data mixes.

The average worst case B66 IL is 3.3dB for TX and 2.7dB for RX. The average worst case B70 IL is 3.3dB for TX and 2.1dB for RX. The difference to respective single band duplexer IL is <1dB for TX and <0.5dB for RX. Thus, the generic 3GPP framework 2 values for can be used for both bands:

Conclusion 1: TIB=0.5dB and RIB=0dB for both B66 and B70

MSD values are FFS.

# 3	TP

## ----- Start of Text proposal -----

### 6.52.4	ΔTIB,c and ΔRIB,c values

Triplexer data is collected in table below.

Table 6.52.4-1 Insertion loss for CA_66A-70A

The average worst case B66 IL is 3.3dB for TX and 2.7dB for RX. The average worst case B70 IL is 3.3dB for TX and 2.1dB for RX. The difference to respective single band duplexer IL is <1dB for TX and <0.5dB for RX. Thus, the generic 3GPP framework values for can be used for both bands.

For the UE which supports CA_66A-70A, the ΔTIB,c and ΔRIB,c  are defined for applicable bands in Table 6.52.4-2 and 6.52.4-3 respectively.

Table 6.52.4-2: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_66A-70A | 66 | 0.5 |
|  | 70 | 0.5 |

Table 6.52.4-3: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_66A-70A | 66 | 0 |
|  | 70 | 0 |

### 6.52.5	Reference sensitivity

Isolation values are collected in table below.

Table 6.52.5-1 Isolation for CA_66A-70A

MSD values are FFS.

----- End of Text proposal -----

# References

- RP-160945, “Revised WID: LTE Advanced inter-band CA Rel-14 for 2DL/1UL”, Qualcomm, 3GPP RAN#81, Reno, Nevada, USA, November 2016

- R4-146833, “Additional framework for Interband CA relaxations”, Nokia Corporation, Telecom italia, Sprint, Ericsson, TeliaSonera, Orange, 3GPP RAN4#72bis, Singapore, October 2014

- R4-1703523, “TP for TR 36.714-02-01:  ΔTIB,c  ΔRIB,c, and MSD for CA_66A-70A”, DISH network, 3GPP RAN4#82bis