3GPP TSG-RAN WG4 Meeting #83						R4-1704529

Hangzhou, China, 15 - 19 May, 2017

Source:		NTT DOCOMO, INC., SGS Wireless

Title:	MIMO OTA measurement results for band 1 and 19

Agenda Item:	7.11.2

Document for:	Discussion

Introduction

This contribution provides measurement result of LTE band 1 and 19 for defining the final requirement of MIMO OTA. There are six samples. These results should be a base of TRMS requirement.

Test Conditions

The measurement is done at SGS Taiwan which is CATL. Test condition aligns with [1] and same chamber is used as [2]. Information of measurement equipment is shown in Table 1. 6-samples are measured in this contribution, which covers several LTE bands.

Table 1.  Measurement equipment

| Equipment | Manufacturer |
| --- | --- |
| Anechoic chamber | MVG Star MIMO |
| Channel emulator | Keysight Technology Anite F32-16C |
| BS emulator | Anritsu MT8820C |
| Network Analyzer | Agilent E5071C |

Test results

measurement results

Table 2 is summary of measurement results of TRMS for band 1 and 19.

Table 2.  Summary for CDF data [4].

| Device and Band | Number of UE positions/rotations that fail to reach target TP |  | Inverse Averaged FOM [dBm/15kHz] |  |
| --- | --- | --- | --- | --- |
|  | 70% | 95% | 70% | 95% |
| Sample 1 Band 1 | 0 | 0 | -100.07 | -98.76 |
| Sample 1 Band 19 | 0 | 0 | -96.11 | -94.82 |
| Sample 2 Band 1 | 0 | 0 | -103.97 | -102.92 |
| Sample 2 Band 19 | 0 | 0 | -96.00 | -94.64 |
| Sample 3 Band 1 | 0 | 0 | -101.75 | -100.51 |
| Sample 3 Band 19 | 0 | 0 | -96.39 | -95.03 |
| Sample 4 Band 1 | 0 | 0 | -99.35 | -98.04 |
| Sample 4 Band 19 | 0 | 0 | -96.33 | -95.10 |
| Sample 5 Band 1 | 0 | 0 | -100.15 | -98.96 |
| Sample 5 Band 19 | 0 | 0 | -97.10 | -95.87 |
| Sample 6 Band 1 | 0 | 0 | -102.42 | -101.33 |
| Sample 6 Band 19 | 0 | 0 | -96.00 | -94.56 |

The details of band 1 are shown in below excels.

The details of band 19 are shown in below excels.

Conclusion

In this contribution, evaluation of 6-samples for band 1 and 19 are provided. Further discussion, and analysis are very helpful to promote this work.

References

[1]    TR 37.977, V14.3.0, “Verification of radiated multi-antenna reception performance of UEs,” 3GPP RAN #75, March 2017

[2]    R4-1702828, Measurement results of PAD devices for lab alignment test activity, NTT DOCOMO, SGS Wireless, RAN4#82bis