3GPP TSG-RAN WG4 #83	R4-1704688

Hangzhou, China, 15 – 19 May, 2017

# Source:	Intel Corporation

# Title:	On supporting channel bandwidth and sub-carrier spacing

# Agenda Item:	10.3.1

# Document for:	Discussion

# 1	Introduction

In RAN4#82Bis, minimum and maximum channel bandwidths [1, 2] and sub-carrier spacing [3] had been agreed for both below-6 GHz and above-24 GHz in which

- Min and Max CBW

- Sub-carrier spacing

From implementation point of view, it would be beneficial to have an agreement on supporting CBW for each of supporting SCS. In this document, we would like to discuss on them.

# 2	Discussion

# 2.1 Sub-6 GHz

For sub-6 GHz range, 15 kHz, 30 kHz and 60 kHz are being supported as sub-carrier spacing and minimum CBW is 5 MHz and maximum CBW is 100 MHz based on the agreement. Considering 4k FFT size as the maximum, we could populate which CBW can be supported for each sub-carrier spacing.

Table 1 shows a summary of this effort. For example, 100 MHz is the maximum CBW for below-6 GHz. However, considering max. FFT size of 4k along with 15 kHz SCS, possible max. CBW is 50 MHz and 100 MHz CBW cannot be supported with 15 kHz SCS. The same way is applied to the rest of CBW. The CBWs between min and max CBW here are examples and RAN4 should discuss further on the exact supporting CBW.

Table 1 Supporting CBW for each sub-carrier spacing for below-6 GHz

Observation 1: CBW larger than 50 MHz is not being supported with 15 kHz SCS for below-6 GHz.

2.2 Above-24 GHz

Table 2 shows the mapping table for above-24 GHz. Considering the max FFT size is 4k, maximum supporting CBW with 60 kHz SCS is 200 MHz and CBW larger than 200 MHz is not being supported with 60 kHz SCS.

Table 2. Supporting CBW for each of sub-carrier spacing for above-24 GHz

Observation 2: The CBW larger than 200 MHz is not being supported with 60 kHz SCS for above-24 GHz

Proposal 1: RAN4 should have an agreement which CBW is being supported for each of SCS at least for min and max CBW cases for both below-6 GHz and above-24 GHz.

# 3	Conclusions

In this document, the following observations and proposals were made:

Observation 1: CBW larger than 50 MHz is not being supported with 15 kHz SCS for below-6 GHz.

Observation 2: The CBW larger than 200 MHz is not being supported with 60 kHz SCS for above-24 GHz.

Proposal 1: RAN4 should have an agreement which CBW is being supported for each of SCS at least for min and max CBW case for both below-6 GHz and above-24 GHz.

# 4	Reference

- R4-1703100, “LS Reply on NR minimum carrier bandwidth”, Qualcomm, RAN4#82Bis

- R4-1704412, “WF on maximum channel bandwidth”, Qualcomm, Vodafone, Dish, RAN4#82Bis

- R4-1704401, “WF on Subcarrier spacing for NR”, Qualcomm, RAN4#82Bis