3GPP TSG-RAN WG4 #83	R4- 1705387

Hangzhou, China, 15th –19th May 2017

Source:	Ericsson

Title:	TP for TR 37.843: Section 5.6.6 Spurious Emissions

Agenda item:	9.12.3.1

Document for:	Approval

# 1.	Introduction

In this contribution a proposed test method is described to capture spurious emissions testing.   In addition to keeping different sparse grids to estimate TRP in the goal of reducing test time but not compromising TRP accuracy, for spurious emissions it has been stated before [1] that two cardinal cuts or the orthogonal cuts can be used to estimate TRP.

# 2.	Discussion

Some careful considerations are needed to be understood when discussions of unwanted emissions testing.  In this contribution, the aspect of spurious emissions will be the focus.  The transmitter spruious emissions limits for conducted requierments apply for the frequency range from 9 kHz to 12.75 GHz.  The lower limit 9 kHz is in line with TS 37.105 however, the question remains if this limit should be changed to 30 MHz to keep in line with ITU SM.329 recommendation [2].  Additionally, the current EMC requirement is defined from 30 MHz as a lower limit.  The decision within RAN4 was to merge these two requirements, and as such the frequency ranges would make more sense to keep in line.

In past contributions [3] optimization of test time and comlexity was discussed by showing that a sparse grid or two orthgonal cuts can be used to get effective TRP estimation for spurious emissions.  It was also shown that different requirements can utilize different sampling grid techniques to eliminate the need for a measurement of a full sphere.  In [4] one such detailed approach is shown that can be used in conjunction of sampling principle planes.  The method is enhanced by a pattern multiplication method as a post processing step to ensure accuracy of the TRP estimation.  This is just one method and other methods such as reverberation chambers or EMC radiated emissions test chambers should also be considered as other available methods.

The proposed test procedure described in more detail in [4] is based upon determining the TRP as the average value of a relevant measured quantity such as EIRP. This method is useful both in the nearfield and farfield. The usability in the ractive nearfield is still for further study. Since the ultimate goal of measuring spurious emissions is to quantify whether a certain level is exceeded or not, over-estimating the TRP is useful to reach a positive verdict, if measurement time is significantly reduced. In the proposed method this is done by various techniques to over-estimate the average EIRP by correctly picking subsets of measurement points. For a given frequency band, i.e., wherein a significant emission is present, the method starts with calculating a frequency-dependent Nyquist sampling criterion for a full sphere measurement. For electrically small devices (dimensions of some wave lengths) a full sphere measurment can be used and the TRP calculated accurately. For electrically large devices the full sphere can also be used, however the estimated measurement time can easily be several weeks. For this case a sequencue of subsets are defined: direction of peak EIRP, coarse and iteratively finer sampling on two orthogonal cuts through the peak, coarse and iteratively finer sampling on a full sphere. At every iteration and grid type the TRP is (over)-estimated as an average value of the measure EIRP values. The sequence of estimated TRP values have been numerically demonstrated to decrease monotonically. In the two cuts sampling stage, a pattern multiplication method can be used if certain requirements on the emission and the antenna array topology are met. There are different methods and approaches on how to sample values around the DUT to accurately capture the TRP estimate.  Another sampling technique is to measure on a full sphere with 15 degree sampling resolution and then calculate the average of all values.  Whichever method of sampling is choosen the grid shall be adapative and frequency dependent to be able to fulfil a TRP estimate which is accurate enough to fulfil the requirement at the certain frequencies.

Spurious emissions requirements has a large frequency range and vary in emissions power level, for example location conducted requirements are measured at -98 dBm/100kHz.  A near field measurement may be more approperiate in this situation to reduce the path loss.  This ensures that the above method can handle the large range over frequency and power density.

# 3.	Conclusions

In this contribution a method to measure spurious emissions was briefly discussed.  This continuation was a continuation of the discussions from previous RAN4 meetings which described a sampling grid of 15 degrees resolution could achieve an accurate TRP estimation.  Additionally a sequence of subsets can be used: direction of peak EIRP, coarse and iteratively finer sampling on two orthogonal cuts through the beam peak, coarse and iterative finer sampling on a full sphere.  A gradual increase of granularity is done in order to ensure an accurately acceptable TRP value.

# 4.	References

R4- 1701468, “TP for TR37.843: Adding background and OTA information for spurious emissions”, Ericsson

Recommendation ITU-R SM.329: "Unwanted emissions in the spurious domain".

R4- 1609979, “Further elaboration on practical measurement Grids for ACLR and unwanted emissions”, Ericsson

R4- 1704855, “On fundamental aspects related to measuring TRP emissions”, Ericsson

[Start of Text Proposal]

#### 5.6.6.2	Transmitter spurious emissions OTA requirement

The metric used to capture spurious emissions OTA is total radiated power (TRP).  In order that the requirement is measurable the total power is defined as the sum of the EIRP at a number of discrete directions around the sphere as follows:

To capture properly EIRP, EIRPd, p1 and EIRPe, p2 is associated to two orthogonal polarizations.

The frequency range from 30 MHz to the 5th harmonic of supported highest carrier frequency should be considered.  For Cat A -13 dBm/100 kHz should be considered within this range.  For Cat B -36 dBm/100 kHz should be considered for 30 MHz – 1 GHz and similarly for -30 dBm/MHz between 1 GHz to the 5th harmonic.

Transmitter spurious emissions such as co-existence and co-location are FFS.

[End of Text Proposal]