3GPP TSG-RAN WG4 #83	R4-1705630

# Hangzhou, China, 15th – 19th May 2017

Agenda Item:	7.16.5.2

Source:	Ericsson

Title:	Further discussion on the V2X demodulation

Document for:	Discussion

# Introduction

In RAN4#82bis, demodulation test cases were discussed for V2X. There are still quite many open issues for V2X demodulation. In this paper, we share our view on these open issues.

# PSSCH and PSCCH decoding processing capability test

## PSSCH decoding processing capability test

In [1], we have the following agreements:

- Introduce new test case for PSSCH decoding capability requirement, to verify SL soft buffer management and the maximum number of bits per TTI supported by the UE, wherein

  - Maximum number of bits per TTI

    - The maximum number of sidelink transport block bits received within a TTI is set to 31704.

    - The maximum number of bits of a single sidelink transport block is 31704.

  - The total SL soft buffer size is 737280.

To verify UE can handle the maximum number of bits per TTI and the maximum number of bits of a single sidelink transport block, it is quite straight forward. We can use the transmission parameters shown in Table 1.

Table 1: Transmission parameters in the test

| Parameters | Value | Note |
| --- | --- | --- |
| Bandwidth | 20 MHz |  |
| numSubchannel | 20 |  |
| sizeSubchannel | 5 |  |
| Number of allocated subchannels | 20 | Corresponding to 96 PRBs |
| I_TBS | 16 | Corresponding the TBS is 31704 |

Regarding the sidelink soft buffer size, the rational behind of 737280 is based on [2]:

The maximum number of coded bits per TTI can be calculated using maximum of 96 RBs for PSSCH using 16QAM modulation and rate matched to 10 symbols. The maximum number of coded bits is thus 96 * 12 * 4 * (14-4) = 46080. Further, there is maximum allowed gap between first transmission and retransmission is 15. Thus the UE has to buffer for 16 TTIs before start of decoding (i.e. when both of the redundancy versions of the first HARQ process are received). The total soft buffer size is then 16 TTI x Maximum coded bits per TTI = 16 x 46080 = 737280.

Thus, in order to verify the soft buffer management, we must set proper SNR to have the following characteristics:

- The first transmission is almost not decodable to force UE to buffer the first 15 TBs

- The second transmission is decodable

Otherwise, if UE don’t buffer so many bits, it still can pass the soft buffer test. This soft buffer test is different from legacy soft buffer test. In legacy soft buffer test, noise free is set and maximum throughout is set as the metric for the test. But in this soft buffer test, noise is needed.

Proper SNR need to be set to reduce the decode probability for the first transmission to force UE to buffer the first 15 TBs in the soft buffer management test.

## PSCCH decoding processing capability test

According to the agreements in [1], we agreed to introduce new test cases for PSCCH decoding capabilities (X=10 and X=20) requirements. Per UE capability definition,

- UE is not expected to attempt to decode more than X PSCCHs in a subframe. UE is able to decode up to X PSCCH in a subframe.

- UE is expected to attempt to decode at least Y RBs per subframe counting both PSCCH and PSSCH decoding RBs.

There are two types of UE capability are defined

- Type 1: X=10, Y=100

- Type 2: X=20, Y=136

For the first type UE, the test setup is quite simple. The test parameters can be given as Table 2.

Table 2: Test parameters for X=10 and Y=100

| Parameters | Value | Note |
| --- | --- | --- |
| Bandwidth | 20 MHz |  |
| numSubchannel | 10 |  |
| sizeSubchannel | 10 |  |
| Number of allocated subchannel | 1 | Corresponding to 8 PRBs |
| I_TBS | 16 |  |

For the type 2 UE, there are several options for the test:

- Option1: Only focus to verify X (PSCCH decoding processing capability)

- Option2: Verify Both X and Y

- For Type 2, baseline is Option1. Companies can bring additional test purposes.

In order to verify both X and Y, the following setups may partially serve the purpose.

Firstly, the system setup is configured as Table 3. Secondly, three transmission patterns are defined as shown in Figure 1. In the figure, the yellow block stands for the allocated PRBs for PSSCH and the blue block stands for the allocated PRBs for PSCCH. In the first pattern, 96 PRBs are allocated for the PSSCH transmission and the SA in the first subchannel is used for PSCCH. In the second pattern, 20 PSCCH and 20 PSSCH are transmitted, and each PSSCH is with 3 PRBs. In the third pattern, 90 PRBs are allocated for PSSCH transmission and the SA in the last subchannel is used for PSCCH transmission. Thirdly, the three pattern can be transmitted sequentially. With the pattern 2, we can verify “UE is not expected to attempt to decode more than 20 PSCCHs in a subframe”. With pattern 1 and pattern 3, we can partially verify “UE is expected to attempt to decode at least 136 RBs per subframe counting both PSCCH and PSSCH decoding RBs”. For the UE decoding PSCCH in ascending subchannel number, for pattern 1, it needs to decodes 2 PRBs (for PSCCH) + 96 PRBs (for PSSCH) =98 PRBs, while for pattern 3, it needs to decode 20*2 PRBs (for PSCCH) + 90 PRBs (for PSSCH) = 130 (PRBs). For the UE decoding PSCCH in decending subchannel order, for pattern 1, it needs to decode 20*2 PRBs (for PSCCH) + 96 (PRBs) = 136 PRBs, while for pattern 3, it only needs to decode 2 PRBs (for PSCCH) + 90 PRBs (for PSSCH) = 92 PRBs. With this three random interleaved pattern, at least we can verify UE can Handle PRBs larger than 100 PRBs case without any PSSCH overlapping configuration.

Table 3: Test parameters for X=20 and Y=136

| Parameters | Value | Note |
| --- | --- | --- |
| Bandwidth | 20 MHz |  |
| numSubchannel | 20 |  |
| sizeSubchannel | 5 |  |
| Number of allocated subchannels | Depends on the selected pattern | Three patterns are defined in the test |
| I_TBS | 16 |  |

Figure 1: Transmission patterns for the PSCCH processing capability test

Based on this proposal, we can verify X and partial verify Y. Thus:

With proper setup, X can be fully verified and Y can be partially verified

# eNB synchronization source based test

In non-standalone V2V/V2X operation, the main difference with standalone V2V/V2X is the synchronization source is changed. For standalone V2V/V2X operation, the GNSS is the synchronization source. The demodulation test case can at least verify UE whether can properly handle the time/frequency error against GNSS, and receiption window adjustment based on GNSS, etc. When the synchronization source is changed from GNSS to eNB, the UE need to track eNB synchronization, adjust reception window based on eNB synchronization, which is further impact on the time/frequency error handling which has great impact on the demodulation performance. The aforementioned behaviour is a new behaviour for V2V/V2X. It is never verified up to now. One possible way is verified by RRM. However, in RRM test, the RSRP and RSSI measurement is too coarse. It is more like a functionality test from demodulation point of view. Without demodulation performance test, we only know UE can support eNB-based synchronization, but we don’t know how good the UE can achieve the demodulation performance when the synchronization source is changed from GNSS to eNB. Thus, we prefer to have demodulation test for the case where the synchronization source is from eNB.

In order to reduce the test case number and don’t repeat all the tests, we can select one test case to cover eNB as synchronization source case. Single link PSSCH test is perferable for the test.

One and only one single link PSSCH test is introduced for the demodulation test when the synchronization source is eNB

In order to reduce the simulation efforts, we can reuse the same time offset and frequency offset for GNSS case in the new test case.

The time offset and frequency offset used in the standalone can be reused in the scenario wherein eNB is set as the synchronization source and the performance requirement defined for GNSS-based synchronization is reused in the test for eNB-based synchronization.

# PC5 and WAN concurrency test

## Background

### Proximity communication concurrent with WAN

In 36.101, the concurrency test for PC5-based communication and WAN is introduced. Theare are two purposes:

- To verify the downlink data rate is not impacted when Sidelink resource are also configured (Purpose I)

- To verify the transmission of ACK/NACK is prioritized over the reception of PSSCH concurrency (Purpose II)

The test scenario is as shown in Figure 2. In the test setup, there are two transmission UEs, UE1 transmits PSCCH+PSSCH, and UE2 transmits PSCCH only. The purpose of UE1 transmission is to verify the PC5 receiption have no any impact on donwlink data receiption. The purpsose of UE2 transmission is to verify the transmission of ACK/NACK is prioritized over the reception of PSSCH. In the test, PC5 and uplink data is multiplexed in time domain. The detail transmission pattern is shown in Figure 3.

Figure 2: Test scenario in Rel-12 D2D

Figure 3: PC5 and WAN concurrency test in D2D communication

In the test, the scheduling pattern for downlink is {01110111 11110111 11110111 11110111 11111110} as illustrated in the first row of Figure 3, and the corresponding Ack/Nack is shown in the second row, and the transmission of UE1 and UE2 is shown in the third row and fourth row. In the column marked as Yellow, UE need to receive both PDSCH and PC5 from UE1, in the column marked as red, UE needs to prioritize  Ack/Nack transmission over the reception from UE2 .

In the test, only PDSCH performance is verified and PC5 performance is omitted.

### Operation scenario for V2V and WAN concurrency

In 36.101, RF has defined the bands for the con-current V2X operation. The details are shown in Table 4 and Table 5. The scenario can be shown in Figure 4.

Table 4: Inter-band con-current V2X operating bands

| V2X con-current configuration | Operating Band | Interface | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_3-47 | 3 | Uu | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | TDD |
| V2X_7-47 | 7 | Uu | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | TDD |
| V2X_8-47 | 8 | Uu | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | TDD |
| V2X_39-47 | 39 | Uu | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | TDD |
| V2X_41-47 | 41 | Uu | 2496 MHz | – | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | TDD |

Table 5: V2X inter-band con-current configurations and bandwidth combination sets

| V2X inter-band Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_3A-47A | 3 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2X_7A_47A | 7 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2X_8A-47A | 8 | Yes | Yes | Yes | Yes |  |  | 30 | 0 |
|  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2X_39A-47A | 39 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2X_41A-47A | 41 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |

Figure 4: con-current PC5 and WAN operation

According to the RF band definition, dedicated component carriers are allocated for PC5 operation and Uu uplink uses different frequency with PC5. As the receiver, UE needs to receive PC5 and PDSCH simultaneously, which is similar as CA operation, where multiple component carriers are received simultaneously. The only difference is in CA case, all component carriers carry PDSCH, while in this scenario, some component carriers carry PDSCH and some component carriers carry PC5.

## Discussion on the V2V and WAN concurrency test

When V2V and WAN is concurrent, they may impact each other without proper implementation. RAN1 have realize this issue very earlier. In order to avoid the improper V2V implementation which lead to some impact on WAN performance, In RAN1 #83, RAN1 have the following agreements:

- The followings are supported for the purpose of coexistence between PC5-based V2V and WAN:

  - The same receiver capability of D2D communication UEs is assumed for V2V UEs. That is, a Rx chain is available at all time to receive V2V signals without affecting WAN reception (from RAN1 perspective) when the UE is configured to receive V2V.

From RAN1 point of view, the conclusion is very clear. How to gurantee this point is one main task for RAN4. RAN4 shall design a proper test case to achieve good coexistence between PC5-based V2V and WAN.

In 36.101, the proximity communication and WAN concurrency test have two purposes:

- To verify the downlink data rate is not impacted when Sidelink resource are also configured (Purpose I)

- To verify the transmission of ACK/NACK is prioritized over the reception of PSSCH concurrency (Purpose II)

For the second purpose, as stated in section 4.1.2, the PC5 uses different component carriers from Uu uplink up to this release. Thus, there is no confliction between Uu uplink transmission and PC5 receiption up to this release. Hence, it is not necessary to set the purpose II for the V2V and WAN concurrency test in this release.

During the discussion of concurrent D2D and WAN test, another test purpose is also discussed but not agreed. This test purpose is to verify WAN TX is prioritized over D2D TX. In V2V and WAN concurrency test, The priotization rule between V2V TX and WAN TX may be also not necessary. The prioritization rule between V2V and WAN is different from D2D communication TX and WAN TX prioritization rule. In 36.331, for the side-link TX prioritization over WAN TX, we have the following signalling and behaviour:

SL-V2X-ConfigCommon-r14 ::=		SEQUENCE {

v2x-CommRxPool-r14					SL-CommRxPoolListV2X-r14			OPTIONAL,	-- Need OR

v2x-CommTxPoolNormalCommon-r14		SL-CommTxPoolListV2X-r14			OPTIONAL,	-- Need OR

p2x-CommTxPoolNormalCommon-r14		SL-CommTxPoolListV2X-r14			OPTIONAL,	-- Need OR

v2x-CommTxPoolExceptional-r14		SL-CommResourcePoolV2X-r14			OPTIONAL,	-- Need OR

v2x-SyncConfig-r14					SL-SyncConfigListV2X-r14			OPTIONAL,	-- Need OR

v2x-InterFreqInfoList-r14			SL-InterFreqInfoListV2X-r14			OPTIONAL,	-- Need OR

v2x-ResourceSelectionConfig-r14			SL-CommTxPoolSensingConfig-r14		OPTIONAL,	-- Need OR

zoneConfig-r14						SL-ZoneConfig-r14					OPTIONAL,	-- Need OR

typeTxSync-r14						SL-TypeTxSync-r14,

thresSL-TxPrioritization-r14		INTEGER (0..7)						OPTIONAL,	-- Need OR

offsetDFN-r14						INTEGER (0..1000)					OPTIONAL	-- Need OR

}

thresSL-TxPrioritization

Indicates the threshold used to determine whether SL TX is prioritized over UL TX overlapping in time. If the SL data to be transmitted has a PPPP value lower than this threshold, then the SL data can be prioritized over UL transmission in the same subframe. This value shall overwrite thresSL-TxPrioritization configured in SL-V2X-Preconfiguration if any.

Based on this definition, SL TX may be priotized over UL Tx overlapping in time. Further, since the PC5 and Uu uplink use different component carriers up to this realease, thus, SL TX and UL TX have no confliction up to this release.

In summary, in V2V concurrency test, the prioritization rule between UL Tx and SL TX/RX may not need to be verified in this release.

One V2V and WAN concurrency test is introduced to verify V2V receiption has no impact on WAN receiption

Further, in D2D proximity communication concurrency test with WAN, only PDSCH performance is verified, and the D2D proximity communication performance is omitted. The main reason lies in the RAN1 agreements:

No standardized mechanism is defined for D2D communication and discovery to share the soft buffer already defined for cellular communications

Based on these agreements, the proximity communication performance may be based on best efforts. Thus, only WAN performance can be guarantee. However, in V2V, the separate soft buffer is defined. In RAN1#88 meeting, RAN1 define the SL soft buffer size. The total SL soft buffer size is 737280. Thus, when we define concurrency test, both WAN and SL V2V performance shall be verified simultaneously.

The performance of WAN and the performance of PC5-based V2V shall be verified simultaneously.

## Setup PC5 and WAN concurrency test

Based on the discussion in section 4.1.2 and 4.2, we can have the setup as shown in Figure 5. In the test, one downlink component carrier, one uplink component carrier and one PC5 carrier is configured. In downlink component carrier, the MCS to achieve maximum throughout is configured. In PC5 carrier, the channel is also fully occupied. To berief, we can combine legacy SDR test with V2V single link or maximum processing capability test into one test. The setup is much simpler than D2D communication concurrency test.

Figure 5: Test setup for V2V and WAN concurrency test

Combine WAN SDR test and V2V standalone SDR test into concurrency test

# Conclusion

In this contribution, we provide our analysis about the V2X demodulate test. We have the following observations:

Observation 1	Proper SNR need to be set to reduce the decode probability for the first transmission to force UE to buffer the first 15 TBs in the soft buffer management test.

Observation 2	With proper setup, X can be fully verified and Y can be partially verified

And we have the following proposals:

Proposal 1	One and only one single link PSSCH test is introduced for the demodulation test when the synchronization source is eNB

Proposal 2	The time offset and frequency offset used in the standalone can be reused in the scenario wherein eNB is set as the synchronization source and the performance requirement defined for GNSS-based synchronization is reused in the test for eNB-based synchronization.

Proposal 3	One V2V and WAN concurrency test is introduced to verify V2V receiption has no impact on WAN receiption

Proposal 4	The performance of WAN and the performance of PC5-based V2V shall be verified simultaneously.

Proposal 5	Combine WAN SDR test and V2V standalone SDR test into concurrency test

# References

R4-1704272, WF on Demodulation Performance for V2X UE, LG Electronics, Huawei, HiSilicon, CATT, RAN4#82bis, Apr. 2017.

R1-1703447, Decoding capability of V2V UE, Qualcomm Incorporated, RAN1#88, Feb. 2017.