3GPP TSG-RAN4 Meeting #83	R4-1705669

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.977 | CR | 0059 | rev | - | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Theoretical V/H Limits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Spirent Communications, Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MIMO_OTA |  |  |  |  |  | Date: |  |  | 2017-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correct theoretical V/H limit for UMi as current limit could not be derived by Spirent and R&S |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct limit from 0.83 to 0.74 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Almost none as difference is small |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.4, 8.4.5, 12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Beginning of proposed text>

#### 8.3.2.4	Cross-polarization

This measurement checks how well the measured vertically or horizontally polarized power levels follow expected values.

Method of measurement: 		Step the emulation and store traces from VNA.

# VNA settings:

# Table 8.3.2.4-1: VNA settings for cross-polarization

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink Center Frequency in 3GPP TS 36.508 [19] as required per band |
| Span | MHz | 10 |
| RF output level | dBm | -15 |
| Number of traces |  | 1000 |
| Distance between traces in channel model | Wavelength (Note) | > 2 |
| Number of points |  | 201 |
| Averaging |  | 1 |
| NOTE: Time [s] = distance [] / MS speed [/s] MS speed [/s] = MS speed [m /s] / Speed of light [m/s] * Center frequency [Hz] |  |  |

# Channel model specification:

# Table 8.3.2.4-2: Channel model specification for cross-polarization.

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink center frequency in 3GPP TS 36.508 [19] as required per band |
| Channel model samples | wavelength | > 2000 |
| Channel model |  | As specified in Clause 8.2 |
| Mobile speed | km/h | 30 |

# Measurement Procedure

1.	Play or step through the channel model -> SCME UMi, or UMa X Corr.

2.	Measure the absolute power received at the center of the array, averaged over a statistically significant number of fades.

a.	Use a vertically polarized sleeve dipole to measure the V component.

b.	Use a horizontally polarized (vertically oriented) magnetic loop dipole, or a horizontally polarized sleeve dipole measured in two orthogonal horizontal positions and summed to measure the H component.

3.	Calculate the V/H ratio.

4.	Compare it with the theory -> 0.74dB for UMi, and 8.13dB for UMa.

# Expected measurement results

V/H ratio (composite, i.e. all 6 paths combined) of the 3GPP SCME Umicro model is 0.74 dB and for Umacro 8.13 dB. 
The BS antennas are isotropic dipoles with +/- 45 degrees slant and subject to a foreshortening of the slanted radiating element. See channel model details specified in clause 8.2

<End of proposed text>

<Beginning of proposed text>

### 8.4.5	Cross polarization

The cross polarization properties of the channel models defined in clause 8.2 have been characterized according to clause 8.3.2.3.  The measured results shown in Table 8.4.5-1 below are reported considering the antenna gain difference of the reference antennas.

# Table 8.4.5-1: Summary of cross polarization verification results for Band 13

|  | Channel emulator A |  | Channel emulator B |  |
| --- | --- | --- | --- | --- |
|  | SCMe UMi | SCMe UMa | SCMe UMi | SCMe UMa |
| Target | 0.74 dB | 8.13 dB | To be added (Note) | To be added (Note) |
| Measurement considering antenna gain difference | 2.0 dB | 9.0 dB | To be added (Note) | To be added (Note) |
| Deviation | 1.3dB | 0.9dB | To be added (Note) | To be added (Note) |
| NOTE: XPR values for channel emulator B will be added at a later stage. |  |  |  |  |

The cross-polarization results for the RTSmethod using geometric implementation of SCME are shown in Table 8.4.5.2:

Table 8.4.5-2: Summary of cross polarization verification results for Band 13 with geometric implementation

| Channel model |  |  | V/H ratio | Theory | Deviation |
| --- | --- | --- | --- | --- | --- |
| UMa MC |  |  | 8.1 | 8.13 | 0.03 |
| UMi MC |  |  | 0.8 | 0.74 | 0.06 |

The cross-polarization results for the RTS method using correlation implementation of SCME (Jake's Doppler spectrum) are shown in Table 8.4.5.3:

Table 8.4.5-3: Summary of cross polarization verification results for Band 13 with correlation implementation (Jake’s Doppler spectrum)

| Channel model | V/H ratio | Theory | Deviation |
| --- | --- | --- | --- |
| UMa | 7.9 | 8.13 | -0.23 |
| UMi | 0.8 | 0.74 | 0.06 |

<End of proposed text>

<Beginning of proposed text>

## 12.4	Comparison of methodologies

The methodologies which tests plans are described Clause 12, can be broadly classified into 3 categories:

1)	Reverberation Chamber (RC);

2)	Anechoic Chamber (AC);

3)	Multi-stage Method.

The content of the Table 12.4-1 is based on what has been validated as well as currently available state-of-the-art information, and may be reconsidered when the state of the art technology progresses.

# Table 12.4-1: Simplified methodology comparison

| Attribute | Reverberation Chamber |  | Anechoic Chamber | Multi-stage methods |
| --- | --- | --- | --- | --- |
|  | RC | RC + CE | Multi probe | 2 stage method rad. |
| Channel Modelling aspects |  |  |  |  |
| 2D/3D dimension over which the signals simultaneously arrive at the DUT location | 3D1 | 3D1 | 2D | 2D11 |
| Directional distribution of angles of arrival | Random | Random | Selected as defined by SCME channel model in Clause 8 | Selected as defined by SCME channel model in Clause 8 |
| Channel model with controllable spatial characteristics | no | no | Yes2 | Yes2 |
| Angular spread | Statistically isotropic | Statistically isotropic | Selected as defined by SCME channel model in Clause 8 | Selected as defined by SCME channel model in Clause 8 |
| Ability to control angular spread | no | no | Yes2 | Yes2 |
| Power delay profile | Exponential decay | Selected as defined by channel model in Annex C | Selected as defined by SCME channel model in Clause 8 | Selected as defined by SCME channel model in Clause 8 |
| Ability to control power delay profile | Partly controllable2,3 | Yes2 | Yes2 | Yes2 |
| UE speed | Approximately 1Km/h | 30Km/h | 30Km/h | 30Km/h |
| Ability to control UE speed | No | Yes2 | Yes2 | Yes2 |
| UE direction of travel | N/A | N/A | 120º as specified in Clause 8 | 120º as specified in Clause 8 |
| Ability to control direction of travel | N/A | N/A | Yes2 | Yes2 |
| Supported channel models | NIST | Short Delay Spread Long Deay Spread | SCME Uma SCME Umi | SCME Uma SCME Umi |
| BS antenna configuration | Uncorrelated | Selected as defined in Clause 8.5 | Selected as defined in Clause 8.5 | Selected as defined Clause 8.5 |
| Ability to control BS antenna configuration | No | Yes2 | Yes2 | Yes2 |
| XPR (defined in Clause 8.2) | N/A | N/A | 9dB | 9dB |
| V/H ratio | 0dB on average | 0dB on average | 0.74 dB for SCME UMi 8.13 dB for SCME UMa | 0.74 dB for SCME UMi 8.13 dB for SCME UMa |
| Ability to control XPR and V/H | No | No | Yes2 | Yes2 |
| MIMO OTA attributes not yet tested |  |  |  |  |
| Ability to control noise and interference direction | Limited4 | Limited4 | Yes2 | Yes2 |
| DUT size constraints | Depends on chamber size5 and stirrer size | Depends on chamber size5 and stirrer size | Depends on chamber size5, and number of active antenna probes and channel emulator ports to fit required active antenna probes | Depends on chamber size5 (SISO chamber quiet zone) |
| Other Considerations |  |  |  |  |
| Non-intrusive test mode for DUT antenna pattern measurement | Not required | Not required | Not required | Required |
| Ability to distinguish performance based on device orientation relative to the field | No | No | Yes | Yes |
| Major equipment elements for MIMO OTA test setup (all need MIMO BS emulator) | MIMO capable reverberation chamber | MIMO capable reverberation chamber and channel emulator | MIMO capable anechoic chamber to fit antenna probes and channel emulator | SISO anechoic chamber with additional antenna and channel emulator |
| Number of channel emulator ports7 | N/A | 4 | 166 | 2 |
| DUT antenna polarization discrimination8 | No | No | Yes | Yes |
| DUT Antenna radiation pattern adaptation, performance discrimination | Feasibility study not yet performed | Feasibility study not yet performed | Yes9 | Feasibility study not yet performed10 |
| Number of independent measurements | 1 after sufficient number of stirrers states to ensure isotropy12 | 1 after sufficient number of stirrers states to ensure isotropy12 | 12 device rotations for 2D evaluation | Measurement of radiation pattern in 1st stage and measurement in radiated stage for 12 rotations for 2D evaluation |
| Note 1: Random distribution of angles of arrival. Isotropy is achieved after sufficient amount of test time as per Annex C. Note 2: Requires validation. Note 3: PDP modification will require new loading of chamber. Note 4: Feasibility study under progress. Note 5: Chamber size depends on the size of the UE and the frequency of the test. Note 6: Minimum setup configuration as per table 6.3.1.1-1. Note 7: Configuration reflects what has been tested. Optimization may be possible. Note 8: Assuming that correlation, gain imbalance, total efficiency are equivalent among DUT, purely isolating antennas polarization. Note 9: Based on preliminary feasibility study. Note 10: It will require DUT feedback mechanism. Note 11: 3D is possible without new test setup if 3D channel models are specified. It requires validation. Note 12: Isotropy is achieved after sufficient amount of isotropic states as per Annex C. The guideline for TRS, number of independent samples should be larger than 100, preferable 200 or 400 (3GPP TS 34.114 [4]). |  |  |  |  |

<End of proposed text>