3GPP TSG-RAN WG4 Meeting #82	R4-1705816

Hangzhou, Zhejiang, China, 15th – 19th May, 2017

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	TRP uncertainty contributors

Agenda item:	9.12.3.2

Document for:	Discussion

# 1	Introduction

This document discusses three main contributors of uncertainty in TRP measurement. These are TRP accuracy, approximation and measurement uncertainty. For each of these contributors, we identify and quantify the major causes of the TRP uncertainty.

In conclusion, we provide the relationship between these uncertainty contributors, which is derived based on our discussion.

# 2	Discussion

TRXU 1

TRXU 2

.

.

.

AAS Base Station under TX test

TRXU NTRXU

TRXU Array

Measured TRP values in far field

Antenna Array

Radio Distribution Network

.

.

.

Figure 1: Total radiated output power of AAS BS

## 2.1 TRP accuracy

The TRP inaccuracy contributor is attributed to the transceiver units and antenna array in an AAS BS as shown in Figure 1.

The same mathematical approach as for the EIRP accuracy is used to model the TRP accuracy. Referring to [1], the EIRP accuracy model is modified and the TRP accuracy is mathematically expressed as

TRPaccuracy = ±σtx2+ σarray2   		[dB]																			(1)

where

- - the maximum conducted output power error at the transceiver unit output

- - the variation due to the error in the passive elements, the RDN, the antenna array gain errors, mismatch errors and insertion losses variation

Further details on the derivation of a proposed TRP accuracy can be found in our companion document [2].

## 2.2 TRP approximation

Neither the AAS BS nor the measurement system setup contributes to the TRP approximation uncertainty. The source of TRP approximation uncertainty is outlined below.

The TRP is obtained by integrating the radiation intensity U over 4π steradians. Thus, the TRP is defined as

TRPactual= ΩUdΩ =  0π02πUsinθdθdϕ 																		(2)

where dΩ	is the element of solid angle = sinθdθdϕ.

Equation (2) can be expressed in terms of EIRP as

TRPactual= 14π0π02π(EIRPco-polθ, ϕ+ EIRPx-pol(θ, ϕ))sinθdθdϕ										(3)

Using numerical integration, we can compute an approximation to equation (3) from a finite set of sampled values of EIRP as

TRPapprox ≈ π2NMn=0N-1m=0M-1(EIRPco-polθn, φm+ EIRPx-pol(θn, φm))sin θn								(4)

where  N and M = the number of discrete angles in the θ- and ϕ-direction, respectively.

TRPapprox= TRPactual as n and m approach ∞, i.e.,

limn,m→(∞,∞)TRPapprox= TRPactual																					(5)

For every positive integer number N and M, the absolute error ϵ due to approximating TRPactual is defined as

TRPactual- TRPapprox=  ϵ>0,   ∀ n<N, m<M																(6)

If TRPactual>0, the relative error η is

η= ϵTRPactual=1- TRPapproxTRPactual 																						(7)

The absolute or relative error is attributed to

- the number of discrete angles N and M in the θ- and ϕ-directions, respectively.

- the amount of reduction in the range of θ and ϕ, where the maximum ranges are 0° ≤ θ ≤180° and 0° ≤ ϕ<360°.

Observation 1: As a result of approximating the actual TRP, there is an inherent error (> 0) for a finite number of N and M, unless (N, M) -> (∞, ∞).

What is the acceptable error be it absolute or relative?

Without knowing such an error, we are unable to assess the effectiveness of reduction methods.

## 2.3 TRP measurement uncertainty

The TRP measurement uncertainty is attributed to measurement system setup.

For this uncertainty, the methodology specified for the EIRP measurement uncertainty in Clause 10.3, TR 37.842 can be reused. The EIRP Test Tolerance values shown in Table 1, which specified in TR 37.842, can be adopted as a baseline considering additional uncertainty contributors which depend on the OTA ranges.

Table 1: EIRP Test Tolerance values (from TR 37.842, Table 10.3.2.3-1)

|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- |
| Test Tolerance [dB] | 1.0 | 1.2 |

# 3	Conclusions

In this document, we have discussed the TRP accuracy, approximation and measurement uncertainty; all these are contributors to the uncertainty in TRP measurement.

Mathematical models were developed for representing TRP accuracy and approximation uncertainty. For TRP measurement uncertainty, the the methodology specified for the EIRP measurement uncertainty in Clause 10.3, TR 37.842 can be reused.

Based on our discussion, the three major TRP uncertainty contributors can be expressed as a sum of three independent quantities in the logarithmic scale as

Y = ([TRP accuracy] + [TRP approximation] + [TestTolerance]) [dB]

Thus, the measured TRP should be within the interval [-Y, +Y] of a declared rated TRP by manufacturers.

# References

- R4-1704340, Way forward on TRP accuracy requirements, NTT DOCOMO

- R4-1705814, eAAS BS output power accuracy window requirements, Nokia, Alcatel-Lucent Shanghai Bell