3GPP TSG-RAN WG4 Meeting #83	R4-1705941

# Hangzhou, China, 15th-19th May, 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | 0776 | rev | 2 | Current version: | 13.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove NB-IoT inband support for 1.4 MHz - Tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, Nokia, Alcatel-Lucent Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  |  | Date: |  | 2017-05-17 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | LTE control signals are concentrated in the central 6 PRBs. RAN1 decided so to not use these middle 6 PRBs for NB-IoT when operating inband, which so excludes 1.4 MHz LTE to support NB-IoT inband. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add reference to NB-IoT channel bandwdth definition in TS 36.104 Remove reference to 1.4MHz LTE when specifying NB-IoT inband requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | One could consider NB-IoT could be operated inband in LTE 1.4 MHz signal, while it’s not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4, 7.4.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 37.104 CR 0780 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR… CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of 1st change>

## 4.4	Operating bands and band categories

MSR requirements are applicable for band definitions and band numbering as defined in the specifications TS 45.005 [6], TS25.104 [3], TS 25.105 [4] and TS 36.104 [5]. For the purpose of defining the BS requirements, the operating bands are divided into three band categories as follows:

-	Band Category 1 (BC1): Bands for E-UTRA FDD and UTRA FDD operation

-	Band Category 2 (BC2): Bands for E-UTRA FDD, UTRA FDD and GSM/EDGE operation

-	Band Category 3 (BC3): Bands for E-UTRA TDD and UTRA TDD operation

NOTE:	For UTRA TDD, requirements in the present document cover the 1.28 Mcps UTRA TDD option.

The paired and unpaired bands for the three Band Categories are shown in Table 4.4-1 and 4.4-2, together with the corresponding E-UTRA, UTRA and GSM/EDGE band designations. In the present specification, the operating band of an MSR Base Stations is designated using the E-UTRA band number according to the tables.

Table 4.4-1: Paired bands in E-UTRA, UTRA and GSM/EDGE

| - MSR and E-UTRA Band number | - UTRA Band number | - GSM/EDGE - Band designation | - Uplink (UL) BS receive UE transmit |  |  |  |  | - Downlink (DL) BS transmit UE receive |  |  |  |  |  | - Band category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | I | - | 1920 MHz | – |  | 1980 MHz |  | 2110 MHz |  | – |  | 2170 MHz |  | 1 |
| 2 | II | PCS 1900 | 1850 MHz | – |  | 1910 MHz |  | 1930 MHz |  | – |  | 1990 MHz |  | 2 |
| 3 | III | DCS 1800 | 1710 MHz | – |  | 1785 MHz |  | 1805 MHz |  | – |  | 1880 MHz |  | 2 |
| 4 | IV | - | 1710 MHz | – |  | 1755 MHz |  | 2110 MHz |  | – |  | 2155 MHz |  | 1 (NOTE 3) |
| 5 | V | GSM 850 | 824 MHz | – |  | 849 MHz |  | 869 MHz |  | – |  | 894 MHz |  | 2 |
| 6 (NOTE 1) | VI | - | 830 MHz | – |  | 840 MHz |  | 875 MHz |  | – |  | 885 MHz |  | 1 (NOTE 1) |
| 7 | VII | - | 2500 MHz | – |  | 2570 MHz |  | 2620 MHz |  | – |  | 2690 MHz |  | 1 (NOTE 3) |
| 8 | VIII | E-GSM | 880 MHz | – |  | 915 MHz |  | 925 MHz |  | – |  | 960 MHz |  | 2 |
| 9 | IX | - | 1749.9 MHz | – |  | 1784.9 MHz |  | 1844.9 MHz |  | – |  | 1879.9 MHz |  | 1 (NOTE 3) |
| 10 | X | - | 1710 MHz | – |  | 1770 MHz |  | 2110 MHz |  | – |  | 2170 MHz |  | 1 (NOTE 3) |
| 11 | XI | - | 1427.9 MHz | – |  | 1447.9 MHz |  | 1475.9 MHz |  | – |  | 1495.9 MHz |  | 1 (NOTE 3) |
| 12 | XII | - | 699 MHz | – |  | 716 MHz |  | 729 MHz |  | – |  | 746 MHz |  | 1 |
| 13 | XIII | - | 777 MHz | – |  | 787 MHz |  | 746 MHz |  | – |  | 756 MHz |  | 1 |
| 14 | XIV | - | 788 MHz | – |  | 798 MHz |  | 758 MHz |  | – |  | 768 MHz |  | 1 (NOTE 3) |
| 15 | XV | - | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  |
| 16 | XVI | - | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  |
| 17 | - | - | 704 MHz | – |  | 716 MHz |  | 734 MHz |  | – |  | 746 MHz |  | 1 (NOTE 4) |
| 18 | - | - | 815 MHz | – |  | 830 MHz |  | 860 MHz |  | – |  | 875 MHz |  | 1 (NOTE 4) |
| 19 | XIX | - | 830 MHz | – |  | 845 MHz |  | 875 MHz |  | – |  | 890 MHz |  | 1 |
| 20 | XX | - | 832 MHz | – |  | 862 MHz |  | 791 MHz |  | – |  | 821 MHz |  | 1 |
| 21 | XXI | - | 1447.9 MHz | – |  | 1462.9 MHz |  | 1495.9 MHz |  | – |  | 1510.9 MHz |  | 1 (NOTE 3) |
| 22 | XXII | - | 3410 MHz | – |  | 3490 MHz |  | 3510 MHz |  | – |  | 3590 MHz |  | 1 (NOTE 3) |
| 23 | - | - | 2000 MHz | – |  | 2020 MHz |  | 2180 MHz |  | – |  | 2200 MHz |  | 1 (NOTE 2) |
| 24 | - | - | 1626.5 MHz | – |  | 1660.5 MHz |  | 1525 MHz |  | – |  | 1559 MHz |  | 1 (NOTE 2) |
| 25 | XXV | - | 1850 MHz | – |  | 1915 MHz |  | 1930 MHz |  | – |  | 1995 MHz |  | 1 (NOTE 3) |
| 26 | XXVI | - | 814 MHz | – |  | 849 MHz |  | 859 MHz |  | – |  | 894 MHz |  | 1 |
| 27 | - | - | 807 MHz | – |  | 824 MHz |  | 852 MHz |  | – |  | 869 MHz |  | 1 (NOTE 2) |
| 28 | - | - | 703 MHz | – |  | 748 MHz |  | 758 MHz |  | – |  | 803 MHz |  | 1 (NOTE 4) |
| 29 | - | - | N/A |  |  |  |  | 717 MHz |  | – |  | 728 MHz |  | 1 (NOTE 2, NOTE 5) |
| 30 | - | - | 2305 MHz | – |  | 2315 MHz |  | 2350 MHz |  | – |  | 2360 MHz |  | 1 (NOTE 2) |
| 31 | - | - | 452.5 MHz | – |  | 457.5 MHz |  | 462.5 MHz |  | – |  | 467.5 MHz |  | 1 (NOTE 2) |
| 32 (NOTE 5) | XXXII (NOTE 6) | - |  |  | N/A |  |  |  | 1452 MHz |  | – |  | 1496 MHz | 1 (NOTE 3) |
| 64 | - | - |  |  |  |  |  |  | Reserved |  |  |  |  |  |
| 65 | - | - | 1920 MHz |  | – |  | 2010 MHz |  | 2110 MHz |  | – |  | 2200 MHz | 1 (NOTE 2) |
| 66 (NOTE 7) | - | - | 1710 MHz |  | - |  | 1780 MHz |  | 2110 MHz |  | - |  | 2200 MHz | 1 (NOTE 4) |
| 67 (NOTE 5) | - | - | N/A |  |  |  |  |  | 738 MHz |  | – |  | 758 MHz | 1 (NOTE 2) |
| 68 | - | - | 698 MHz |  | - |  | 728 MHz |  | 753 MHz |  | - |  | 783 MHz | 1 (NOTE 2) |
| NOTE 1: The band is for UTRA only. NOTE 2: The band is for E-UTRA only. NOTE 3: The band is for E-UTRA and UTRA only. NOTE 4: The band is for E-UTRA and NB-IoT only. NOTE 5: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 6: Restricted to UTRA operation when dual band is configured (e.g., DB-DC-HSDPA or dual band 4C-HSDPA). The down link frequency(ies) of this band are paired with the uplink frequenc(ies) of the other FDD band (external) of the dual band configuration. NOTE 7: The range 2180-2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE:	For BS capable of multi-band operation, the supported operating bands may belong to different Band Categories.

Table 4.4-2: Unpaired bands in E-UTRA and UTRA

| MSR and E-UTRA Band number | UTRA Band number | Uplink (UL) BS receive UE transmit |  |  | Downlink (DL) BS transmit UE receive |  |  | Band category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33 | a) | 1900 MHz | – | 1920 MHz | 1900 MHz | – | 1920 MHz | 3 |
| 34 | a) | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | 3 |
| 35 | b) | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | 3 |
| 36 | b) | 1930 MHz | – | 1990 MHz | 1930 MHz | – | 1990 MHz | 3 |
| 37 | c) | 1910 MHz | – | 1930 MHz | 1910 MHz | – | 1930 MHz | 3 |
| 38 | d) | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | 3 |
| 39 | f) | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | 3 |
| 40 | e) | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | 3 |
| 41 | - | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | 3 |
| 42 | - | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | 3 |
| 43 | - | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | 3 |
| 44 | - | 703 MHz | – | 803 MHz | 703 MHz | – | 803 MHz | 3 |
| 45 | - | 1447 MHz | – | 1467 MHz | 1447 MHz | – | 1467 MHz | 3 |

Table 4.4-3. Void

Table 4.4-4. Void

E-UTRA is designed to operate for the carrier aggregation bands defined in Tables 5.5-2, 5.5-3 and 5.5-4 of TS 36.104 [5]. The E-UTRA channel bandwidth BWChannel for a single carrier and the Aggregated Channel Bandwidth BWChannel_CA for E-UTRA carrier aggregation are specified in Section 5.6 of TS 36.104 [5].

The NB-IoT channel bandwidth BWChannel is specified in Section 5.6 of TS 36.104 [5].

<End of 1st change>

<Start of 2nd change>

#### 7.4.5.2	General narrowband blocking test requirement

For the narrowband blocking requirement, the interfering signal shall be an E-UTRA 1RB signal as specified in Annex A.3.

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For BS operating in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 3MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For BS capable of multi-band operation, the requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least 3MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal coupled to the Base Station antenna input, using the parameters in Table 7.4.5.2-1 the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], subclause 7.2.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.104 [3], subclause 7.2.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.105 [4], subclause 7.2.

-	For any measured GSM/EDGE carrier, the conditions are specified in TS 45.005 [6], Annex P.2.1.

-	For any measured NB-IoT carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], subclause 7.2.

# Table 7.4.5.2-1: Narrowband blocking requirement

| Base Station Type | RAT of the carrier | Wanted signal mean power [dBm] (Note 1) | Interfering signal mean power [dBm] | Interfering RB (Note 3) centre frequency offset from the Base Station RF Bandwidth edge or sub-block edge inside a gap [kHz] |
| --- | --- | --- | --- | --- |
| Wide Area BS | E-UTRA, NB-IoT(Note 4), UTRA and GSM/EDGE | PREFSENS + x dB (Note 2) | -49 | ±(240 +m*180), m=0, 1, 2, 3, 4, 9, 14 |
| Medium Range BS |  |  | -44 |  |
| Local Area BS |  |  | -41 |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see subclause 7.2 in TS 37.104. NOTE 2: “x” is equal to 6 in case of E-UTRA or UTRA wanted signals and equal to 3 in case of GSM/EDGE wanted signal. “x” is specified in Table 7.4.2-2 for NB-IoT. NOTE 3: Interfering signal (E-UTRA 3MHz) consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF Bandwidth edge. NOTE 4: The requirement is valid for NB-IoT Wide Area BS. |  |  |  |  |

# Table 7.4.5.2-2: “x” for NB-IoT wanted signals

| Operation mode | LTE channel bandwidth for in-band/guard band operation | x |
| --- | --- | --- |
| Standalone | - | 12 |
| In Band |  |  |
|  | 3 MHz | 11 |
|  | 5 MHz | 9 |
|  | 10 MHz | 6 |
|  | 15 MHz | 6 |
|  | 20 MHz | 6 |
| Gaurd band | 5 MHz | 13 |
|  | 10 MHz | 6 |
|  | 15 MHz | 6 |
|  | 20 MHz | 6 |

<End of 2nd change>