3GPP TSG-RAN WG4 Meeting #83	 R4-1706059

Hangzhou, China, 15-19 May, 2017

Agenda Item:	10.3.3

Source: 	Samsung

Title: 	Discussion on spectrum utilization

Document for:	Discussion

Introduction

In companion paper on how to define transmission bandwidth configuration we show the template on how to define NRB for in specification. In this contribution we share our evaluation result on achievable transmission spectrum utilization for both sub-6GHz and mm-Wave range based on latest agreement for Channel bandwidth and SCS in [1] [2] [3].

Discussion

Evaluation assumptions

The primary simulation assumptions are summarized here. First of all, the PA model is based on the one shared in reply LS to RAN1 shown in [4] with adjustment on operation point according to certain output power level. For the emission level out of wanted channel bandwidth and ACLR for emission confinement, the IMT parameters for WP5D response [5] are utilized for mm-Wave range. And for below 6GHz range, the corresponding LTE requirements are used for simulation.

Based on the evaluation and the feasibility consideration of transmitter implementation, initial observations for achievable spectrum utilization under different combinations of SCS and channel bandwidths in sub 6GHz and above 6GHz were in tables below.

In this initial evaluation, only transmission requirements i.e. ACLR, SEM and EVM were considered. For final conclusions, further considerations on receiver impact were needed i.e. receiver complexity and performance as well (ADC dynamic range, ACS as per the analog/digital filters, etc.)

Evaluation results for mm-Wave range

Table 1: spectrum utilization for SCS=60 KHz

| CBW (MHz) | 50 | 100 | 150 | 200 | 400 |
| --- | --- | --- | --- | --- | --- |
| FFT size | 1024 | 2048 | 4096 | 4096 | 8192 |
| PRBs | 66 | 132 | 198 | 264 | 528 |
| Spectrum utilization | 95% | 95% | 95% | 95% | 95% |

Table 2: spectrum utilization for SCS=120 KHz

| CBW (MHz) | 50 | 100 | 150 | 200 | 400 |
| --- | --- | --- | --- | --- | --- |
| FFT size | 512 | 1024 | 2048 | 2048 | 4096 |
| # PRBs | 32 | 66 | 98 | 132 | 264 |
| Spectrum utilization | 92% | 95% | 94% | 95% | 95% |

Evaluation results for sub-6GHz range

Table 3: spectrum utilization for SCS=15 KHz

| CBW (MHz) | 5 | 10 | 20 | 40 | 50 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FFT size | 512 | 1024 | 2048 | 4096 | 4096 | 8192 | 8192 |
| # PRBs | 26 | 54 | 108 | 216 | 264 | 432 | 528 |
| Spectrum utilization | 93.6% | 97.2% | 97.2% | 97.2% | 95% | 97.2% | 95% |

Table 4: spectrum utilization for SCS=30 KHz

| CBW (MHz) | 5 | 10 | 20 | 40 | 50 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FFT size | 256 | 512 | 1024 | 2048 | 2048 | 4096 | 4096 |
| # PRBs | 13 | 26 | 54 | 108 | 132 | 216 | 264 |
| Spectrum utilization | 93.6% | 93.6% | 97.2% | 97.2% | 95% | 97.2% | 95% |

Table 5: spectrum utilization for SCS=60 KHz

| CBW (MHz) | 10 | 20 | 40 | 50 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- |
| FFT size | 256 | 512 | 1024 | 1024 | 2048 | 2048 |
| # PRBs | 13 | 26 | 54 | 66 | 108 | 132 |
| Spectrum utilization | 93.6% | 93.6% | 97.2% | 95% | 97.2% | 95% |

Conclusion

In this contribution we share the initial evaluation result for spectrum utilization based on latest RAN1/4 agreements. And it is suggested to be taken into account in corresponding requirement definition.

Reference

[1] R4-1703100, LS Reply on NR minimum carrier bandwidth

[2] R4-1704401, WF on Subcarrier Spacing for NR

[3] R4-1704223, LS on SCS for NR

[4] R4-164542, Response LS on realistic power amplifier model for NR waveform evaluation

[5] TR38.803

- Annex

- Evaluation results for UL

PA model with ACLR 30dB

50MHz with 60 kHz SCS (47.52MHz with 95% SU)

200MHz with 60 kHz SCS (190.08 MHz with 95% SU)

- Evaluation results for DL

50MHz with 60 kHz SCS (47.52MHz with 95% SU)

200MHz with 60 kHz SCS (190.08 MHz with 95% SU)