3GPP TSG RAN WG4 Meeting #84	R4-1707020

Berlin, Germany, 21 - 25 August, 2017

3GPP TSG RAN WG1 NR Ad-Hoc#2		R1-1711839

Qingdao, P.R. China 27th – 30th June 2017

Title:	LS on subcarrier alignment and channel raster

Response to:	-

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN1

To:	RAN4

Cc:

Contact Person:

#### Name:	Ralf Bendlin

E-mail Address:	ralf.bendlin@att.com

1. Overall Description:

In order to support configuration of subcarrier alignment between NR UL (15 kHz) and LTE UL in paired spectrum, RAN1 reached the following agreement concerning the NR UL channel raster:

Agreements: - In NR, support configuration between the following for paired spectrum (support of scheme 2 below is conditioned on the assumption that 100kHz is adopted as a supported UL channel raster in NR to support LTE/NR co-existence with LTE FDD) - Scheme 1: Do nothing to allow subcarrier alignment between NR UL (15 kHz) and LTE UL - Scheme 2: allow subcarrier alignment between NR UL (15 kHz) and LTE UL, where NR UL raster is with a 7.5 kHz shift to the LTE UL raster

2. Actions:

To RAN4 WG:

RAN1 kindly requests RAN4 to take the above RAN1 agreement into account when discussing NR channel raster.

3. Dates of next TSG-RAN WG1 Meetings:

RAN1 #90	21st – 25th August 2017	Prague, Czech Republic

RAN1 NR Ad-Hoc#3	18th – 21th September 2017	Nagoya, Japan