3GPP TSG-RAN WG4 Meeting #84	R4-1707146

Berlin, Germany, 21 - 25 August, 2017

Agenda Item:	8.24.2.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Transmitter requirements for microcell NB-IoT BS

Document for:	Approval

# 1.	Introduction

The WI proposal on Further NB-IoT enhancements was agreed in RAN#73 [1]. One of the objectives of the WI is to specify necessary support (appropriate eNB classes) for NB-IoT to be used in microcell, picocell, and femtocell deployments.

This contribution provides proposals on the transmitter requirements for microcell NB-IoT BS in the RAN4 specifications, focusing on the requirements that are currently specified depending on the BS class. For the requirements that are currently specified agnostic to the BS class, it is proposed to adopt the macrocell NB-IoT BS requirements to the microcell NB-IoT BS.

# 2.	Discussion

The proposals to specify the microcell NB-IoT BS transmitter requirements are provided below.

2.1	Base station output power

The base station output power and accuracy requirements are specified for the corresponding deployment scenario of each BS class, and the wide area E-UTRA or MSR BS output power and accuracy requirements are applied to the wide area NB-IoT BS. Therefore, it is proposed to apply the medium range E-UTRA or MSR BS output power and accuracy requirements also to the medium range NB-IoT BS.

2.2	Output power dynamics

These requirements apply during the transmitter ON period, during which the specified transmit signal quality shall be maintained. These requirements are currently specified agnostic to the BS class, hence it is proposed to adopt the wide area NB-IoT BS output power dynamics requirements (including NB-IoT RB power dynamic range for in-band or guard band operation) to the medium range NB-IoT BS.

2.3	Transmit ON/OFF power

These requirements are only applied for TDD BS and are currently specified agnostic to the BS class, hence it is proposed to adopt the wide area TDD BS transmit OFF power and transient period requirements to the medium range NB-IoT BS.

2.4	Transmitted signal quality

The frequency error requirement is the measure of the difference between the actual BS transmit frequency and the assigned frequency. It is specified depending on the expected UE speed and BS implementation feasibility of each BS class, and the wide area E-UTRA BS frequency error requirement is applied to the wide area NB-IoT BS. Therefore, it is proposed to apply the medium range E-UTRA BS frequency error requirement also to the medium range NB-IoT BS.

The other transmitted signal quality requirements are currently specified agnostic to the BS class, hence it is proposed to adopt the wide area NB-IoT BS EVM, TAE and DL RS power requirements to the medium range NB-IoT BS.

2.5	Out-of-band emissions

Out-of-band emissions are unwanted emissions immediately outside the channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions.

The occupied bandwidth and relative ACLR requirements are currently specified agnostic to the BS class, hence it is proposed to adopt the wide area NB-IoT BS occupied bandwidth and ACLR requirements to the medium range NB-IoT BS.

For the absolute ACLR limit, currently the wide area E-UTRA or MSR BS limit is applied to wide area NB-IoT BS. Therefore, it is proposed to apply the absolute ACLR limit for the medium range E-UTRA or MSR BS also to the medium range NB-IoT BS.

For operating band unwanted emissions, currently the wide area E-UTRA or MSR BS requirements are applied to wide area NB-IoT BS for in-band and/or guard band operations. Therefore, it is proposed to apply the medium range E-UTRA or MSR BS operating band unwanted emissions requirements also to the medium range NB-IoT BS for in-band and/or guard band operations.

For NB-IoT standalone operation, the wide area MSR with GSM/EDGE BS operating band unwanted emissions requirements are applied to wide area NB-IoT BS. Therefore, it is proposed to apply the medium range MSR with GSM/EDGE BS operating band unwanted emissions requirements also to the medium range NB-IoT BS for standalone operations.

2.6	Transmitter spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The requirements for protection of own BS receiver and the limits for co-location with other BS are specified depending on the BS class, based on a 30 dB MCL (BS Tx to BS Rx) and 0.8 dB allowed degradation of the BS receiver. Therefore, it is proposed to use the same -91 dBm/100 kHz emission limit as the medium range E-UTRA or MSR BS for the medium range NB-IoT BS for protection of own BS receiver and co-location with other BS.

The mandatory spurious emission limits and requirements for co-existence with systems operating in other frequency bands are currently specified agnostic to the BS class, hence it is proposed to adopt the wide area NB-IoT BS mandatory spurious emission limits and requirements for co-existence with systems operating in other frequency bands to the medium range NB-IoT BS.

2.7	Transmitter intermodulation

This is a measure of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the own transmit signal and an interfering signal reaching the transmitter via the antenna, mainly to verify the BS RF impairment of IMD3 and IMD5. The interfering signal is a 5MHz E-UTRA signal at a mean power level of 30 dB lower than that of the mean power of the wanted signal.

This requirement is currently specified agnostic to the BS class, hence it is proposed to adopt the wide area NB-IoT BS transmitter intermodulation requirement to the medium range NB-IoT BS.

# 3.	Conclusion and proposals

This contribution has provided proposals on the transmitter requirements for microcell NB-IoT BS in the RAN4 specifications.

Proposals:

1)	To apply the medium range E-UTRA or MSR BS output power and accuracy requirements also to the medium range NB-IoT BS.

2)	To adopt the wide area NB-IoT BS output power dynamics requirements (including NB-IoT RB power dynamic range for in-band or guard band operation) to the medium range NB-IoT BS.

3)	To adopt the wide area TDD BS transmit OFF power and transient period requirements to the medium range NB-IoT BS.

4)	To apply the medium range E-UTRA BS frequency error requirement also to the medium range NB-IoT BS.

5)	To adopt the wide area NB-IoT BS EVM, TAE and DL RS power requirements to the medium range NB-IoT BS.

6)	To adopt the wide area NB-IoT BS occupied bandwidth and relative ACLR requirements to the medium range NB-IoT BS.

7)	To apply the absolute ACLR limit for the medium range E-UTRA or MSR BS also to the medium range NB-IoT BS.

8)	To apply the medium range E-UTRA or MSR BS operating band unwanted emissions requirements also to the medium range NB-IoT BS for in-band and/or guard band operations.

9)	To apply the medium range MSR with GSM/EDGE BS operating band unwanted emissions requirements also to the medium range NB-IoT BS for standalone operations.

10)	To use the same -91 dBm/100 kHz emission limit as the medium range E-UTRA or MSR BS for the medium range NB-IoT BS for protection of own BS receiver and co-location with other BS.

11)	To adopt the wide area NB-IoT BS mandatory spurious emission limits and requirements for co-existence with systems operating in other frequency bands to the medium range NB-IoT BS.

12)	To adopt the wide area NB-IoT BS transmitter intermodulation requirement to the medium range NB-IoT BS.

# References

[1]	RP-170852, “New WID on Further NB-IoT enhancements”, Huawei, HiSilicon, Neul.