3GPP TSG-RAN WG4 Meeting #84	R4-1707276

Berlin, Germany, 21 – 25 August, 2017

Source:	Ericsson, Southern Linc

Title:	TP to TR 36.715-02-02: CA_2DL_26A-46A_2UL_26A-46A_BCS0

Agenda item:	8.6.2

Document for:	Approval

# 1. Introduction

This is a text proposal for 36.715-02-02 for inclusion of CA_2DL_26A-46A_2UL_26A-46A_BCS0 as defined in RP-171127 [1].

# 2. Text Proposal for TR 36.715-02-02

##### ---Start of changes---

## 5.x	UE co-existence table for 2UL CA including Band 46

This clause specifies the requirements for the specified carrier aggregation configurations including Band 46 for coexistence with protected bands.

# Table 5.x-1: Requirements for uplink inter-band carrier aggregation (including band 46)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_26A-46A | E-UTRA Band 1, 2, 3, 4, 5, 10, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. |  |  |  |  |  |  |  |

##### ---Text omitted---

## 6.x	CA_CA_2DL_26A-46A_2UL_26A-46A_BCS0

### 6.x.1	Operating bands for CA

# Table 6.x.1-1: Inter-band CA operating bands for CA_26-46

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_26-46 | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for CA

# Table 6.x.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_26-46

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_26A-46A | 26 |  | Yes | Yes | Yes |  |  | 30 | 0 |
|  | 46 |  |  |  |  |  | Yes |  |  |

### 6.x.3	Co-existence studies

#### 6.x.3.1	UE-UE co-existence

Table 6.x.3.1-1 lists B26 + B46 2UL CA 2nd and 3rd order harmonics. Table 6.x.3.1-2 lists the IMDs from 2nd order to 5th order for UE-UE co-existence analysis.

# Table 6.x.3.1-1: Band 26 and Band 46 UL harmonics

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 5150 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 10300 | 12500 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 15450 | 17775 |

# Table 6.x.3.1-2: 2UL Band 26 and Band 46 IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 5150 | 5925 |
| 2nd order IMD products | ¦fx_low – fy_high¦ | ¦fx_high – fy_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5111 | 4301 | 5964 | 6774 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4297 | 3452 | 9451 | 11036 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6778 | 7623 | 11114 | 12699 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | -10222 | -8692 | 11928 | 13548 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | -3483 | -2603 | 14601 | 16961 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7592 | 8472 | 16264 | 18624 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | -22886 | -19751 | 1754 | 2669 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 21414 | 24549 | 8406 | 9321 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | -16147 | -13752 | 7753 | 9408 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17078 | 19473 | 12742 | 14397 |

Based on Table 6.x.3.1-2:

3rd order IMD may also fall into Rx frequencies of bands 22, 42, 43 and 48.

4th order IMD may also fall into Rx frequencies of bands 7, 38, 41, 42 and 69.

5th order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 7, 9, 10, 23, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 65, 66, 69 and 70.

#### 6.x.3.2	Co-existence with other systems

When 2ULs inter-band CA UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.x.3.2-1 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

# Table 6.x.3.2-1: 2UL Band 26 and Band 46 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

### 6.x.4 	∆TIB and ∆RIB values

For inter-band CA_26-46, same requirements on values for ∆TIB and ∆RIB can apply with 1UL-2DL case. Therefore, for two simultaneous UL, the TIB,c and RIB,c values are shown in table 6.x.4-1 and in table 6.x.4-2:

# Table 6.x.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_26A-46A | 26 | 0 |

# Table 6.x.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_26A-46A | 26 | 0 |

# 6.x.5 	MSD

As shown in Table 6.x.3.1-2 IMD up to 5th order does not fall into own Rx of band 26 and 46. Therefore, no MSD requirement is needed for this 2DL/2UL CA configuration in TS 36.101.

## ---End of changes---

# Reference

[1]	 RP-171127, “New WID: LTE Advanced inter-band CA Rel-15 for 2DL/2UL”, RAN#74, Huawei, HiSilicon