3GPP TSG-RAN4 Meeting #84	R4-1707321

# Berlin, Germany, 21st Aug 2017 - 25th Aug 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 5031 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CA RRM: Correction of PRS Subframe Offset for TC A.8.17.10 and A.8.17.11 (Rel-14) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12 |  |  |  |  |  | Date: |  |  | 2017-08-24 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Rel-14 Mirror CR of the agreed Rel-12 CR in R4-1707319 PRS Subframe Offset values for Table A.8.17.10.1-1 and Table A.8.17.11.1-1 indicates value of 310 for the PCC and 320 for the SCC1. Based on the PRS Config-Index for these two cells (171, 181) and the reference cell SCC2 (191) and as the PRS Periodicity is 320, the PRS Subframe Offset should be 300 and 310 instead. For PCC PRS configuration index 171 Next PRS occasion for PCC is in subframe 171 + 320 = 491. PRS Subframe offset for PCC is 491-191 (PRS configuration index for SCC2 = reference) = 300 For SCC1 PRS configuration index 182 Next PRS occasion for PCC is in subframe 181 + 320 = 501. PRS Subframe offset for PCC is 501-191 (PRS configuration index for SCC2 = reference) = 310 |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table A.8.17.10.1-1 and Table A.8.17.11.1-1 Prs-SubframeOffset parameters have been corrected: Cells on PCC: from 310 to 300 Cells on SCC1: from 320 to 310 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PRS-Subframe offset will not match the prs-ConfigIndex which are configured by the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.8.17.10, A.8.17.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 37.571-5 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case

#### A.8.17.10.1	Test Purpose and Environment

The purpose of the test case is to verify that the RSTD measurements meet the requirements specified in Clause 8.4 in a synchronized network environment with fading propagation conditions. This test case will verify the measurement period requirements specified in Clause 8.4.3 for RSTD measurements performed on the same secondary component carrier, the measurement period requirements for RSTD measurements performed on both the primary and secondary component carriers specified in Clause 8.4.4, and also the measurement period requirements for RSTD measurements performed on different secondary component carriers specified in Clause 8.4.5.

In the tests, there are three configured component carriers: PCC, SCC1 and SCC2, and four synchronous cells: Cell 1, Cell 2, Cell 3 and Cell 4. Cell 1 is PCell on the PCC, Cell 2 is SCell on the SCC1, Cell 3 is SCell on the SCC2 and Cell 4 is a neighbour cell on the SCC2. In all tests, Cell 3 is the OTDOA assistance data reference cell.

The test case includes two tests. Test 1 is designed for the scenario where the UE receives OTDOA assistance data with cells only on SCC2, and the UE is expected to report RSTD measurements performed on SCC2 only. Test 2 is designed for the scenario where the UE receives OTDOA assistance data with cells on PCC, SCC1 and SCC2, and the UE is expected to report RSTD measurements performed on PCC, SCC1 and SCC2.

Each test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, Cell 2 is active only in T2 and T3, Cell 3 is active only during T2 and T3, and Cell 4 is active only during T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the OTDOA assistance data reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T3, and Cell 4 transmits PRS only in T2.

Note:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.17.10.1-1, Table A.8.17.10.1-2, Table A.8.17.10.1-3 and Table A.8.17.10.1-4.

Table A.8.17.10.1-1: General test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Parameter | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |  |
| PCell |  | Cell 1 |  |  | PCell is on RF channel 1 (PCC). |
| SCell 1 |  | Cell 2 |  |  | SCell 1 on RF channel 2 (SCC1). |
| SCell 2 |  | Cell 3 |  |  | SCell 2 on RF channel 3 (SCC2). Cell 3 is the assistance data reference cell. |
| Other neighbor cell |  | Cell 4 |  |  | Neighbor cell on RF channel 3 (SCC2). |
| PRS configuration index |  | 171 for all cells on PCC 181 for all cells on SCC1 191 for all cells on SCC2 |  |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Physical cell ID PCI |  | (PCI of Cell 3 – PCI of Cell 4)mod6=0 |  |  | The PCIs of Cell 1 and Cell 2 are selected randomly. PCIs of Cell 3 and Cell 4 are selected randomly such that the relative subcarrier shifts of PRS patterns among these cells are as given by the condition |
| CP length |  | Normal |  |  |  |
| DRX |  | ON |  |  | DRX parameters are further specified in Table A.8.17.10.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 1 to Cell 3: 1 Cell 2 to Cell 3: -1 Cell 4 to Cell 3: 3 |  |  | PRS are transmitted from synchronous cells |
| Time alignment errors between cell1, cell2 and cell3 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. |  |  | The value of time alignment error depends upon the type of carrier aggregation. |
| Expected RSTD | s | Cell 4: 2 Other neighbour cells: randomly between -3 and 3 |  | Cell 1: -2 Cell 2: 0 Cell 4: 2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Cells in OTDOA assistance data |  | 16 cells in total |  |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) and 15 other cells, all received in OTDOA-ProvideAssistanceData [24]. Cell 1 and Cell 2 (when included) appears at random places in the first half of the neighbour cell list in the OTDOA assistance data. Cell 4 always appears at random places in the second half of the list. |
|  |  | OTDOA neighbor cells include Cell 4 and other 14 cells on SCC2 | OTDOA neighbor cells include Cell 1 and other 3 cells on PCC, Cell 2 and other 3 cells on SCC1 and Cell 4 and other 6 cells on SCC2 |  |  |
| ‘prs-SubframeOffset |  | Cells on PCC: 300 Cells on SCC1: 310 Cells on SCC2, except reference cell: 0 |  |  | Subframe offset, counted in full subframes. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | Cells on PCC: 0 Cells on SCC1: 0 Cells on SCC2, except reference cell: 0 |  |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell. The corresponding parameter in the OTDOA assistance data is slotNumberOffset specified in TS 36.355 [24]. |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ Cell 4: ‘00001111’ | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ Cell 4: 0000000011111111’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  |  | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 | 2.48 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 1.28 | 2.48 |  | The length of the time interval that follows immediately after time interval T2 |

Table A.8.17.10.1-2: Cell-specific test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions during T1 for carrier aggregation

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | N/A | N/A | N/A |
| Channel Bandwidth (BWchannel) | MHz | 5,10,20 | N/A | N/A | N/A |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low | 1x2 Low | 1x2 Low |
| PCFICH/PDCCH/PHICH parameters as specified in clause A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | N/A | N/A | N/A |
| OCNG patterns defined in A.3.2.1 |  | 5MHz: OP.18 FDD 10MHz: OP.5 FDD 20MHz: OP.13 FDD | N/A | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A | N/A |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 | N/A | N/A | N/A |
| PRS | dB | -Infinity | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 +10log (NRB,c /50) | N/A | N/A | N/A |
|  | dB | 0 | -Infinity | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.8.17.10.1-3: Cell-specific test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions during T2 and T3 for carrier aggregation

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  | 3 |  |
| Channel Bandwidth (BWchannel) | MHz | 5,10,20 |  | 5,10,20 |  | 5,10,20 |  | 5,10,20 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| PCFICH/PDCCH/PHICH parameters as specified in clause A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | N/A |
| OCNG patterns defined in A.3.2.1 (There is no PDSCH allocated in the subframe transmitting PRS) |  | 5MHz: OP.18 FDD 10MHz: OP.5 FDD 20MHz: OP.13 FDD |  | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD |  | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD |  | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD | N/A |
| PRS Transmission Bandwidth (PRS transmission bandwidth depends on selected channel bandwidth. PRS are transmitted over the system bandwidth) | RB | 5MHz: 25 10MHz: 50 20MHz:100 |  | 5MHz: 25 10MHz: 50 20MHz:100 |  | 5MHz: 25 10MHz: 50 20MHz:100 |  | 5MHz: 25 10MHz: 50 20MHz:100 | N/A |
| Number of consecutive downlink positioning subframes . also depends on selected channel bandwidth. As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |  | 5MHz: 2 10MHz: 1 20MHz:1 |  | 5MHz: 2 10MHz: 1 20MHz:1 |  | 5MHz: 2 10MHz: 1 20MHz:1 |  | 5MHz: 2 10MHz: 1 20MHz:1 | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |  |  |
| PRS_RA | dB | -6 | N/A | N/A | 3 | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -4 | -Infinity | -Infinity | -1 | -Infinity | -1 | -8 | -Infinity |
| PRS Note 4 | dB | -4 | -Infinity | -Infinity | -1 | -Infinity | -1 | -8 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.94 +10log (NRB,c /50) | N/A | N/A | -66.68 +10log (NRB,c /50) | N/A | -66.68 +10log (NRB,c /50) | -70.11 +10log (NRB,c /50) | N/A |
| PRP Note 4 | dBm/ 15 kHz | -102 | -Infinity | -Infinity | -96 | -Infinity | -96 | -106 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -96 | -105 | -99 | -105 | -99 | -109 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -4 | -7 | -4 | -11 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |  |  |

Table A.8.17.10.1-4: DRX parameters for the test of E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.17.10.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 8.4.

In Test 1, the UE shall perform and report the RSTD measurements from Cell 4 and Cell 3 within 2560 ms starting from the beginning of time interval T2.

In Test 2, the UE shall perform and report the RSTD measurements from Cell 1 and Cell 3, RSTD measurements from Cell 2 and Cell 3, and RSTD measurements from Cell 4 and Cell 3 within 4960 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement times in the tests are derived from the following expression, ,

where =8 and =16  for Test 1, and =16 and =16 for Test 2. For Test 1, the  and  parameters specified in Clause 8.1.2.5.1, Table 8.1.2.5.1-1, under Note 1, which gives the total RSTD measurement time of 2560 ms for Cell 4 with respect to the reference cell Cell 3. For Test 2, the  and  parameters are specified in Clause 8.1.2.6.1, Table 8.1.2.6.1-1, under Note 1, which gives the total RSTD measurement time of 4960 ms for reporting the RSTD measurements of Cell 1, Cell 2 and Cell 4 with respect to the reference cell Cell 3.

### A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case

#### A.8.17.11.1	Test Purpose and Environment

The purpose of the test case is to verify that the RSTD measurements meet the requirements specified in Clause 8.4 in a synchronized network environment with fading propagation conditions. This test case will verify the measurement period requirements specified in Clause 8.4.3 for RSTD measurements performed on the same secondary component carrier, the measurement period requirements for RSTD measurements performed on both the primary and secondary component carriers specified in Clause 8.4.4, and also the measurement period requirements for RSTD measurements performed on different secondary component carriers specified in Clause 8.4.5.

In the tests, there are three configured component carriers: PCC, SCC1 and SCC2, and four synchronous cells: Cell 1, Cell 2, Cell 3 and Cell 4. Cell 1 is PCell on the PCC, Cell 2 is SCell on the SCC1, Cell 3 is SCell on the SCC2 and Cell 4 is a neighbour cell on the SCC2. In all tests, Cell 3 is the OTDOA assistance data reference cell.

The test case includes two tests. Test 1 is designed for the scenario where the UE receives OTDOA assistance data with cells only on SCC2, and the UE is expected to report RSTD measurements performed on SCC2 only. Test 2 is designed for the scenario where the UE receives OTDOA assistance data with cells on PCC, SCC1 and SCC2, and the UE is expected to report RSTD measurements performed on PCC, SCC1 and SCC2.

Each test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, Cell 2 is active only in T2 and T3, Cell 3 is active only during T2 and T3, and Cell 4 is active only during T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the OTDOA assistance data reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T3, and Cell 4 transmits PRS only in T2.

Note:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.17.11.1-1, Table A.8.17.11.1-2, Table A.8.17.11.1-3 and Table A.8.17.11.1-4.

Table A.8.17.11.1-1: General test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Parameter |  | Unit |  | Value |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  | Test 2 |  |  |  |
| PCell |  |  |  | Cell 1 |  |  |  | PCell is on RF channel 1 (PCC). |  |
| SCell 1 |  |  |  | Cell 2 |  |  |  | SCell 1 on RF channel 2 (SCC1). |  |
| SCell 2 |  |  |  | Cell 3 |  |  |  | SCell 2 on RF channel 3 (SCC2). Cell 3 is the assistance data reference cell. |  |
| Other neighbor cell |  |  |  | Cell 4 |  |  |  | Neighbor cell on RF channel 3 (SCC2). |  |
| Channel Bandwidth (BWchannel) |  | MHz |  | 5MHz or 10MHz or 20MHz |  |  |  | All channels in a test have the same bandwidth. |  |
| PRS configuration index |  |  |  | 174 for all cells on PCC 184 for all cells on SCC1 194 for all cells on SCC2 |  |  |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |  |
| Physical cell ID PCI |  |  |  | (PCI of Cell 3 – PCI of Cell 4)mod6=0 |  |  |  | The PCIs of Cell 1 and Cell 2 are selected randomly. PCIs of Cell 3 and Cell 4 are selected randomly such that the relative subcarrier shifts of PRS patterns among these cells are as given by the condition |  |
| TDD uplink-downlink configuration |  |  |  | 1 |  |  |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |  |
| TDD special subframe configuration |  |  |  | 6 |  |  |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |  |
| CP length |  |  |  | Normal |  |  |  |  |  |
| DRX |  |  |  | ON |  |  |  | DRX parameters are further specified in Table A.8.17.11.1-3 |  |
| Radio frame receive time offset between the cells at the UE antenna connector |  |  | s |  | Cell 1 to Cell 3: 1 Cell 2 to Cell 3: -1 Cell 4 to Cell 3: 3 |  |  |  | PRS are transmitted from synchronous cells |  |
| Time alignment errors among cell1, cell2 and cell3 |  | s |  | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. |  |  |  | The value of time alignment error depends upon the type of carrier aggregation. |  |
| Expected RSTD |  | s |  | Cell 4: 2 Other neighbour cells: randomly between -3 and 3 | Cell 1: -2 Cell 2: 0 Cell 4: 2 Other neighbour cells: randomly between -3 and 3 |  |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |  |
| Expected RSTD uncertainty for all neighbour cells |  | s |  | 5 |  |  |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |  |
| Cells in OTDOA assistance data |  |  |  | 16 cells in total |  |  |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) and 15 other cells, all received in OTDOA-ProvideAssistanceData [24]. Cell 1 and Cell 2 (when included) appears at random places in the first half of the neighbour cell list in the OTDOA assistance data. Cell 4 always appears at random places in the second half of the list. |  |
|  |  |  |  |  | OTDOA neighbor cells include Cell 4 and other 14 cells on SCC2 |  | OTDOA neighbor cells include Cell 1 and other 3 cells on PCC, Cell 2 and other 3 cells on SCC1 and Cell 4 and other 6 cells on SCC2 |  |  |  |
| prs-SubframeOffset |  |  |  | Cells on PCC: 300 Cells on SCC1: 310 Cells on SCC2, except reference cell: 0 |  |  |  | Subframe offset, counted in full subframes. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |  |
| slotNumberOffset |  |  |  | Cells on PCC: 0 Cells on SCC1: 0 Cells on SCC2, except reference cell: 0 |  |  |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell. The corresponding parameter in the OTDOA assistance data is slotNumberOffset specified in TS 36.355 [24]. |  |
| PRS muting info |  |  |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ Cell 4: ‘00001111’ |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ Cell 4: 0000000011111111’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |  |
| T1 |  | s |  | 3 |  |  |  | The length of the time interval from the beginning of each test |  |
| T2 |  | s |  | 1.28 |  | 2.48 |  | The length of the time interval that follows immediately after time interval T1 |  |
| T3 |  | s |  | 1.28 |  | 2.48 |  | The length of the time interval that follows immediately after time interval T2 |  |

Table A.8.17.11.1-2: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T1 for carrier aggregation

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | N/A | N/A | N/A |
| Channel Bandwidth (BWchannel) | MHz | 5,10,20 | N/A | N/A | N/A |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low | 1x2 Low | 1x2 Low |
| PCFICH/PDCCH/PHICH parameters as specified in clause A.3.1.2.1 |  | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD | N/A | N/A | N/A |
| OCNG patterns defined in A.3.2.2 |  | 5MHz: OP.10 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | N/A | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A | N/A |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 | N/A | N/A | N/A |
| PRS | dB | -Infinity | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 +10log (NRB,c /50) | N/A | N/A | N/A |
|  | dB | 0 | -Infinity | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.8.17.11.1-3: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T2 and T3 for carrier aggregation

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  | 3 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Channel Bandwidth (BWchannel) | MHz | 5,10,20 |  | 5,10,20 |  | 5,10,20 |  | 5,10,20 |  |
| PCFICH/PDCCH/PHICH parameters as specified in clause A.3.1.2.1 |  | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD |  | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD |  | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD |  | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD | N/A |
| OCNG patterns defined in A.3.2.1 |  | 5MHz: OP.10 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | N/A |
| PRS Transmission Bandwidth (PRS transmission bandwidth depends on selected channel bandwidth. PRS are transmitted over the system bandwidth) | RB | 5MHz: 25 10MHz: 50 20MHz:100 |  | 5MHz: 25 10MHz: 50 20MHz:100 |  | 5MHz: 25 10MHz: 50 20MHz:100 |  | 5MHz: 25 10MHz: 50 20MHz:100 | N/A |
| Number of consecutive downlink positioning subframes . also depends on selected channel bandwidth. As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |  | 5MHz: 2 10MHz: 1 20MHz:1 |  | 5MHz: 2 10MHz: 1 20MHz:1 |  | 5MHz: 2 10MHz: 1 20MHz:1 |  | 5MHz: 2 10MHz: 1 20MHz:1 | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |  |  |
| PRS_RA | dB | -6 | N/A | N/A | 3 | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -4 | -Infinity | -Infinity | -1 | -Infinity | -1 | -8 | -Infinity |
| PRS Note 4 | dB | -4 | -Infinity | -Infinity | -1 | -Infinity | -1 | -8 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.94 +10log (NRB,c /50) | N/A | N/A | -66.68 +10log (NRB,c /50) | N/A | -66.68 +10log (NRB,c /50) | -70.11 +10log (NRB,c /50) | N/A |
| PRP Note 4 | dBm/ 15 kHz | -102 | -Infinity | -Infinity | -96 | -Infinity | -96 | -106 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -96 | -105 | -99 | -105 | -99 | -109 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -4 | -7 | -4 | -11 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |  |  |

Table A.8.17.11.1-4: DRX parameters for the test of E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.17.11.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 8.4.

In Test 1, the UE shall perform and report the RSTD measurements from Cell 4 and Cell 3 within 2560 ms starting from the beginning of time interval T2.

In Test 2, the UE shall perform and report the RSTD measurements from Cell 1 and Cell 3, RSTD measurements from Cell 2 and Cell 3, and RSTD measurements from Cell 4 and Cell 3 within 4960 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement times in the tests are derived from the following expression, ,

where =8 and =16  for Test 1, and =16 and =16 for Test 2. For Test 1, the  and  parameters specified in Clause 8.1.2.5.1, Table 8.1.2.5.1-1, under Note 1, which gives the total RSTD measurement time of 2560 ms for Cell 4 with respect to the reference cell Cell 3. For Test 2, the  and  parameters are specified in Clause 8.1.2.6.1, Table 8.1.2.6.1-1, under Note 1, which gives the total RSTD measurement time of 4960 ms for reporting the RSTD measurements of Cell 1, Cell 2 and Cell 4 with respect to the reference cell Cell 3.

< End of changes >