3GPP TSG-RAN WG4 Meeting #84		R4-1707379

Berlin, Germany, 21 - 25 August 2017

Agenda item:	7.15.2.2

Source:	Intel Corporation

Title:	E-FD-MIMO Class A PMI reporting performance requirements

Document for:	Discussion

# Introduction

In the RAN4 #84 meeting a WF on eFD-MIMO performance requirements [1] was agreed. The following agreements on the Class A PMI reporting performance requirements were made:

- Test 1 Single PMI test case: EPA5Hz, PUSCH 3-1 mode, CDM4, (O1,O2) = (8,4), (N1,N2) = (3,4) - Antenna Ports: - 24 ports (N1,N2) = (3,4) - Test 1-a: without CSI-RS density reduction - Test 1-b: with CSI-RS density reduction - d = 1/3, Comb-offset list: {0,1,2} - Pending on UE capability for supporting CSI-RS density reduction, pick up one of above test case to pass - Test 2 Multiple PMI test case(32 ports): EVA5Hz, PUSCH 1-2 mode, CDM8, (O1,O2) = (8,4),(N1,N2) = (4,4), d=1 - MCS and Rank - Single PMI test case: - Option1: 16QAM ½ Rank 2 - Option2: 64QAM ½ Rank 1 - Multiple PMI test case - Option 1: 16QAM ½ Rank 2 - Option 2: 64QAM ½ Rank 2 - Reusing existing values for parameters α1, α2, β and γ for high spatial correlation

In this contribution we provide our link level results for Class A PMI test cases and share our views on the test parameters.

# Discussion

In this section we provide our views on the remaining open issues on eFD-MIMO Class A PMI test case definition:

- MCS and Rank

- Performance requirement

Modulation format/Rank settings

In Rel-13 FD-MIMO WI it was decided to use 64QAM Rank 1 for Single PMI tests (12 ports) and 16QAM Rank 2 for Multiple PMI test (16 ports). So, our suggestion is to reuse assumptions from Rel-13 FD-MIMO for Rel-14 E-FD-MIMO Class A requirements definition.

Proposal #1: Use the following MCS for Class A requirements definition: Single PMI 24 ports – 64QAM Rank 1, Multiple PMI 32 ports – 16QAM Rank2.

Performance requirement

In Rel-13 FD-MIMO the PMI requirements was defined as ratio of throughputs in scenarios with follow PMI and random PMI:

γ=tue,follow1,1,follow1,2,follow2trnd1,1,rnd1,2,rnd2

where tue,follow1,1,follow1,2,follow2 is 90% of the maximum throughput obtained at SNRfollow1,1,follow1,2,follow2 using the precoders configured according to the UE reports, and trnd1,1,rnd1,2,rnd2 is the throughput measured at SNRfollow1,1,follow1,2,follow2  with random precoding.

From our simulation results from Section 3 in can be observed that γ can achieve rather high values (up to 19) and the particular value is very sensitive to the SNR point. For requirements definition usually a conservative value is used (e.g. minimum proposal from all the companies). Assuming very big reported γ values, using conservative requirement may lead to situation that UE can pass test in case of wrong implementations (e.g. W1 chosen correctly and W2 – incorrectly). Based on such observation, we think that additional analysis on the possible correct/wrong UE implementations is needed in order to ensure that performance requirement value is chosen in a way that proper UE implementation is guaranteed.

Proposal #2: Reuse Rel-13 FD-MIMO test metric and methodology for Rel-14 eFD-MIMO Class A PMI requirements. Further discuss the criteria to define the γ value taking into account possible wrong/correct UE implementations.

# Simulation results

In the last RAN4 meeting it was agreed to consider the following scenarios for Class A UE requirements definition

- Test 1. Single PMI test case : EPA5Hz, PUSCH 3-1 mode, 24 ports, CDM4, (O1,O2) = (8,4), (N1,N2) = (3,4)

  - Test 1-a. Without CSI-RS density reduction

  - Test 1-b. With CSI-RS density reduction, d =1/3, Comb-offset list: {0,1,2}

- Test 2. Multiple PMI test case: EVA5Hz, PUSCH 1-2 mode, 32 ports, CDM8, (O1,O2) = (8,4),(N1,N2) = (4,4), d=1

In Figure 1 we provide our link level results for E-FD-MIMO Class A Single and Multiple PMI test cases using MCS configuration from Table 1.

Table 1. Class A test cases FRC

| Reference channel |  | Test 1a/1b + Rank 2 16QAM 1/2 | Test 1a/1b + Rank 1 64QAM 1/2 | Test 2 + Rank 2 16QAM 1/2 | Test 2 + Rank 2 64QAM 1/2 |
| --- | --- | --- | --- | --- | --- |
| Modulation |  | 16QAM | 64QAM | 16QAM | 64QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 1/2 | 1/2 |
| Information Bit Payload |  |  |  |  |  |
| For Sub-Frames (Non CSI-RS subframe) | Bits | 11448 (MCS 13) | 18336 (MCS 19) | 11448 (MCS13) | 18336 (MCS 19) |
| For Sub-Frames (CSI-RS subframe) | Bits | 9912 (MCS 12)/ 11448 (MCS 13) | 15264 (MCS 17)/ 16416 (MCS 18) | 8760 (MCS 11) | 15264 (MCS 17) |
| For Sub-Frames (ZeroPowerCSI-RS subframe) | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 9528 (MCS 13) | 14688 (MCS 19) | 9528 (MCS 13) | 14688 (MCS 19) |

| Test 1-a. 16QAM Rank 2. |  |
| --- | --- |
| Test 1-a. 64QAM Rank 1. |  |
| Test 1-b. 16QAM Rank 2. |  |
| Test 1-b. 64QAM Rank 1. |  |
| Test 2. 16 QAM Rank 2. |  |
| Test 2. 64 QAM Rank 2. |  |
| Figure 1. Class A simulation results. |  |

Table 2. Results statistics (SNR point/T-put ratio) for CSS1

| Requirement value | Test 1-a. |  | Test 1-b. |  | Test 2. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 16QAM | 64QAM | 16QAM | 64QAM | 16QAM | 64QAM |
| 80% @ Max T-put | 2.1 / 10.8 | -1.3 / 25.7 | 1.9 / 11.2 | -1.6 / 28.0 | 0.3 / 21.7 | 7.0 / 13.8 |
| 85% @ Max T-put | 3.1 / 9.3 | -0.8 / 21.7 | 2.9 / 9.5 | -1.1 / 23.5 | 0.9 / 18.9 | 7.7 / 12.5 |
| 90% @ Max T-put | 4.2 / 7.8 | -0.1 / 18.1 | 4.1 / 7.9 | -0.4 / 19.4 | 1.7 / 16.2 | 8.5 / 11.0 |
| 95% @ Max T-put | 5.8 / 6.2 | 0.9 / 14.2 | 5.7 / 6.2 | 0.6 / 14.9 | 2.7 / 13.3 | 9.6 / 9.3 |

# Conclusions

The complete set of the alignment results is provided in the attached Excel spreadsheet:

In this contribution we provided our views on the UE demodulation performance requirements for E-FD-MIMO Class A test cases. In summary, we make the following proposals:

Proposal #1: Use the following MCS for Class A requirements definition: Single PMI 24 ports – 64QAM Rank 1, Multiple PMI 32 ports – 16QAM Rank2.

Proposal #2: Reuse methodology from Rel-13 FD-MIMO as one of methodology for Rel-14 eFD-MIMO Class A PMI requirements. FFS if additional requirement metric for Rel-14 eFD-MIMO Class A PMI requirement is necessary.

# References

- R4-1706194 “Way forward on eFD-MIMO performance requirements”, Samsung, Qualcomm, Huawei, Ericsson, RAN4 #83, May 2017.