3GPP TSG-RAN WG4 Meeting #84	R4-1707658

Berlin, Germany, 21 – 25 August 2017

Agenda Item:	6.2.4.2

Source:	Ericsson

Title:	Simulation result for Non-BL CE UE subband CQI test

Document for:	Discussion

# 1	Introduction

RAN4#83 agreed with the way forward on requirements for Rel-13 Non-BL CE UEs [1]:

- Interested companies are encouraged to propose the test points for the following CQI tests for non-BL UE with 2Rx/4Rx in RAN4#84, e.g., 3dB for 2Rx and 6dB for 4Rx.

  - CQI definition tests specified in TS36.101 9.8.1

  - UE-selected subband CQI tests specified in TS36.101 9.8.2

This contribution presents the simulation result for subband CQI test considering 2Rx/4Rx UE supporting coverage enhancements.

# 2	Simulation results

Figure 1 shows the median of the reported CQI values for 1Rx/2Rx/4Rx UE based on the parameters specified in TS36.101 Table 9.8.2.1-1. However, as we are discussing in [2], we set the following values:

- MPDCCH narrowband = 7 (mpdcch-Narrowband)

- MPDCCH repetition = 4 (mpdcch-NumRepetition)

- G = 2.5 (mpdcch-StartSF-UESS)

- Hopping interval = 1 (interval-FDD)

Since our proposal in [2] keeps SNR=5dB and 6dB, the median CQI from the reported CQI values are 7 and 8 for single Rx UE, respectively. From the simulation result, we can reduce the test point by 3dB for 2Rx and by 6dB for 4Rx.

Figure 1	Median CQI from subband CQI test.

Table 1 and Table 2 summarizes the simulation results for CQI definition test for 2Rx and 4Rx, respectively, to investigate the non-BL UE can satisfy the following requirements:

- the ratio of the throughput obtained when transmitting on the best narrowband reported by the UE the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected narrowband in set S shall be ≥ γ.

For 2Rx UE, if we focus on SNR=2dB/3dB, the throughput ratio values of followed CQI/narrowband over WB CQI median with random narrowband are 1.72 and 1.58, respectively. For 4Rx, if we focus on SNR=-1dB/0dB, the throughput ratio values are 1.78 and 1.62, respectively. For reference, we show the single Rx simulation result in Table 3. If we compared three tables, there is no difference with regard to the throughput ratio values. We therefore propose to set the same requirements, i.e., gamma=1.3, for non-BL UEs with 2Rx/4Rx.

Table 1	Simulation result of subband CQI test for 2Rx UE.

| SNR [dB] | BLER | Wideband median CQI | Throughput ratio |
| --- | --- | --- | --- |
| 0 | 0.24 | 6 | 1.75 |
| 1 | 0.21 | 6 | 1.88 |
| 2 | 0.25 | 7 | 1.72 |
| 3 | 0.23 | 8 | 1.58 |
| 4 | 0.23 | 8 | 1.72 |
| 5 | 0.20 | 9 | 1.58 |

Table 2	Simulation result of subband CQI test for 4Rx UE.

| SNR [dB] | BLER | Wideband median CQI | Throughput ratio |
| --- | --- | --- | --- |
| -3 | 0.26 | 6 | 1.78 |
| -2 | 0.22 | 6 | 1.91 |
| -1 | 0.24 | 7 | 1.78 |
| 0 | 0.22 | 8 | 1.62 |
| 1 | 0.24 | 8 | 1.73 |
| 2 | 0.19 | 9 | 1.61 |

Table 3	Simulation result of subband CQI test for 1Rx UE ([2]).

| SNR [dB] | BLER | Wideband median CQI | Throughput ratio |
| --- | --- | --- | --- |
| 3 | 0.24 | 6 | 1.72 |
| 4 | 0.20 | 6 | 1.91 |
| 5 | 0.23 | 7 | 1.72 |
| 6 | 0.21 | 8 | 1.61 |
| 7 | 0.24 | 8 | 1.67 |
| 8 | 0.21 | 9 | 1.56 |

# 3	Conclusion

In this contribution, we show the simulation results for suband CQI test with 2Rx/4RX UE and observed that we can reduce the SNR test points by 3dB for 2Rx and by 6dB for 4Rx UE.

Proposal 1: Set the following test points for non-BL UE subband CQI tests.

|  | FDD | TDD |
| --- | --- | --- |
| 2Rx | -1/0dB | -1/0dB |
| 4Rx | 2/3dB | 2/3dB |
| 1Rx (Ref) | 5/6dB | 5/6dB |

Proposal 2: Set γ=1.3 for non-BL UE.

# 4	References

R4-1706196, “WF on requirements for Rel-13 Non-BL UE”, Huawei, HiSilicon, Ericsson, Intel Corporation, Qualcomm.

R4-1707659, “Scheduling pattern for eMTC subband CQI test”, Ericsson.