3GPP TSG-RAN WG4 Meeting #84	R4-1707674

Berlin, Germany, 21 – 25 August 2017

Agenda Item:	7.9.4

Source:	Ericsson

Title:	RLM requirements for non-BL CE UE in Rel-14

Document for:	Discussion

# 1	Introduction

RAN4#83 agreed with the way forward on RLM requirements for non-BL/CE UE as follow [1]:

- RAN4 will study the RLM evaluation period for BL/CE UE with 2Rx, when no DRX is used, for the following scenarios in Rel-14

  - CE Mode A Out-of-synch

  - CE Mode A In-synch

  - CE Mode B Out-of-synch

  - CE Mode B In-synch

- Companies are encouraged to provide the investigation results for the RLM evaluation period considering the effectiveness of receiver diversity in RAN4#84

  - Side condition:

    - CE Mode A: EPA5/ETU30 with SNR = -6dB

    - CE Mode B: EPA1/ETU1 with SNR = -15dB

This contribution discusses the measurement period for non-BL/CE UE in Rel-14.

# 2	RLM requirements for non-BL UE

## 2.1	Discussion

Radio link monitoring (RLM) is the functionality to monitor the quality of the serving cell. For RLM, UE estimates the downlink radio link quality and compare it with the threshold Qout and Oin every radio frame. If the quality level is lower than Qout, UE judges it is out-of-synch. If the quality level is higher than Qin, then UE judges it is in-synch. For BL/CE UEs such as Category M1 or M2, Qout is derived from 10% BLER of the hypothetical MPDCCH transmission and Qin is derived from the 2% BLER of the hypothetical MPDCCH transmission, because BL/CE UEs should monitor MPDCCH instead of PCFICH/PDCCH.

The quality level evaluation is supposed to be based on the estimated SNR values from the cell-specific reference symbols. In TS36.133, the out-of-synch evaluation period and in-synch evaluation period for CE Mode A is specified as 400ms and 200ms, respectively, when no DRX is used. Similarly, the out-of-synch evaluation period and in-synch evaluation period for CE Mode B is specified as 4,000ms and 2,000ms, respectively, when no DRX is used. These L1 measurement periods are derived from the assumption UE has only single receiver antenna.

For non-BL CE UE, we have proposed to shorten the measurement period because of two receive antennas. RAN4#83 discussed the concern to shorten the measurement period because very slow Doppler fading environment such as 1Hz assumed for CE Mode B needs longer measurement time to capture the longer coherent time. We think the two receive antennas can help to shorten the measurement period considering the receive diversity. This contribution compares the RSRP measurement errors between single Rx UE and dual Rx antenna UE.

## 2.2	Simulation parameters

Table 1 is the simulation parameters used for evaluating RLM in UE Category M1.

Table 1	Simulation parameters for RLM for coverage enhancement UE with 2Rx.

| Parameter |  |
| --- | --- |
| System Bandwidth | 10 MHz |
| Channel model | AWGN, ETU30, EPA5, EPA1, ETU1 |
| Antenna configuration | 1x1, 1x2 |
| Antenna correlation | Low |
| RSRP sampling period | 10ms |
| L3 filtering | Disabled |

## 2.3	Simulation results

Figure 3 shows examples of RSRP errors for single Rx receiver and dual Rx receiver. For the dual Rx receiver case, we average over RSRP samples from each receive antenna. These figures show the 5%-ile, 50%-ile, and 95%-ile of RSRP errors for EPA1 with different L1 measurement periods: 400ms, 800ms, 1600ms, and 2000ms. It is observed that the difference between the 95%-ile of RSRP error and 5%-ile of RSRP with dual receive antenna is smaller compared with single Rx results regardless of L1 measurement period. This implicates RSRP estimation with 2Rx becomes more accurate than 1Rx.

In this paper, we focus on how much the L1 measurement period is reduced with keeping the same accuracy.

### 2.3.1	CE Mode A

Figure 1	RSRP measurement error for EPA5

Figure 2	RSRP measurement error for ETU30.

Table 2 summarizes the L1 measurement period for 2Rx UE to keep the same RSRP error considering CE Mode A (SNR = -5dB). For example, the 5%-ile of RSRP error for ETU30 at SNR=-5dB is -2.79dB for 1Rx UE with 400ms measurement period. For 2Rx UE, the same RSRP error can be achieved with 220ms. It is observed from our simulation result that the measurement period could be reduced from 400ms to 130 - 220ms. Considering the margin, we propose to reduce the measurement period by 2dB.

Proposal 1: For non-BL UE supporting CE Mode A, the L1 measurement periods for out-of-synch is 260ms and for in-synch is 130ms.

Table 2	Summary of RSRP measurement error for CE Mode A scenario.

|  |  |  |  | Measurement time |  |
| --- | --- | --- | --- | --- | --- |
| Channel model | SNR | Condition | RSRP Error [dB] | 1Rx | 2Rx |
| ETU30 | -5dB | 5%-ile | -2.79 | 400ms | 220ms |
|  |  | 95%-ile | +0.07 | 400ms | 170ms |
| EPA5 | -5dB | 5%-ile | -3.21 | 400ms | 190ms |
|  |  | 95%-ile | +2.86 | 400ms | 130ms |

### 2.3.2	CE Mode B

Figure 3	Example of RSRP measurement simulation result with different measurement period.

Figure 4	RSRP measurement error for EPA1.

Figure 5	RSRP measurement error for ETU1.

Table 3	Summary of RSRP measurement error for CE Mode B scenario.

|  |  |  |  | Measurement time |  |
| --- | --- | --- | --- | --- | --- |
| Channel model | SNR | Condition | RSRP Error [dB] | 1Rx | 2Rx |
| EPA1 | -15dB | 5%-ile | +0.79 | 4000ms | 1500ms |
|  |  | 95%-ile | +3.16 | 4000ms | 3100ms |
| ETU1 | -15dB | 5%-ile | -0.21 | 4000ms | 1200ms |
|  |  | 95%-ile | +2.11 | 4000ms | 3200ms |

Table 3 summarizes the L1 measurement period for 2Rx UE to keep the same RSRP error considering CE Mode B (SNR=-15dB). For example, the 5%-ile of RSRP error for EPA1 at SNR=-15dB is +0.79dB for 1Rx UE with 4000ms measurement period. For 2Rx UE, the same RSRP error can be achieved with 1,500ms. It is observed from our simulation result that the measurement period could be reduced from 4,000ms to 1,200 – 3,220ms. Considering the margin, we propose to reduce the measurement by 20%.

Proposal 2: For non-BL UE supporting CE Mode B, the L1 measurement periods for out-of-synch is 3,200ms and for in-synch is 1,600ms.

# 3	Conclusion

Proposal 1: For non-BL UE supporting CE Mode A, the L1 measurement periods for out-of-synch is 260ms and for in-synch is 130ms.

Proposal 2: For non-BL UE supporting CE Mode B, the L1 measurement periods for out-of-synch is 3,200ms and for in-synch is 1,600ms.

# 4	References

R4-1706184, “Way forward on non-BL/CE UE RLM evaluation period”, Ericsson.