3GPP TSG-RAN WG4 Meeting #84	R4-1707680

Berlin, Germany, 21 – 25 August 2017

Agenda Item:	7.8.5

Source:	Ericsson

Title:	Cat-NB2 NPDSCH simulation results with impairments

Document for:	Discussion

# 1	Introduction

RAN4#83 agreed to define one NPDSCH demodulation performance requirements with larger TBS 936bits for Rel-14 NB-IoT UE Category NB2 irrespective of the support of 1 or 2 HARQ process [1], and ideal simulation results are summarized in [2].

This contribution provides our simulation results with impairment to finalize the demodulation requirements.

# 2	Simulation results

Table 1 is the summary of test case for NDPSCH demodulation requirements for Cat-NB2. Table 2 gives the SNR to achieve 70% of the maximum throughput without impairments [3] and with impairments.

We propose to consider our results to specify the NPDSCH demodulation requirements for Cat-NB2.

Table 1	Test cases for NPDSCH for Cat-NB2.

| Test number | Deployment mode | Carrier Type | (ITBS, ISF) | Coding rate | Repetition number | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | Stand-alone | Non-Anchor | (9, 5) (TBS=936 bits) | 0.5 | 1 | EPA5 | 1 | 70% | TBD |

Table 2	Simulation results for PDSCH for Cat-NB2.

| Test number | SNR@70% of maximum throughput (Ideal) | SNR@70% of maximum throughput (with impairments) |
| --- | --- | --- |
| 1 | 8.8 | 10.8 |

# 3	References

R4-1707001, “RAN4#83 Meeting report”, 3GPP TSG RAN WG4.

R4-1705526, “Summary of NB-IoT enh UE simulation results”, Hauwei, HiSilicon.

R4-1704809, “UE demodulation simulation results for eNB-IoT”, Ericsson.