3GPP TSG-WG RAN4 Meeting #84 	R4-1707687

# Berlin, Germany, 21st – 25th Aug, 2017

Source: 	Nokia, Nokia Shanghai Bell, Telia

Title: 					TP to TR 36.715-04-01: CA_4DL_3C-20A-28A_1UL_BCS0

Agenda Item: 			8.4.2

Document for:	Approval

# Introduction

This TP introduces 4DL/1UL CA configuration CA_3C-20A-28A in to the TR 36.715-04-01

**************************** Start of changes ********************************************

## 6.X	CA_3C-20A-28A_BCS0

### 6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3-20-28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_3C-20A-28A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  | 80 | 0 |
|  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.X.3	Co-existence studies

Table 6.X.3-3 summarizes frequency ranges where harmonics occur due to Band 3, Band 20 and Band 28 CA with 1 UL.

Table 6.X.3-3: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 3 | 1710 | 1785 | 3420 | 3575 | 5130 | 5355 |
| 20 | 832 | 862 | 1664 | 1724 | 2496 | 2586 |
| 28 | 703 | 748 | 1406 | 1496 | 2109 | 2244 |

It can be seen from Table 6.X.3-3 for CA_3C-20A-28A that there is no harmonic interterference from low bands to the high band receiver band.

### 6.X.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band 3, Band 20 and Band 28 for 1UL/4DL CA_3C-20A-28A, the TIB,c and RIB,c values are shown in table 6.X.4-1 and table 6.X.4-2, respectively, which are the same as CA_3A-20A-28A.

Table 6.X.4-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3C-20A-28A | 3 | 0.3 |
|  | 20 | 0.5 |
|  | 28 | 0.5 |

Table 6.X.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3C-20A-28A | 3 | 0 |
|  | 20 | 0.1 |
|  | 28 | 0.1 |

### 6.X.5	Out-of-band blocking

For a UE supporting CA_3C-20A-28A, the out-of-band blocking requirements for Band 20 and Band 28 apply with FDL_low given by the lower limit of the restricted operating frequency range in Band 28 and FDL_high by Band 20, where FDL_low and FDL_high are specified in Table 5.5A-2 of TS36.101.

**************************** End of changes ********************************************