3GPP TSG-RAN4 Meeting #84	R4-1707811

# Berlin, DE, 21st – 25th August 2017

Source: 	Qualcomm Incorporated

Title: 	Sensitivity for FDD for CAT-M2

Agenda item:	7.9.2

Document for:	Approval

# Introduction

In [1], the sensitivity values for FDD CAT-M2 were set in [] to be finalized in this meeting. This paper presents the values for the same.

# Discussion

When deriving the sensitivity values for CAT0 from LTE with diversity, the sensitivity values are relaxed by 2.5 dB for all bands except for B13. For B13 CAT0 sensitivity is relaxed by 3 dB for all BWs. So, following the same approach for CATM2 the following sensitivity values are proposed.

Proposal 1:

Table 7.3.1E-8:  Reference sensitivity for FDD /TDD UE category [M2] QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
|  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |  |
| 1 |  |  | -97.7 | -97.7 | -97.7 | -97.7 | FDD |
| 2 | -100.2 | -97.2 | -95.7 | -95.7 | -95.7 | -95.7 | FDD |
| 3 | -99.2 | -96.2 | -94.7 | -94.7 | -94.7 | -94.7 | FDD |
| 4 | -102.2 | -99.2 | -97.7 | -97.7 | -97.7 | -97.7 | FDD |
| 5 | -100.7 | -97.7 | -95.7 | -95.7 |  |  | FDD |
| 7 |  |  | -95.7 | -95.7 | -95.7 | -95.7 | FDD |
| 8 | -99.7 | -96.7 | -94.7 | -94.7 |  |  | FDD |
| 11 |  |  | -97.7 | -97.7 |  |  |  |
| 12 | -99.2 | -96.2 | -94.7 | -94.7 |  |  | FDD |
| 13 |  |  | -94.2 | -94.2 |  |  | FDD |
| 18 |  |  | -98.4 | -98.4 | -98.4 |  | FDD |
| 19 |  |  | -97.7 | -97.7 | -97.7 |  | FDD |
| 20 |  |  | -94.7 | -94.7 | -94.7 | -94.7 | FDD |
| 21 |  |  | -97.7 | -97.7 | -97.7 | -97.7 | FDD |
| 25 | -98.7 | -95.7 | -94.2 | -94.2 | -94.2 | -94.2 | FDD |
| 26 | -100.2 | -97.2 | -95.2 | -95.2 | -95.2 |  | FDD |
| 27 | -100.7 | -97.7 | -95.7 | -95.7 | 0.0 |  | FDD |
| 28 |  |  | -96.2 | -96.2 | -96.2 | -96.2 | FDD |
| 31 | -96.5 | -93.2 | -91.2 |  |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 39 |  |  | -97.7 | -97.7 | -97.7 | -97.7 | TDD |
| 40 |  |  | -97.7 | -97.7 | -97.7 | -97.7 | TDD |
| 41 |  |  | -95.7 | -95.7 | -95.7 | -95.7 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 |  |  |  |  |  |  |  |
| NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 |  |  |  |  |  |  |  |
| NOTE 3: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. |  |  |  |  |  |  |  |
| NOTE 4: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. |  |  |  |  |  |  |  |
| NOTE 5: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |

# Conclusion

In this paper, Sensitivity numbers for CAT-M2 have been proposed for TDD and FDD bands.

# References

[1] R4-1706105, CR for CAT-M2 REFSENS, MPR and adding note for protection band 5, 3GPP TSG-RAN WG1 Meeting #83, Hangzhou, China, 15-19 May 2017