3GPP TSG-RAN WG4 Meeting #84	R4-1707831

Berlin, Germany, 21 - 25 August, 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 4566 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 4Rx spec correction CR for 36.101 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_4Rx_AP_DL_bands-Core |  |  |  |  |  | Date: |  |  | 2017-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CR R4-1706077 was approved in RAN4#83, the new 4Rx REFSENS approach improved the spec a lot. However, the parrallel CA big CRs didn’t use the new approache whick make the spec unclear. This CR updated the new added band combinations’ 4Rx REFSENS wording. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 4Rx+CA requirements spec is updated using the improved approach in R4-1706077. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some of the 4Rx+CA REFSENS spec is redundant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# <Start of changes>

Table 7.3.1A-0a: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-5A-41A | 1 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 3 |  |  |  | N/A | N/A | N/A | N/A |  |
|  | 5 |  |  |  | N/A | N/A |  |  |  |
|  | 41 |  |  |  |  |  |  | N/A | TDD |
| CA_1A-3A-7A-8A4 | 1 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 3 |  |  |  | N/A | N/A | N/A | N/A |  |
|  | 7 |  |  |  | N/A | N/A | N/A | N/A |  |
|  | 8 |  |  |  | N/A | N/A |  |  |  |
| CA_1A-3A-7A-8A4,5,6 | 1 |  |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 733 |  |  |  | -88 | -87.4 | -87 | -86.7 |  |
|  | 8 |  |  |  | -96.8 | -93.8 |  |  |  |
| CA_1A-3A-7A-20A-28A5,6 |  | 1 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  |  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  |  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  |  | 20 |  |  |  | -93.8 | -92 | -90.8 |  |
|  |  | 28 |  |  | -98,3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-7A-20A-42A9,10 | 1 |  |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 7 |  |  |  |  | -94.8 | -93 | -91.8 |  |
|  | 20 |  |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-7A-20A-42A11 | 1 |  |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 7 |  |  |  |  | -94.8 | -93 | -91.8 |  |
|  | 20 |  |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 4233 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-7A-28A5,6 | 133 |  |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 3 |  |  |  |  | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  |  | -95 | -93.2 | -92 |  |
|  | 28 |  |  |  |  | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-7A-42A9,10 | 1 |  |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 7 |  |  |  |  | -94.8 | -93 | -91.8 |  |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-7A-42A11 | 1 |  |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 7 |  |  |  |  | -94.8 | -93 | -91.8 |  |
|  | 4233 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-8A4 | 1 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 3 |  |  |  | N/A | N/A | N/A | N/A |  |
|  | 8 |  |  | N/A | N/A | N/A |  |  |  |
| CA_1A-3A-8A-11A4 | 1 |  |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  |  | N/A | N/A | N/A | N/A |  |
|  | 8 |  |  |  | -97 | -94 |  |  |  |
|  | 11 |  |  |  | -100 | -97 |  |  |  |
| CA_1A-3A-8A-40A4 | 1 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 3 |  |  |  | N/A | N/A | N/A | N/A |  |
|  | 8 |  |  | N/A | N/A | N/A |  |  |  |
|  | 40 |  |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_1A-3A-19A-42A9,10 | 1 |  |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 19 |  |  |  | -100 | -97 | -95.2 |  |  |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-19A-42A11 | 1 |  |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 19 |  |  |  | -100 | -97 | -95.2 |  |  |
|  | 42 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-20A-28A | 1 |  |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 3 |  |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 20 |  |  |  |  | -93.8 | -92 | -90.8 |  |
|  | 28 |  |  |  | -98,3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-21A-28A4,21 | 1 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 3 |  |  |  | N/A | N/A | N/A | N/A |  |
|  | 21 |  |  |  | N/A | N/A | N/A |  |  |
|  | 28 |  |  |  | N/A | N/A |  |  |  |
| CA_1A-3A-21A-42A22,23 | 1 |  |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  |  | -96.7 | -93.7 | -91.9 | -90.7 |  |
|  | 21 |  |  |  | -99.5 | -96.5 | -94.7 |  |  |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-21A-42A24 | 1 |  |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  |  | -96.7 | -93.7 | -91.9 | -90.7 |  |
|  | 21 |  |  |  | -99.5 | -96.5 | -94.7 |  |  |
|  | 4233 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-28A-42A22,23 | 1 |  |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 28 |  |  |  | -98.3 | -95.3 |  |  |  |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-28A-42A24 | 1 |  |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 28 |  |  |  | -98.3 | -95.3 |  |  |  |
|  | 4233 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-28A | 133 |  |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 3 |  |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 28 |  |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-1A-3A-28A | 1 |  |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 3 |  |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 28 |  |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-42A9,10 | 1 |  |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-42A11 | 1 |  |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 4233 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-5A-41A8 | 1 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 5 |  |  |  | N/A | N/A |  |  |  |
|  | 41 |  |  |  |  |  |  | N/A | TDD |
| CA_1A-7A-8A5,6 | 1 |  |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 733 |  |  |  |  | -87.4 | -87 | -86.7 |  |
|  | 8 |  |  |  | -96.8 | -93.8 |  |  |  |
| CA_1A-7A-20A-28A | 1 |  |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 7 |  |  |  |  | -95 | -93.2 | -92 |  |
|  | 20 |  |  |  |  | -93.8 | -91 | -89.8 |  |
|  | 28 |  |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-7A-28A5,6 | 133 |  |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 7 |  |  |  |  | -95 | -93.2 | -92 |  |
|  | 28 |  |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-11A-28A5,6 | 133 |  |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 11 |  |  |  | -100 | -97 |  |  |  |
|  | 28 |  |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-11A-28A9,10 | 1 |  |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 11 |  |  |  | -75.2 | -75.2 |  |  |  |
|  | 28 |  |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-18A-28A14 | 1 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 18 |  |  |  | N/A | N/A | N/A |  |  |
|  | 28 |  |  |  | N/A | N/A |  |  |  |
| CA_1A-19A-28A14 | 1 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 19 |  |  |  | N/A | N/A | N/A |  |  |
|  | 28 |  |  |  | N/A | N/A |  |  |  |
| CA_1A-20A-28A | 1 |  |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 20 |  |  |  |  | -93.8 | -91 | -89.8 |  |
|  | 28 |  |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-21A-28A4,21 | 1 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 21 |  |  |  | N/A | N/A | N/A |  |  |
|  | 28 |  |  |  | N/A | N/A |  |  |  |
| CA_1A-21A-28A-42A4,21 | 1 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 21 |  |  |  | N/A | N/A | N/A |  |  |
|  | 28 |  |  |  | N/A | N/A |  |  |  |
|  | 42 |  |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_1A-28A5,6,14 | 133 |  |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 28 |  |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-1A-28A5,6,14 | 1 |  |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 28 |  |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-28A-42A5,6,17,18 | 133 |  |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 28 |  |  |  | -98.3 | -95.3 |  |  |  |
|  | 4233 |  |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_2A-46A15,16 | 2 |  |  |  | -70 | -67 | -65.2 | -64 | FDD |
|  | 46 |  |  |  |  |  |  | -90 | TDD |
| CA_2A-48A9,10 | 2 |  |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 48 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A11 | 2 |  |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 48 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-4A-12A5,6 | 2 |  |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 4 |  |  |  | -90 | -89.5 | -89 | -88.5 |  |
|  | 12 |  |  |  | -96.5 | -93.5 |  |  |  |
| CA_2A-4A-5A-12A5,6 | 2 |  |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 4 |  |  |  | -90 | -89.5 | -89 | -88.5 |  |
|  | 5 |  |  |  | -97.5 | -94.5 |  |  |  |
|  | 12 |  |  |  | -96.5 | -93.5 |  |  |  |
| CA_2A-4A-7A-12A5,6 | 2 |  |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 4 |  |  |  | -90 | -89.5 | -89 | -88.5 |  |
|  | 7 |  |  |  | -97.5 | -94.5 | -92.7 | -91.5 |  |
|  | 12 |  |  |  | -96.5 | -93.5 |  |  |  |
| CA_2A-4A-12A-30A5,6 | 2 |  |  |  | -97.6 | -94.6 | -92.8 | -91.6 | FDD |
|  | 4 |  |  |  | -90 | -89.5 | -89 | -88.5 |  |
|  | 12 |  |  |  | -96.5 | -93.5 |  |  |  |
|  | 30 |  |  |  | -98.5 | -95.5 |  |  |  |
| CA_2A-5A-12A-66A 5,6 | 2 |  |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 5 |  |  |  | -97.5 | -94.5 |  |  |  |
|  | 12 |  |  |  | -96.5 | -93.5 |  |  |  |
|  | 66 |  |  |  | -89.5 | -89 | -88.5 | -88 |  |
| CA_2A-12A-66A5,6 | 2 |  |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 12 |  |  |  | -96.5 | -93.5 |  |  |  |
|  | 66 |  |  |  | -89.5 | -89 | -88.5 | -88 |  |
| CA_2A-48A-48A10,31 | 2 |  |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 48 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A-48A11 | 2 |  |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 48 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48C9,10 | 2 |  |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 48 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48C11 | 2 |  |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 48 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48A-66A10,32 | 2 |  |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 48 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| CA_2A-48A-66A11 | 2 |  |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 48 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| CA_3A-5A-41A | 3 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 5 |  |  |  | N/A | N/A |  |  |  |
|  | 41 |  |  |  |  |  |  | N/A | TDD |
| CA_3A-7A-8A4 | 3 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 7 |  |  |  | N/A | N/A | N/A | N/A |  |
|  | 8 |  |  |  | N/A | N/A |  |  |  |
| CA_3A-7A-8A4,5,6 | 3 |  |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 733 |  |  |  | -88 | -87.4 | -87 | -86.7 |  |
|  | 8 |  |  |  | -96.8 | -93.8 |  |  |  |
| CA_3A-7A-20A-42A9,10 | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
|  | 7 |  |  |  | -97.8 | -94.8 | -93 | -91.8 |  |
|  | 20 |  |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-7A-20A-42A11 | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
|  | 7 |  |  |  | -97.8 | -94.8 | -93 | -91.8 |  |
|  | 20 |  |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 4233 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-7A-42A9,10 | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
|  | 7 |  |  |  | -97.8 | -94.8 | -93 | -91.8 |  |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-7A-42A11 | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
|  | 7 |  |  |  | -97.8 | -94.8 | -93 | -91.8 |  |
|  | 4233 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-8A4 | 3 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 8 |  |  | N/A | N/A | N/A |  |  |  |
| CA_3A-8A-11A4 | 3 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 8 |  |  |  | N/A | N/A |  |  |  |
|  | 11 |  |  |  | N/A | N/A |  |  |  |
| CA_3A-11A-28A9,10 | 3 |  |  |  | -96.7 | -93.7 | -91.9 | -90.7 | FDD |
|  | 11 |  |  |  | -75.2 | -75.2 |  |  |  |
|  | 28 |  |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_3A-19A-21A-42A25,26 | 3 |  |  |  | -96.7 | -93.7 | -91.9 | -90.7 | FDD |
|  | 19 |  |  |  | -100 | -97 | -95.2 |  |  |
|  | 21 |  |  |  | -99.5 | -96.5 | -94.7 |  |  |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-19A-21A-42A27 | 3 |  |  |  | -96.7 | -93.7 | -91.9 | -90.7 | FDD |
|  | 19 |  |  |  | -100 | -97 | -95.2 |  |  |
|  | 21 |  |  |  | -99.5 | -96.5 | -94.7 |  |  |
|  | 4233 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-19A-42A9,10 | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
|  | 19 |  |  |  | -100 | -97 | -95.2 |  |  |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-19A-42A11 | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
|  | 19 |  |  |  | -100 | -97 | -95.2 |  |  |
|  | 4233 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-20A-42A9,10 | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
|  | 20 |  |  |  | -97 | -94 | -91.2 | -90 |  |
|  |  |  |  |  | -99.711 | -96.711 | -93.911 | -92.711 |  |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-20A-42A11 | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
|  | 20 |  |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 4233 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-21A-28A4,21 | 3 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 21 |  |  |  | N/A | N/A | N/A |  |  |
|  | 28 |  |  |  | N/A | N/A |  |  |  |
| CA_3A-21A-42A1,2 | 3 |  |  |  | -96.7 | -93.7 | -91.9 | -90.7 | FDD |
|  | 21 |  |  |  | -99.5 | -96.5 | -94.7 |  |  |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-21A-42A3 | 3 |  |  |  | -96.7 | -93.7 | -91.9 | -90.7 | FDD |
|  | 21 |  |  |  | -99.5 | -96.5 | -94.7 |  |  |
|  | 4233 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-40A15,16 | 3 |  |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 28 |  |  |  | -60.7 | -60.7 | -60.7 | -60.7 |  |
|  | 40 |  |  |  | -100 | -97 | -95.2 | -94 | TDD |
| CA_3A-28A-41A-42A9,10,29 | 3 |  |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
|  | 28 |  |  |  | -98.3 | -95.3 |  |  |  |
|  | 41 |  |  |  |  | -94.5 | -92.7 | -91.5 | TDD |
|  | 4233 |  |  |  |  | -71.7 | -71.7 | -71.7 |  |
| CA_3A-28A-41A-42A11,29 | 3 |  |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
|  | 28 |  |  |  | -98.3 | -95.3 |  |  |  |
|  | 41 |  |  |  |  | -94.5 | -92.7 | -91.5 | TDD |
|  | 4233 |  |  |  |  | -94.7 | -93.2 | -92.5 |  |
| CA_3A-28A-41A-42A17,18, 29 | 3 |  |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
|  | 28 |  |  |  | -98.3 | -95.3 |  |  |  |
|  | 41 |  |  |  |  | -94.5 | -92.7 | -91.5 | TDD |
|  | 4233 |  |  |  |  | -85.4 | -85.1 | -84.9 |  |
| CA_3A-28A-42A9,10 | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |  |
|  | 28 |  |  |  | -98.3 | -95.3 |  |  |  |  |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |  |
| CA_3A-28A-42A11 | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |  |
|  | 28 |  |  |  | -98.3 | -95.3 |  |  |  |  |
|  | 4233 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |  |
| CA_3A-28A-42A17,18 | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |  |
|  | 28 |  |  |  | -98.3 | -95.3 |  |  |  |  |
|  | 4233 |  |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |  |
| CA_3A-31A12,13 | 333 |  |  |  | -86.9 | -86.4 | -86 | -85.6 | FDD |
|  | 31 |  |  | -95.5 | -93.3 |  |  |  |  |
| CA_3A-42A9,10 | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42A11 | 3 |  |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
|  | 4233 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_4A-5A-12A5,6 | 4 |  |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
|  | 5 |  |  |  | -97.5 | -94.5 |  |  |  |
|  | 12 |  |  |  | -96.5 | -93.5 |  |  |  |
| CA_4A-7A-12A5,6 | 4 |  |  |  | [-90] | [-89.5] | [-89] | [-88.5] | FDD |
|  | 7 |  |  |  | -97.5 | -94.5 |  |  |  |
|  | 12 |  |  |  | -96.5 | -93.5 |  |  |  |
| CA_4A-12A5,6 | 4 |  | -89.2 | -89.2 | -90 | -89.5 | -89 | -88.5 | FDD |
|  | 12 |  |  | -98.2 | -96.5 | -93.5 |  |  |  |
| CA_4A-12A-30A5,6 | 4 |  |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
|  | 12 |  |  |  | -96.5 | -93.5 |  |  |  |
|  | 30 |  |  |  | -98.5 | -95.5 |  |  |  |
| CA_4A-17A5,6 | 4 |  |  |  | -90 | -89.5 |  |  | FDD |
|  | 17 |  |  |  | -96.5 | -93.5 |  |  |  |
| CA_4A-28A5,6 | 433 |  |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 28 |  |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_5A-12A-66A5,6 | 5 |  |  |  | -97.5 | -94.5 |  |  | FDD |
|  | 12 |  |  |  | -96.5 | -93.5 |  |  |  |
|  | 66 |  |  |  | -90 | -89.5 | -89 | -88.5 |  |
| CA_5A-38A19 | 5 |  |  |  | N/A | N/A |  |  | FDD |
|  | 38 |  |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_5A-41A8 | 5 |  |  |  | N/A | N/A |  |  | FDD |
|  | 41 |  |  |  |  |  |  | N/A | TDD |
| CA_5A-40A-41A8 | 5 |  |  |  | N/A | N/A |  |  | FDD |
|  | 40 |  |  |  |  | N/A | N/A | N/A |  |
|  | 41 |  |  |  |  |  |  | N/A |  |
| CA_7A-8A5,6 | 733 |  |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
|  | 8 |  |  | -99 | -96.8 | -93.8 |  |  |  |
| CA_7A-8A-20A5,6 | 733 |  |  |  |  | -87.4 | -87 | -86.7 | FDD |
|  | 8 |  |  | -99 | -96.8 | -93.8 |  |  |  |
|  | 20 |  |  |  | [-96.8] | [-93.8] |  |  |  |
| CA_7A-20A-38A19 | 7 |  |  |  |  | N/A | N/A | N/A | FDD |
|  | 20 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 38 |  |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_8A-28A-41A28 | 8 |  |  |  | N/A | N/A |  |  | FDD |
|  | 28 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 41 |  |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_8A-41A8 | 8 |  | N/A | N/A | N/A | N/A |  |  | FDD |
|  | 41 |  |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_8A-42A12,13 | 8 |  | -102 | -99 | -96.8 | -93.8 |  |  | FDD |
|  | 4233 |  |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_11A-28A9,10 | 1133 |  |  |  | -75.2 | -75.2 |  |  | FDD |
|  | 28 |  |  |  | -98.3 | -95.3 | -93.5 | -90.8 | FDD |
| CA_12A-30A-66A5,6 | 12 |  |  |  | -96.5 | -93.5 |  |  | FDD |
|  | 30 |  |  |  | -98.5 | -95.5 |  |  |  |
|  | 66 |  |  |  | -89.5 | -89 | -88.5 | -88 |  |
| CA_12A-66A5,6 | 12 |  |  | -98.2 | -96.5 | -93.5 |  |  | FDD |
|  | 66 |  | -88.7 | -88.7 | -89.5 | -89 | -88.5 | -88 |  |
| CA_20A-38A19 | 20 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 38 |  |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_20A-40A15,16 | 2033 |  |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
|  | 40 |  |  |  | -100 | -97 | -95.2 | -94 | TDD |
| CA_20A-42A12,13, CA_20A-42A-42A12,13 | 20 |  |  |  | -97 | -94 | -91.2 | -90 | FDD |
|  | 4233 |  |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_21A-28A4,21 | 21 |  |  |  | N/A | N/A | N/A |  | FDD |
|  | 28 |  |  |  | N/A | N/A |  |  |  |
| CA_21A-28A-42A4,21 | 21 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 28 |  |  |  | N/A | N/A | N/A |  |  |
|  | 42 |  |  |  | N/A | N/A |  |  |  |
| CA_26A-41A8 | 26 |  |  |  | N/A | N/A | N/A |  | FDD |
|  | 41 |  |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_28A-40A15,16 | 28 |  |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
|  | 40 |  |  |  | -100 | -97 | -95.2 | -94 | TDD |
| CA_28A-41A-42A17,18 | 28 |  |  |  | -98.3 | -95.3 |  |  | FDD |
|  | 41 |  |  |  |  | -94.6 | -92.8 | -91.6 |  |
|  | 4233 |  |  |  |  | -85.4 | -85.1 | -84.9 | TDD |
| CA_28A-42A17,18 | 28 |  |  |  | -98.3 | -95.3 | -93.5 | -92.3 | FDD |
|  | 4233 |  |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_48A-66A9,10 | 48 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_48A-66A11 | 48 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_48A-48A-66A10,30 | 48 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| CA_48A-48A-66A11 | 48 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| CA_48C-66A10,30 | 48 |  |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| CA_48C-66A11 | 48 |  |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 7: Void. NOTE 8: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_1A-3A-42A, CA_2A-48A, CA_2A-48C, CA_3A-19A-42A, CA_1A-3A-19A-42A and CA_3A-28A-41A-42A. FHD = 0MHz for CA_11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 12: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 13: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 14: For the UE that supports CA_1A-18A-28A or CA_1A-19A-28A, no requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 15: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 16: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 17: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 18: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 19: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 20: Void NOTE 21: No requirements apply when there is at least one individual RE on band 28 uplink outside frequencies 728 – 738 MHz. The reference sensitivity is only verified when all configured RE’s are confined within frequencies 728 – 738 MHz (the requirements specified in clause 7.3.1 of [6] apply). NOTE 22: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_1A-3A-21A-42A. NOTE 23: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 24: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 25: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-19A-21A-42A. NOTE 26: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 27: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 28: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of Band 8 for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of Band 41. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 29: The B41 requirements are modified by -0.1dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 30: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_48A-48A-66A NOTE 31: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2A-48A-48A NOTE 32: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2A-48A-66A NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

# <Next change>

Table 7.3.1A-0bA: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for two bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 39 |  | [-95.7] | -94 | -91.5 | -90 | -89 |  |
| CA_1A-3A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 |  |
| CA_1A-1A-3A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
| CA_1A-1A-3A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-3A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 39 |  |  | -94 | -91.5 | -90 | -89 |  |
| CA_1A-3A-3A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3C4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 39 |  |  | -94 | -91.5 | -90 | -89 |  |
| CA_1A-3C5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-1A-3C4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
| CA_1A-1A-3C5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1C-3A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 39 |  |  | -94 | -91.5 | -90 | -89 |  |
| CA_1C-3A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_18A-28A6 | 18 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 28 |  |  | -94 | -92.5 |  |  |  |
| CA_19A-28A7 | 19 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 28 |  |  | -94 | -92 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: These requirements apply when the uplink is active in Band 18 and the downlink channels in Band 28 are confined within the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 18. NOTE 7: These requirements apply when the uplink is active in Band 19 and the downlink channels in Band 28 are allocated at the middle of the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 19. NOTE 8: Void NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

# <Next change>

Table 7.3.1A-0bC: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for three bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-5A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |
| CA_1A-3A-5A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |
| CA_1A-1A-3A-5A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |
| CA_1A-1A-3A-5A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |
| CA_1A-3C-5A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |
| CA_1A-3C-5A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |
| CA_1A-3A-7A9 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
| CA_1A-3A-7A10 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
| CA_1A-3A-7A-7A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 39,12 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 310 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
| CA_1A-3A-7C9 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  |  | -91.5 | -90 | -89 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
| CA_1A-3A-7C10 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  |  | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
| CA_1A-3C-7C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 39,12 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 310 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
| CA_1A-3A-8A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |
| CA_1A-3A-8A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |
| CA_1A-3A-3A-8A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 8 |  |  | -97 | -94 |  |  |  |
| CA_1A-3A-3A-8A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 8 |  |  | -97 | -94 |  |  |  |
| CA_1A-3C-8A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |
| CA_1A-3C-8A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |
| CA_1A-3A-11A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 11 |  |  | -100 | -97 |  |  |  |
| CA_1A-3A-11A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 11 |  |  | -100 | -97 |  |  |  |
| CA_1A-3A-19A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |
| CA_1A-3A-19A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |
| CA_1A-3A-20A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
| CA_1A-3A-20A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
| CA_1A-3A-21A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 21 |  |  | -100 | -97 | -95.2 |  |  |
| CA_1A-3A-21A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 21 |  |  | -100 | -97 | -95.2 |  |  |
| CA_1A-3A-26A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 26 |  |  | -97.57 | -94.57 |  |  |  |
| CA_1A-3A-26A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 26 |  |  | -97.57 | -94.57 |  |  |  |
| CA_1A-3A-28A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-28A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-1A-3A-28A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-1A-3A-28A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3C-28A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 312 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3C-28A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-42A4 | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 312 |  |  | -93.8 | -91.3 | -89.8 | -88.8 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-42A5 | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-42C4 | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 312 |  |  | -93.8 | -91.3 | -89.8 | -88.8 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-42C5 | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-18A-28A6 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 18 |  |  | -100 | -97 | -95.2 |  |  |
|  | 28 |  |  | -94 | -92.5 |  |  |  |
| CA_1A-19A-28A8 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |
|  | 28 |  |  | -94 | -92 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the Bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the Bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: These requirements apply when the uplink is active in Band 18 and the downlink channels in Band 28 are confined within the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 18. NOTE 7: 7 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 8: These requirements apply when the uplink is active in Band 19 and the downlink channels in Band 28 are allocated at the middle of the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 19. NOTE 9: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 7, the requirement applies regardless of channel bandwidth in Band 1. NOTE 10: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 7, the requirement applies regardless of channel bandwidth in Band 1. NOTE 11: Void NOTE 12: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

# <Next change>

Table 7.3.1A-0bD1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-5A-7A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34,9 |  |  |  | -91.5 | -90 | -89 |  |
|  | 35 |  |  |  | -94 | -92.2 | -91 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
| CA_1A-3A-5A-7A-7A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34,9 |  |  |  | -91.5 | -90 | -89 |  |
|  | 35 |  |  |  | -94 | -92.2 | -91 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
| CA_1A-3A-5A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34,9 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 40 |  |  |  | -91.9 | -90.4 | -89.4 | TDD |
| CA_1A-3A-5A-41A7,8 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 41 |  |  |  |  |  | [88.1] | TDD |
| CA_1A-3A-7A-7A-26A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34 |  |  |  | -91.5 | -90 | -89 |  |
|  | 35 |  |  |  | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 26 |  |  | -98 | -95 |  |  |  |
| CA_1A-3A-7A-8A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34,9 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 8 |  |  | -96.8 | -93.8 |  |  |  |
| CA_1A-3A-7A-20A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34,9 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
| CA_1A-3A-7A-26A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34 |  |  |  | -91.5 | -90 | -89 |  |
|  | 35 |  |  |  | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 26 |  |  | -98 | -95 |  |  |  |
| CA_1A-3A-7A-28A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34,9 |  |  |  | [-91.5] | [-90] | [-89] |  |
|  | 35 |  |  |  | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 28 |  |  |  | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-7C-28A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34,9 |  |  |  | [-91.5] | [-90] | [-89] |  |
|  | 35 |  |  |  | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 28 |  |  |  | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-7A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 |  |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 7 |  |  |  | -94.8 | -93 | -91.8 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-8A-11A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34,9 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 8 |  |  | -97 | -94 |  |  |  |
|  | 11 |  |  | -100 | -97 |  |  |  |
| CA_1A-3A-8A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34,9 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |
|  | 40 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |
| CA_1A-3A-19A-21A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 39 |  |  | -93.7 | -91.2 | -89.7 | -88.7 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |
| CA_1A-3A-19A-21A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -96.7 | -93.7 | -91.9 | -90.7 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |
| CA_1A-3A-7A-40C4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |
|  | 40 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |
| CA_1A-3A-7A-40C5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |
|  | 40 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |
| CA_1A-3A-19A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 |  |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-19A-42C | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 |  |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-20A-28A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34 |  |  | -94,3 | -91.5 | -90 | -89 |  |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 20 |  |  |  | -93.8 | -91 | -89.8 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-20A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 |  |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-21A-28A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 39 |  |  | -93.7 | -91.2 | -89.7 | -88.7 |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |
| CA_1A-3A-21A-28A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -96.7 | -93.7 | -91.9 | -90.7 |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |
| CA_1A-3A-21A-42A4 | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 39 |  |  | -93.7 | -91.2 | -89.7 | -88.7 |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-21A-42A5 | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  | -96.7 | -93.7 | -91.9 | -90.7 |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-21A-42C | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 34,9 |  |  | -93.7 | -91.2 | -89.7 | -88.7 |  |
|  | 35 |  |  | -96.7 | -93.7 | -91.9 | -90.7 |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-28A-42A4 | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 39 |  |  | -93.8 | -91.3 | -89.8 | -88.8 |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-28A-42A5 | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-28A-42C | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 |  |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: Void NOTE 7: The Band 41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690MHz. NOTE 8: The Band 41 requirements also apply to the supported CA_1A-41A. NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

# <Next change>

Table 7.3.1A-0bD3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for five bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-7A-20A-28A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34 |  |  | -94,3 | -91.5 | -90 | -89 |  |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 20 |  |  |  | -93.8 | -91 | -89.8 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-7A-20A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD |
|  | 34,7 |  |  | -93.8 | -91.3 | -89.8 | -88.8 |  |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
|  | 7 |  |  |  | -94.8 | -93 | -91.8 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: Void NOTE 7: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

# <Next change>

# Table 7.3.1A-0bE: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |  |
| CA_1A-3A-5A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |  |
|  | 4019 |  |  |  | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3A-5A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-5A-41A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |  |
|  | 41 |  |  |  |  |  | [-88.1] | TDD |  |  |
| CA_1A-3A-7A-20A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 7 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |  |  |
|  | 7 |  |  |  | -94.8 | -93 | -91.8 |  |  |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |  |  |
|  | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
| CA_1A-3A-7A-20A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 42 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |  |  |
|  | 719 |  |  |  | -93 | -91.3 | -90.1 |  |  |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_1A-3A-7A-40A15 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 319 |  |  | -94 | -91.5 | -90 | -89 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-40A16 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD |  |  |
| CA_1A-3A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 7 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |  |
| CA_1A-3A-7A-40A | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |  |
| CA_1A-3A-7A-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -94.6 | -92.1 | -90.5 | -89.4 |  |  |  |
| CA_1A-3A-7A-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
|  |  |  |  |  |  |  |  | FDD | 7 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |  |
| CA_1A-3A-7A-40C | 1 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 7 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |  |
| CA_1A-3A-7A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 7 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |  |  |
|  | 7 |  |  |  | -94.8 | -93 | -91.8 |  |  |  |
|  | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 42 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |  |  |
|  | 719 |  |  |  | -93 | -91.3 | -90.1 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
| CA_1A-3A-8A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3A-8A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-38A12 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 319 |  |  | -94 | -91.5 | -90 | -89 |  |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 119 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |  |
|  | 319 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |  |
|  | 38 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-38A13 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 119 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |  |
|  | 319 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |  |
|  | 38 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 4019 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 4019 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |  |  |
| CA_1A-3A-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 4019 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3A-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 4019 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |  |  |
| CA_1A-3C-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 4019 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3C-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3C-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 4019 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |  |  |
| CA_1A-3A-41A12,14 | 319 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-41A13,14 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3C-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  | 4019 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD |  |
| CA_1A-3C-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_1A-3C-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  | 4019 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |  |
| CA_1A-5A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |  |
|  | 4019 |  |  |  | -91.9 | -90.4 | -89.4 | TDD |  |  |
| CA_1A-5A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-7A-20A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 7 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |  |  |
|  | 20 |  |  | -96.8 | -93.8 | -91 | -89.8 |  |  |  |
|  | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
| CA_1A-7A-20A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 42 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 |  |  |  |
|  | 20 |  |  | -96.8 | -93.8 | -91 | -89.8 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
| CA_1A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 7 |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |  |
| CA_1A-7A-40A | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |  |
| CA_1A-7A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 7 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |  |  |
|  | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 119 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 42 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
| CA_1A-8A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |  |  |
| CA_1A-8A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |  |  |
| CA_1A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |  |  |
| CA_1A-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-5A-40A | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 5 |  | -100.2 | -98 | -95 |  |  |  |  |  |
|  | 4019 |  |  |  | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-5A-40A | 319 |  | [-95.3] | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 5 |  | -100.2 | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-5A-41A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  | 5 |  |  | -98 | -95 |  |  |  |  |
|  | 41 |  |  |  |  |  | [-88.1] | TDD |  |
| CA_3A-5A-41A5,18 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 5 |  |  | -98 | -95 |  |  |  |  |
|  | 41 |  |  |  |  |  | -91.5 | TDD |  |
| CA_3A-7A-38A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] |  |  |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3A-7A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 7 |  |  | -97.7 | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD |  |  |
| CA_3A-7A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 7 |  |
|  | 7 |  |  | -97.7 | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |  |
| CA_3A-7A-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 719 |  |  | -96.8 | -94 | -92.4 | -91.2 |  |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |  |
| CA_3A-7A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 7 |  |  | -97.7 | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD |  |  |
| CA_3A-7A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 7 |  |
|  | 7 |  |  | -97.7 | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |  |
| CA_3A-7A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 719 |  |  | -96.8 | -94 | -92.4 | -91.2 |  |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |  |
| CA_3A-7A-42A | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD | 7 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |  |  |
|  | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 3 |  |  | [-96.6] | [-93.6] | [-91.8] | [-90.6] | FDD | 42 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
| CA_3A-8A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-8A-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-20A-32A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |  |  |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |  |  |
| CA_3A-28A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-28A-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-28A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 28 |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 4019 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |  |
| CA_3A-28A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-28A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 28 |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 4019 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |  |
| CA_3A-28A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-28A-40D | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 4019 |  |  |  | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-28A-40D | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 28 |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 4019 |  |  |  | -92.9 | -91.4 | -90.5 | TDD |  |  |
| CA_3A-28A-40D | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 |  |  |  |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-28A-41A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
| CA_3A-28A-41C5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
|  | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
| CA_3A-28A-41A-42A | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |
|  | 41 |  |  |  | -94.517 | -92.717 | -91.517 | TDD |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |
|  | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |
|  | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |
| CA_3A-28A-41A-42C | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |  |
|  | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
|  | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |  |
|  | 41 |  |  |  | -94.517 | -92.717 | -91.517 | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-28A-41C-42A | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |  |
|  | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
|  | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |  |
|  | 41 |  |  |  | -94.517 | -92.717 | -91.517 | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-40A | 3 | -101.7 | -98.7 | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-40A | 319 | [-97.4] | [-95.3] | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-40A-40A | 3 |  |  | -97 | -94 |  |  | FDD | 3 |  |
|  | 4019 |  |  |  | -92.9 |  | -90.2 | TDD |  |  |
| CA_3A-40A-40A | 319 |  |  | -94.2 | -91.2 |  |  | FDD | 40 |  |
|  | 40 |  |  |  | -97 |  | -94 | TDD |  |  |
| CA_3A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-40D | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |
| CA_3A-40D | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_3A-40E | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 4019 |  |  |  |  | -91.3 | -90.2 | TDD |  |  |
| CA_3A-40E | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 40 |  |  |  |  | -95.2 | -94 | TDD |  |  |
| CA_3A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3C-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3C-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3C-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3C-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-41A5 | 319 |  | [-95.3] | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3A-41C5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3A-41D5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3A-3A-41A | 3 |  | [-95.3] | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
|  | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
| CA_3C-41A5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3C-41C5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3C-41D5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
| CA_3A-41A-42A5,6,7,8 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 4219 |  |  | -71.7 | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41A-42A5,6,9 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 4219 |  |  | -97.1 | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41A-42A5,6,10 | 319 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-41A-42C5,6,7,8 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 4219 |  |  |  | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41A-42C5,6,9 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 4219 |  |  |  | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41A-42C5,6,10 | 319 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
|  | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-41C-42A5,6,7,8 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 4219 |  |  |  | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41C-42A5,6,9 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  |  | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41C-42A5,6,10 | 319 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
|  | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-41C-42C5,6,7,8 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
|  | 4219 |  |  | -71.7 | -71.7 | -71.7 | -71.7 |  |  |
| CA_3A-41C-42C5,6,9 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
|  | 4219 |  |  | -97.1 | -94.7 | -93.2 | -92.5 |  |  |
| CA_3A-41C-42C5,6,10 | 319 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |  |
| CA_7A-20A-42A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |  |  |
|  | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
| CA_7A-40A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD |  |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
|  | 40 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | TDD |  |  |
| CA_7A-40C | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD |  |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
|  | 40 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | TDD |  |  |
| CA_7A-42A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
| CA_7A-42A-42A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
| CA_28A-40A | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD | 28 |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |  |
| CA_28A-40A | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_28A-40C | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD | 28 |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |  |
| CA_28A-40C | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_28A-40D | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD | 28 |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 |  |  |  |
| CA_28A-40D | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply regardless of the channel bandwidth and the location of UL band. NOTE 5: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 6: The antenna isolation for MSD calculation is assumed as 10 dB. For conducted mode REFSENS test such antenna isolation is not observed as the antennas are disconnected. Additionally antenna isolation assumption is under discussion depending on the frequency range NOTE 7: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-42C, CA_1A-3A-42A, CA_1A-3A-42C, CA_3A-19A-42A, CA_3A-19A-42C and CA_1A-3A-19A-42A, CA 3A-41A-42A. NOTE 8: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 9: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 11: Void NOTE 12: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 13: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 14: The B41 requirements also apply to the supported CA_1A-41A. NOTE 15: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1 NOTE 16: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 17: The B41 requirements are modified by -0.1dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 18: No requirements apply when there is at least one individual RE within the downlink transmission bandwidth of band5 for which the 3rd transmitter harmonic is within the uplink transmission bandwidth of band41. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 19: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |

# <Next change>

Table 7.3.1A-0d: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-4A-5A-29A | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 4 |  |  | -99.7 | -96.7 | -94.9 | -93.7 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_2A-4A-29A | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 4 |  |  | -99.7 | -96.7 | -94.9 | -93.7 |  |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_2A-4A-29A-30A | 2 |  |  | -97.6 | -94.6 | -92.8 | -91.6 | FDD |
|  | 4 |  |  | -99.6 | -96.6 | -94.8 | -93.6 |  |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
| CA_2A-5A-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_2A-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  | -98.7 | -97 | -94 |  |  |  |
| CA_2A-2A-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_2C-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_2A-29A-30A | 2 |  |  | -97.6 | -94.6 | -92.8 | -91.6 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
| CA_2A-2A-29A-30A | 2 |  |  | -97.6 | -94.6 | -92.8 | -91.6 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
| CA_2C-29A-30A | 2 |  |  | -97.6 | -94.6 | -92.8 | -91.6 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
| CA_2A-29A-66A | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 66 |  |  | -99.2 | -96.2 | -94.4 | -93.2 |  |
| CA_4A-4A-29A | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_3A-7A-20A-32A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_3A-7A-32A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_3A-32A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_3A-69A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 69 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_4A-4A-29A-30A | 4 |  |  | -99.6 | -96.6 | -94.8 | -93.6 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
| CA_4A-5A-29A | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_4A-29A | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 29 |  | -98.7 | -97 | -94 |  |  |  |
| CA_4A-29A-30A | 4 |  |  | -99.6 | -96.6 | -94.8 | -93.6 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
| CA_5A-29A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_7A-20A-32A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_7A-32A | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_20A-32A | 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_20A-67A | 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
|  | 67 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_23A-29A | 23 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 29 |  | -98.7 | -97 | -94 |  |  |  |
| CA_29A-30A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 30 |  |  | -99 | -96 |  |  |  |
| CA_29A-30A-66A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
|  | 66 |  |  | -99.1 | -96.1 | -94.3 | -93.1 |  |
| CA_29A-66A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_29A-70A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 70 |  |  | -100 | -97 | -95.2 | -94 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port. NOTE 4: Void |  |  |  |  |  |  |  |  |

# <Next change>

For E-UTRA CA configurations with uplink and downlink assigned to two E-UTRA bands given in Table 7.3.1A-0f, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3.1A-0f. For E-UTRA CA configurations with uplink assigned to two E-UTRA bands and downlink assigned to three E-UTRA bands given in Table 7.3.1A-0g, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3.1A-0g. For these test points the reference sensitivity requirement specified in Table 7.3.1-1 and Table 7.3.1-1a are relaxed by the amount of the corresponding parameter MSD given in Table 7.3.1A-0f and Table 7.3.1A-0g.

The allowed exceptions defined in Table 7.3.1A-0a and Table 7.3.1A-0b for inter-band carrier aggregation with a single active uplink are also applicable for dual uplink operation.

Table 7.3.1A-0f: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_1A-3A | 1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | 3 | 1760 | 5 | 25 | 1855 | N/A |  | N/A |
| CA_1A-8A | 1 | 1965 | 5 | 25 | 2155 | 6 | FDD | IMD4 |
|  | 8 | 887.5 | 5 | 25 | 932.5 | N/A |  | N/A |
| CA_2A-4A | 2 | 1860 | 20 | 502 | 1940 | 5 | FDD | IMD3 |
|  | 4 | 1752.5 | 5 | 25 | 2152.5 | N/A |  | N/A |
| CA_2A-4A | 2 | 1868.3 | 5 | 25 | 1948.3 | N/A | FDD | N/A |
|  | 4 | 1735 | 5 | 25 | 2135 | 5 |  | IMD5 |
| CA_2A-46A | 2 | 1880 | 5 | 25 | 1960 | 12.0 | FDD | IMD34 |
|  | 46 | 5720 | 20 | 100 | 5720 | N/A |  | N/A |
| CA_2A-66A | 2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A |  | N/A |
| CA_2A-66A | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 |  | IMD5 |
| CA_3A-5A | 3 | 1771 | 10 | 50 | 1866 | 4 | FDD | IMD4 |
|  | 5 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-5A | 3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | 5 | 838 | 5 | 25 | 883 | 24 |  | IMD24 |
| CA_3A-7A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | 7 | 2535 | 10 | 50 | 2655 | 13 |  | IMD4 |
| CA_3A-8A | 3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 8 |  | IMD44 |
| CA_3A-8A | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | FDD | IMD5 |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A |  | N/A |
| CA_3A-19A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 |
|  | 19 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-19A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | 19 | 838 | 5 | 25 | 883 | 27 |  | IMD24 |
| CA_3A-20A | 3 | 1775 | 5 | 25 | 1870 | 4 | FDD | IMD4 |
|  | 20 | 840 | 5 | 25 | 799 | N/A |  | N/A |
| CA_3A-20A | 3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | 20 | 847 | 5 | 25 | 806 | 9 |  | IMD4 |
| CA_3A-26A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 |
|  | 26 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-26A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | 26 | 838 | 5 | 25 | 883 | 26 |  | IMD24 |
| CA_3A-41A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | TDD | N/A |
| CA_3A-42A | 3 | 1740 | 5 | 25 | 1835 | 29.8 | FDD | IMD24 |
|  | 42 | 3575 | 5 | 25 | 3575 | N/A | TDD | N/A |
| CA_3A-42A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  | 42 | 3435 | 5 | 25 | 3435 | N/A | TDD | N/A |
| CA_4A-5A | 4 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
|  | 5 | 838 | 5 | 25 | 883 | 26 |  | IMD24 |
| CA_4A-7A | 4 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | 7 | 2535 | 5 | 25 | 2655 | 15 |  | IMD4 |
| CA_5A-7A | 5 | 834 | 5 | 25 | 879 | 12 | FDD | IMD34 |
|  | 7 | 2547 | 10 | 50 | 2667 | N/A |  | N/A |
| CA_5A-66A | 5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A |  | N/A |
| CA_7A-20A | 7 | 2512 | 10 | 50 | 2632 | N/A | FDD | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 12 |  | IMD34 |
| CA_8A-41A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | FDD | IMD34 |
|  | 41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_19A-42A3 | 19 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | 42 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_21A-28A | 21 | 1450.4 | 5 | 25 | 1498.4 | [2.5] | FDD | IMD5 |
|  | 28 | 735.5 | 5 | 25 | 790.5 | N/A | TDD | N/A |
| CA_21A-42A3 | 21 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | 42 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_28A-42A | 28 | 705.5 | 5 | 25 | 760.5 | [5.5] | FDD | IMD5 |
|  | 42 | 3582.5 | 5 | 25 | 3582.5 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void NOTE 6: For operations with 4 antenna ports, the MSD in the applicable bands shall be modified by the absolute value of ΔRIB,4R in Table 7.3.1-1a when MSD > 0. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0g: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| CA_1A-3A-28A | CA_1A-28A | 1 | 1975 | 5 | 25 | 2165 | 5 | N/A | FDD | N/A |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  | N/A |
|  |  | 3 | 1723.5 | 5 | 25 | 1818.5 | 5 | 4.0 |  | IMD5 |
|  | CA_3A-28A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD | N/A |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  | N/A |
|  |  | 1 | 1949 | 5 | 25 | 2139 | 5 | 11.0 |  | IMD4 |
| CA_1A-3A-40A | CA_1A-3A | 1 | 1950 | 5 | 25 | 2140 | 5 | N/A | FDD | N/A |
|  |  | 3 | 1735 | 5 | 25 | 1830 | 5 | N/A | FDD | N/A |
|  |  | 40 | 2380 | 5 | 25 | 2380 | 5 | 8.0 | TDD | IMD5 |
| CA_1A-3A-42A | CA_1A-3A | 1 | 1922.5 | 5 | 25 | 2112.5 | 5 | N/A | FDD | N/A |
|  |  | 3 | 1782.5 | 5 | 25 | 1877.5 | 5 | N/A | FDD | N/A |
|  |  | 42 |  |  |  | 3425 | 5 | 13.0 | TDD | IMD4 |
| CA_1A-5A-7A | CA_1A-7A | 1 | 1968 | 5 | 25 | 2158 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2512 | 10 | 50 | 2632 | 10 | N/A |  | N/A |
|  |  | 5 | 835 | 5 | 25 | 880 | 5 | 1.0 |  | IMD5 |
| CA_1A-5A-40A | CA_1A-5A | 1 | 1977.5 | 5 | 25 | 2167.5 | 5 | N/A | FDD | N/A |
|  |  | 5 | 826.5 | 5 | 25 | 871.5 | 5 | N/A | FDD | N/A |
|  |  | 40 | 2305 | 10 | 50 | 2305 | 10 | 9.0 | TDD | IMD4 |
| CA_1A-28A-42A | CA_1A-28A | 1 | 1955 | 5 | 25 | 2145 | 5 | N/A | FDD | N/A |
|  |  | 28 | 735 | 5 | 25 | 790 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3425 | 5 | 25 | 3425 | 5 | 15.0 | TDD | IMD3 |
|  | CA_28A-42A | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3560 | 5 | 25 | 3560 | 5 | N/A | TDD | N/A |
|  |  | 1 | 1949 | 5 | 25 | 2139 | 5 | 11.0 | FDD | IMD3 |
| CA_2A-12A-30A | CA_2A-12A | 2 | 1885 | 5 | 25 | 1965 | 5 | N/A | FDD | N/A |
|  |  | 12 | 708.5 | 5 | 25 | 738.5 | 5 | N/A |  | N/A |
|  |  | 30 | 2308 | 5 | 25 | 2353 | 5 | 12.0 |  | IMD4 |
| CA_3A-5A-7A | CA_3A-5A | 3 | 1780 | 10 | 50 | 1875 | 10 | N/A | FDD | N/A |
|  |  | 5 | 845 | 5 | 25 | 890 | 5 | N/A |  | N/A |
|  |  | 7 | 2505 | 10 | 50 | 2625 | 10 | 30.0 |  | IMD21 |
|  | CA_3A-7A | 3 | 1725 | 10 | 50 | 1820 | 10 | N/A | FDD | N/A |
|  |  | 7 | 2565 | 10 | 50 | 2685 | 10 | N/A |  | N/A |
|  |  | 5 | 840 | 5 | 25 | 885 | 5 | 19.0 |  | IMD3 |
| CA_3A-7A-8A | CA_3A-7A | 3 | 1735 | 5 | 25 | 1830 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2530 | 10 | 50 | 2650 | 10 |  |  | N/A |
|  |  | 8 | 895 | 5 | 25 | 940 | 5 | 18.0 |  | IMD3 |
|  | CA_3A-8A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD | N/A |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 |  |  | N/A |
|  |  | 7 | 2550 | 10 | 50 | 2670 | 10 | 29.0 |  | IMD2+IMD34 |
| CA_3A-7A-20A | CA_3A-7A | 3 | 1737 | 5 | 25 | 1832 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2543 | 10 | 50 | 2663 | 10 | N/A |  | N/A |
|  |  | 20 | 847 | 10 | 20 | 806 | 10 | 10.5 |  | IMD2 |
|  | CA_3A-20A | 3 | 1775 | 10 | 50 | 1870 | 10 | N/A | FDD | N/A |
|  |  | 20 | 855 | 5 | 25 | 896 | 5 | N/A |  | N/A |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | 26.0 |  | IMD21 |
| CA_3A-7A-28A | CA_3A-7A | 3 | 1747 | 5 | 25 | 1842 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2543 | 5 | 25 | 2663 | 5 | N/A |  | N/A |
|  |  | 28 | 741 | 5 | 25 | 796.0 | 5 | 20.0 |  | IMD2 |
|  | CA_7A-28A | 7 | 2543 | 5 | 25 | 2663 | 5 | N/A | FDD | N/A |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  | N/A |
|  |  | 3 | 1737.5 | 5 | 25 | 1832.5 | 5 | 26.0 |  | IMD2 |
| CA_3A-19A-21A | CA_19A-21A | 19 | 832.5 | 5 | 25 | 877.5 | 5 | N/A | FDD | N/A |
|  |  | 21 | 1460.4 | 5 | 25 | 1508.4 | 5 | N/A |  | N/A |
|  |  | 3 | 1774.6 | 5 | 25 | 1869.6 | 5 | 4.0 |  | IMD5 |
| CA_3A-21A-28A | CA_3A-21A | 3 | 1782 | 5 | 25 | 1877 | 5 | N/A | FDD | N/A |
|  |  | 21 | 1451 | 5 | 25 | 1499 | 5 | N/A |  | N/A |
|  |  | 28 | 734 | 5 | 25 | 789 | 5 | 3.0 |  | IMD5 |
| CA_3A-41A-42A | CA_41A-42A | 41 | 2640 | 10 | 50 | 2640 | 10 | N/A | TDD | N/A |
|  |  | 42 | 3425 | 10 | 50 | 3425 | 10 |  | TDD | N/A |
|  |  | 3 | 1760 | 5 | 25 | 1855 | 5 | 16.0 | FDD | IMD3 |
| CA_19A-21A-42A | CA_19A-21A | 19 | 842.5 | 5 | 25 | 887.5 | 5 | N/A | FDD | N/A |
|  |  | 21 | 1450.4 | 5 | 25 | 1498.4 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3508.7 | 5 | 25 | 3508.7 | 5 | 13.0 | TDD | IMD4 |
|  | CA_21A-42A | 21 | 1460.4 | 5 | 25 | 1508.4 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3500 | 5 | 25 | 3500 | 5 | N/A | FDD | N/A |
|  |  | 19 | 836.2 | 5 | 25 | 881.2 | 5 | 13.0 | TDD | IMD4 |
| CA_28A-41A-42A | CA_41A-42A | 41 | 2672 | 10 | 50 | 2672 | 10 | N/A | TDD | N/A |
|  |  | 42 | 3460 | 10 | 50 | 3460 | 10 |  | TDD | N/A |
|  |  | 28 | 733 | 5 | 25 | 788 | 5 | 26.0 | FDD | IMD2 |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 3: Void NOTE 4: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 5: For operations with 4 antenna ports, the MSD in the applicable bands shall be modified by the absolute value of ΔRIB,4R in Table 7.3.1-1a when MSD > 0. |  |  |  |  |  |  |  |  |  |  |

# <Next change>

For intra-band non-contiguous carrier aggregation with one uplink carrier and two downlink sub-blocks, the throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.1-1, Table 7.3.1-1a, Table 7.3.1-1A, Table 7.3.1-1B, Table 7.3.1-1C and Table 7.3.1A-3 with the reference sensitivity power level increased by  RIBNC given in Table 7.3.1A-3 for the SCC(s). The requirements apply with all downlink carriers active. Unless given by Table 7.3.1-3, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.4-1) configured.

Table 7.3.1A-3: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_1A-1A | 25RB + 25RB | 0.0 < Wgap ≤ 50.0 | 251 | 0.5 | FDD |
|  | 25RB + 50RB | 0.0 < Wgap ≤ 45.0 | 251 | 0.5 |  |
|  | 25RB + 75RB | 0.0 < Wgap ≤ 40.0 | 251 | 0.5 |  |
|  | 25RB + 100RB | 0.0 < Wgap ≤ 35.0 | 251 | 0.5 |  |
|  | 50RB + 25RB | 0.0 < Wgap ≤ 45.0 | 501 | 0.5 |  |
|  | 50RB + 50RB | 0.0 < Wgap ≤ 40.0 | 501 | 0.5 |  |
|  | 50RB + 75RB | 0.0 < Wgap ≤ 35.0 | 501 | 0.5 |  |
|  | 50RB + 100RB | 0.0 < Wgap ≤ 30.0 | 501 | 0.5 |  |
|  | 75RB + 25RB | 0.0 < Wgap ≤ 40.0 | 751 | 0.5 |  |
|  | 75RB + 50RB | 0.0 < Wgap ≤ 35.0 | 751 | 0.5 |  |
|  | 75RB + 75RB | 0.0 < Wgap ≤ 30.0 | 751 | 0.5 |  |
|  | 75RB + 100RB | 0.0 < Wgap ≤ 25.0 | 751 | 0.5 |  |
|  | 100RB + 25RB | 20.0 < Wgap ≤ 35.0 | 901 | 0.5 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 1001 | 0.5 |  |
|  | 100RB + 50RB | 15.0 < Wgap ≤ 30.0 | 901 | 0.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 1001 | 0.5 |  |
|  | 100RB + 75RB | 10.0 < Wgap ≤ 25.0 | 901 | 0.5 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 1001 | 0.5 |  |
|  | 100RB + 100RB | 5.0 < Wgap ≤ 20.0 | 901 | 0.5 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 1001 | 0.5 |  |
| CA_2A-2A | 25RB+25RB | 30.0 < Wgap ≤ 50.0 | 121 | 5.3 | FDD |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 25RB+50RB | 25.0 < Wgap ≤ 45.0 | 121 | 4.4 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0 |  |
|  | 25RB+75RB | 20.0 < Wgap ≤ 40.0 | 121 | 4.2 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 251 | 0 |  |
|  | 25RB+100RB | 15.0 < Wgap ≤ 35.0 | 121 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 251 | 0 |  |
|  | 50RB+25RB | 15.0 < Wgap ≤ 45.0 | 121 | 5.9 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 50RB+50RB | 10.0 < Wgap ≤ 40.0 | 121 | 4.6 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 50RB+75RB | 5.0 < Wgap ≤ 35.0 | 121 | 4.1 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 50RB+100RB | 0.0 < Wgap ≤ 30.0 | 121 | 4.0 |  |
|  | 75RB+25RB | 10.0 < Wgap ≤ 40.0 | 1212 | 6.7 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 361 | 0 |  |
|  | 75RB+50RB | 5.0 < Wgap ≤ 35.0 | 1212 | 5.4 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 361 | 0 |  |
|  | 75RB+75RB | 0.0 < Wgap ≤ 30.0 | 1212 | 4.6 |  |
|  | 75RB+100RB | 0.0 < Wgap ≤ 25.0 | 1212 | 4.2 |  |
|  | 100RB+25RB | 0.0 < Wgap ≤ 35.0 | 1613 | 7.2 |  |
|  | 100RB+50RB | 0.0 < Wgap ≤ 30.0 | 1613 | 5.8 |  |
|  | 100RB+75RB | 0.0 < Wgap ≤ 25.0 | 1613 | 5.0 |  |
|  | 100RB+100RB | 0.0 < Wgap ≤ 20.0 | 1613 | 4.6 |  |
| CA_3A-3A | 15RB+25RB | 47.0 < Wgap ≤ 67.0 | 121 | 4.5 | FDD |
|  |  | 0.0 < Wgap ≤ 47.0 | 151 | 0 |  |
|  | 25RB+15RB | 47.0 < Wgap ≤ 67.0 | 121 | 4.9 |  |
|  |  | 0.0 < Wgap ≤ 47.0 | 251 | 0 |  |
|  | 25RB+25RB | 45.0 < Wgap ≤ 65.0 | 121 | 4.7 |  |
|  |  | 0.0 < Wgap ≤ 45.0 | 251 | 0 |  |
|  | 25RB+50RB | 40.0 < Wgap ≤ 60.0 | 121 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 40.0 | 251 | 0 |  |
|  | 25RB+75RB | 35.0 < Wgap ≤ 55.0 | 121 | 3.6 |  |
|  |  | 0.0 < Wgap ≤ 35.0 | 251 | 0 |  |
|  | 25RB+100RB | 30.0 < Wgap ≤ 50.0 | 121 | 3.4 |  |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 50RB+25RB | 30.0 < Wgap ≤ 60.0 | 129 | 5.1 |  |
|  |  | 0.0 < Wgap ≤ 30.0 | 321 | 0 |  |
|  | 50RB+50RB | 25.0 < Wgap ≤ 55.0 | 129 | 4.3 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 50RB+75RB | 20.0 < Wgap ≤ 50.0 | 129 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 50RB+100RB | 15.0 < Wgap ≤ 45.0 | 129 | 3.4 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 75RB+25RB | 25.0 < Wgap ≤ 55.0 | 1210 | 6.0 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 75RB+50RB | 20.0 < Wgap ≤ 50.0 | 1210 | 4.7 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 75RB+75RB | 15.0 < Wgap ≤ 45.0 | 1210 | 4.2 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 75RB+100RB | 10.0 < Wgap ≤ 40.0 | 1210 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 100RB+25RB | 15.0 < Wgap ≤ 50.0 | 1611 | 6.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 100RB+50RB | 10.0 < Wgap ≤ 45.0 | 1611 | 5.1 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 100RB+75RB | 5.0 < Wgap ≤ 40.0 | 1611 | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 100RB+100RB | 0.0 < Wgap ≤ 35.0 | 1611 | 4.1 |  |
| CA_4A-4A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | FDD |
| CA_5A-5A | 25RB+25RB | NOTE 7 | 121 | 5.3 | FDD |
|  | 25RB+50RB | NOTE 7 | 121 | 4.4 |  |
|  | 50RB+25RB | NOTE 7 | 121 | 5.9 |  |
|  | 50RB+50RB | NOTE 7 | 121 | 4.6 |  |
| CA_7A-7A | 25RB+25RB | 0< Wgap ≤ 60 | 25 | 0.0 | FDD |
|  | 25RB+50RB | 0 < Wgap ≤ 55 | 25 | 0.0 |  |
|  | 25RB+75RB | 0 < Wgap ≤ 50 | 25 | 0.0 |  |
|  | 25RB+100RB | 0 < Wgap ≤ 45 | 25 | 0.0 |  |
|  | 50RB+25RB | 30 < Wgap ≤ 55 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 30 | 50 | 0.0 |  |
|  | 50RB+50RB | 25.0 < Wgap ≤ 50.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 50 | 0.0 |  |
|  | 50RB+75RB | 20 < Wgap ≤ 45 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 20 | 50 | 0.0 |  |
|  | 50RB+100RB | 15 < Wgap ≤ 40 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 15 | 50 | 0.0 |  |
|  | 75RB+25RB | 20.0 < Wgap ≤ 50.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 501 | 0.0 |  |
|  | 75RB+50RB | 20.0 < Wgap ≤ 45.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 501 | 0.0 |  |
|  | 75RB+75RB | 15.0 < Wgap ≤ 40.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 501 | 0.0 |  |
|  | 75RB+100RB | 10 < Wgap ≤ 35 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 10 | 501 | 0.0 |  |
|  | 100RB+25RB | 25 < Wgap ≤ 45 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 25 | 451 | 0.0 |  |
|  | 100RB+50RB | 20 < Wgap ≤ 40 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 20 | 451 | 0.0 |  |
|  | 100RB+75RB | 15.0 < Wgap ≤ 35.0 | 361 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 501 | 0.0 |  |
|  | 100RB+100RB | 15.0 < Wgap ≤ 30.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 451 | 0.0 |  |
| CA_12A-12A | 25RB+25RB | 0.0 < Wgap ≤ 7.0 | 518 | 3 | FDD |
| CA_23A-23A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | FDD |
| CA_25A-25A | 25RB+25RB | 30.0 < Wgap ≤ 55.0 | 101 | 5.0 | FDD |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0.0 |  |
|  | 25RB+50RB | 25.0 < Wgap ≤ 50.0 | 101 | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0.0 |  |
|  | 25RB+75RB | 20 < Wgap ≤ 45 | 101 | 4.3 |  |
|  |  | 0 < Wgap ≤ 20 | 251 | 0 |  |
|  | 25RB+100RB | 15 < Wgap ≤ 40 | 101 | 4.1 |  |
|  |  | 0 < Wgap ≤ 15 | 251 | 0 |  |
|  | 50RB+25RB | 15.0 < Wgap ≤ 50.0 | 104 | 5.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0.0 |  |
|  | 50RB+50RB | 10.0 < Wgap ≤ 45.0 | 104 | 5.0 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0.0 |  |
|  | 50RB+75RB | 5 < Wgap ≤ 40 | 104 | 4.5 |  |
|  |  | 0 < Wgap ≤ 5 | 321 | 0 |  |
|  | 50RB+100RB | 0 < Wgap ≤ 35 | 104 | 4.2 |  |
|  | 75RB+25RB | 10 < Wgap ≤ 45 | 1014 | 7.6 |  |
|  |  | 0 < Wgap ≤ 10 | 321 | 0 |  |
|  | 75RB+50RB | 5 < Wgap ≤ 40 | 1014 | 6.7 |  |
|  |  | 0 < Wgap ≤ 5 | 321 | 0 |  |
|  | 75RB+75RB | 0 < Wgap ≤ 35 | 1014 | 5.6 |  |
|  | 75RB+100RB | 0 < Wgap ≤ 30 | 1014 | 4.8 |  |
|  | 100RB+25RB | 0 < Wgap ≤ 40 | 1215 | 8 |  |
|  | 100RB+50RB | 0 < Wgap ≤ 35 | 1215 | 6.7 |  |
|  | 100RB+75RB | 0 < Wgap ≤ 30 | 1215 | 6.1 |  |
|  | 100RB+100RB | 0 < Wgap ≤ 25 | 1215 | 5.7 |  |
| CA_40A-40A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_40A-40C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_40C-40C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41A-41A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41A-41C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41A-41D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41C-41C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_42A-42A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_42A-42C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_42A-42D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_42C-42C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48E | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48C-48C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48C-48D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_66A-66A | NOTE 6 | NOTE 7 | NOTE 8, NOTE 16 | 0.0 | FDD |
| CA_66A-66B | NOTE 6 | NOTE 7 | NOTE 8, NOTE 16 | 0.0 | FDD |
| CA_66A-66C | NOTE 6 | NOTE 7 | NOTE 8, NOTE 16 | 0.0 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The carrier center frequency of PCC in the UL operating band is configured closer to the DL operating band. NOTE 4: 4 refers to the UL resource blocks shall be located at RBstart=33. NOTE 5: For the TDD intra-band non-contiguous CA configurations, the minimum requirements apply only in synchronized operation between all component carriers. NOTE 6: All combinations of channel bandwidths defined in Table 5.6A.1-3. NOTE 7: All applicable sub-block gap sizes. NOTE 8: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. In case of uplink sub-block is TDD intra-band contiguous CA then the uplink PCC and SCC allocations are the same as NRB_agg defined in Table 7.3.1A-1. NOTE 9: 9 refers to the UL resource blocks shall be located at RBstart=25. NOTE 10: 10 refers to the UL resource blocks shall be located at RBstart=35. NOTE 11: 11 refers to the UL resource blocks shall be located at RBstart=50. NOTE 12: 12 refers to the UL resource blocks shall be located at RBstart=39. NOTE 13: 13 refers to the UL resource blocks shall be located at RBstart=57. NOTE 14: 14 refers to the UL resource blocks shall be located at RBstart=44. NOTE 15: 15 refers to the UL resource blocks shall be located at RBstart=62. NOTE 16: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 17: Void NOTE 18: 18 refers to the UL resource blocks shall be located at RBstart=12. NOTE 19: For operations with 4 antenna ports, the MSD in the applicable bands shall be modified by the absolute value of ΔRIB,4R in Table 7.3.1-1a when MSD > 0. |  |  |  |  |  |

# <End of changes>