3GPP TR 38.813 V0.0.1 (2017-08)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

New frequency range for NR (3.3-4.2 GHz)

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Radio, LTE

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Background	6

5	NR Frequency band definition	6

6	Channel numbering and channel bandwidth	6

7	NR band 3.3GHz - 3.8GHz specific issues	6

7.1	UE specific	6

7.1.1	Transmitter characteristics	6

7.1.1.1	UE maximum output power	6

7.1.1.2	UE maximum output power for modulation / channel bandwidth	6

7.1.1.3	UE maximum output power with additional requirements	6

7.1.1.4	Adjacent Channel Leakage Ratio (ACLR)	6

7.1.1.5	Spectrum emission mask	6

7.1.2	Receiver characteristics	6

7.1.2.1	Reference sensitivity	6

7.1.2.2	Adjacent Channel Selectivity (ACS)	6

7.1.2.3	Blocking	6

7.2	BS specific	6

7.2.1	Operating band unwanted emissions	7

7.2.2	Additional spurious emissions requirements	7

7.2.3	Co-location with other base stations	7

7.2.4	General blocking requirement	7

7.2.5	Blocking requirement for co-location with other base stations	7

8	NR band 3.3GHz - 4.2GHz specific issues	7

8.1	UE specific	7

8.1.1	Transmitter characteristics	7

8.1.1.1	UE maximum output power	7

8.1.1.2	UE maximum output power for modulation / channel bandwidth	7

8.1.1.3	UE maximum output power with additional requirements	7

8.1.1.4	Adjacent Channel Leakage Ratio (ACLR)	7

8.1.1.5	Spectrum emission mask	7

8.1.2	Receiver characteristics	7

8.1.2.1	Reference sensitivity	7

8.1.2.2	Adjacent Channel Selectivity (ACS)	7

8.1.2.3	Blocking	8

8.2	BS specific	8

8.2.1	Operating band unwanted emissions	8

8.2.2	Additional spurious emissions requirements	8

8.2.3	Co-location with other base stations	8

8.2.4	General blocking requirement	8

8.2.5	Blocking requirement for co-location with other base stations	8

9	Required changes to NR, E-UTRA, UTRA and MSR specifications	8

Annex <A> (normative): <Normative annex title>	9

Annex B: Change history		10

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

<Text to be added>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

4	Background

<Text to be added>

5	NR Frequency band definition

<Text to be added>

6	Channel numbering and channel bandwidth

<Text to be added>

7	NR band 3.3GHz - 3.8GHz specific issues

7.1	UE specific

<Text to be added>

7.1.1	Transmitter characteristics

7.1.1.1	UE maximum output power

<Text to be added>

7.1.1.2	UE maximum output power for modulation / channel bandwidth

<Text to be added>

7.1.1.3	UE maximum output power with additional requirements

<Text to be added>

7.1.1.4	Adjacent Channel Leakage Ratio (ACLR)

<Text to be added>

7.1.1.5	Spectrum emission mask

<Text to be added>

7.1.2	Receiver characteristics

7.1.2.1	Reference sensitivity

<Text to be added>

7.1.2.2	Adjacent Channel Selectivity (ACS)

<Text to be added>

7.1.2.3	Blocking

<Text to be added>

7.2	BS specific

<Text to be added>

7.2.1	Operating band unwanted emissions

<Text to be added>

7.2.2	Additional spurious emissions requirements

<Text to be added>

7.2.3	Co-location with other base stations

<Text to be added>

7.2.4	General blocking requirement

<Text to be added>

7.2.5	Blocking requirement for co-location with other base stations

<Text to be added>

8	NR band 3.3GHz - 4.2GHz specific issues

8.1	UE specific

<Text to be added>

8.1.1	Transmitter characteristics

8.1.1.1	UE maximum output power

<Text to be added>

8.1.1.2	UE maximum output power for modulation / channel bandwidth

<Text to be added>

8.1.1.3	UE maximum output power with additional requirements

<Text to be added>

8.1.1.4	Adjacent Channel Leakage Ratio (ACLR)

<Text to be added>

8.1.1.5	Spectrum emission mask

<Text to be added>

8.1.2	Receiver characteristics

8.1.2.1	Reference sensitivity

<Text to be added>

8.1.2.2	Adjacent Channel Selectivity (ACS)

<Text to be added>

8.1.2.3	Blocking

<Text to be added>

8.2	BS specific

<Text to be added>

8.2.1	Operating band unwanted emissions

<Text to be added>

8.2.2	Additional spurious emissions requirements

<Text to be added>

8.2.3	Co-location with other base stations

<Text to be added>

8.2.4	General blocking requirement

<Text to be added>

8.2.5	Blocking requirement for co-location with other base stations

<Text to be added>

9	Required changes to NR, E-UTRA, UTRA and MSR specifications

The required changes to the 3GPP specifications for the NR band 3.3-3.8GHz are summarised in a Table 9-1.

Table 9-1: Overview of 3GPP specifications with required changes

| Affected existing specifications |  |  |  |
| --- | --- | --- | --- |
| Spec No. | Subject of the CR | Comments | CR/TP (Tdoc) |

The required changes to the 3GPP specifications for the NR band 3.3-4.2GHz are summarised in a Table 9-2.

Table 9-2: Overview of 3GPP specifications with required changes

| Affected existing specifications |  |  |  |
| --- | --- | --- | --- |
| Spec No. | Subject of the CR | Comments | CR/TP (Tdoc) |

Annex <A> (normative):
<Normative annex title>

Annexes are only to be used where appropriate:

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 08/2017 | RAN4#84 | R4-1707861 |  |  | TR skeleton | N/A | 0.0.1 |