3GPP TSG-RAN WG4 Meeting NR #84	R4-1707933

Berlin, Germany, 21-25 August 2017

Source: 	NTT DOCOMO, INC.

Title: 	Link level evaluation results on NR-PSS/SSS detection

Agenda item:	9.6.6.2.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, evaluation assumptions for system level and link level simulations to discuss RRM requirements were agreed [1]. In this contribution, we provide our initial evaluation results regarding NR-PSS/SSS detection based on the agreed link level simulation assumption [2].

- Evaluation assumptions

Table 1 to 4 show the simulation assumptions for PSS/SSS detection. Based on agreed simulation assumption in [2], we add some corrections as proposed in [3] and select some specific parameters for initial evaluation as highlighted by red.

Table 1: Cell-specific Parameters for PSS/SSS Detection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| Carrier frequency |  | 4GHz; 30GHz | The same frequency as for Cell 1 is used |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 4GHz; 120 kHz; 240 kHz for 30GHz | The same values as for Cell 1 are used |
| RB Utilization | % | 100 | 100 |
| Power boosting for PSS and SSS | dB | 0 | 0 |
| Power boosting for Data and Control | dB | 0 | 0 |
| Data Modulation | - | QPSK | QPSK |
| CP Length | - | Normal | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 (baseline) FFS: 4 | 1(baseline) FFS: 4 |
| SS burst set periodicity | ms | 5 | 5 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | CP/2 |
| Relative Delay of 1st Path (asynchronous) | ms | 0 | 1.5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Eor/Ioc for K=1 | dB | 6.4 | 1.25; FFS: 0.25; -0.75 (SINR: -6dB; FFS: -8dB, -7dB) |
| Eor/Ioc for K=4 | dB | 2, 6,4, -2 | 2, 1, 1, 1 (SINR: -2.12, -5.97, -4.45, -1.12); -1, 5, -3, 2 dB (SINR: -5.12, -1.97, -8.45, -0.12) |
| Number of Tx antennas | - | 1 | 1 |
| PSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| SSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| Propagation Condition | - | AWGN, EPA5, ETU30 TDL-A/B/C/D with 300ns delay spread Note1 CDL-A/B/C/D with 300ns delay spread Note1 (secondary priority) |  |
| Ioc Model Note3 | - | AWGN noise |  |
| NOTE 1: The channel models of CDL and TDL are used for simplified link level evaluations, which are defined in TR 38.900. NOTE 2: Ioc value doesn’t include the three simulated BS signals’ power. |  |  |  |

Table 2: UE-specific Parameters for Cell Detection

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 and Cell 2 by the UE | Yes |
| False detect threshold | Required as in a real UE implementation |
| UE having apriority knowledge of system being synchronous or asynchronous | No |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence IDs. |
| CP length detection | Not simulated |
| Receive antennas | 2, 4 (uncorrelated. Option 1: With receiver beamforming Option 2: Without receiver beamforming) |

Table 3: Example Cell ID Combinations that can be simulated

| Case # | Cell 1 (Interferer Cell) |  | Cell 2 (Desired Cell) |  | Scenario |
| --- | --- | --- | --- | --- | --- |
| 1 | psc1 | ssc1 | psc2 | ssc2 | Synchronous |
| 2 | psc1 | ssc1 | psc1 | ssc3 | Synchronous |
| 3 | psc1 | ssc1 | psc2 | ssc2 | Asynchronous |
| 4 | psc1 | ssc1 | psc1 | ssc3 | Asynchronous |

Table 4: Example PSS, SSS indices that can be simulated simulations

| PSS Indices |  | SSS Indices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Label | Code index | Label | Code index (m0, m1) | Cell ID | (,) |
| psc1 | 43 | ssc1 | (20, 0) | 337 | (112, 1) |
| psc2 | 86 | ssc2 | (40, 0) | 674 | (224, 2) |
|  |  | ssc3 | (35,.0) | 673 | (224, 1) |

- Evaluation results

Initial evaluation results on PSS/SSS detection are shown in this section. Table 5 and Table 6 show the one-shot detection rate and the 90%-ile PSS/SSS acquisition time for the case of 15 kHz SCS on 4 GHz frequency carrier, respectively. We could observe that four cases with different cell ID combinations show almost the same cell detection performance. Therefore, we focus on one case i.e., Case 4 in following further evaluations.

Table 7 and Table 8 show the one-shot detection rate and the 90%-ile PSS/SSS acquisition time for Case 4 with different SCSs, respectively. It is observed that cell detection with larger SCS such as 120/240 kHz in 30 GHz carrier frequency needs more number of PSS/SSS samples to achieve reliable cell detection in fading channel due to short CP/symbol length compared with multipath delay.

Table 5. One-shot detection rate for 15 kHz SCS

| SINR | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AWGN | EPA5 | ETU30 | AWGN | EPA5 | ETU30 | AWGN | EPA5 | ETU30 | AWGN | EPA5 | ETU30 |
| -6 dB | 1.00 | 0.69 | 0.61 | 1.00 | 0.69 | 0.60 | 1.00 | 0.69 | 0.60 | 1.00 | 0.69 | 0.59 |
| -7 dB | 1.00 | 0.63 | 0.52 | 1.00 | 0.63 | 0.51 | 0.99 | 0.63 | 0.51 | 0.99 | 0.63 | 0.52 |
| -8 dB | 0.99 | 0.57 | 0.44 | 0.99 | 0.56 | 0.43 | 0.94 | 0.57 | 0.41 | 0.93 | 0.56 | 0.42 |

Table 6. The 90%-ile PSS/SSS acquisition time for 15 kHz SCS

| SINR | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AWGN | EPA5 | ETU30 | AWGN | EPA5 | ETU30 | AWGN | EPA5 | ETU30 | AWGN | EPA5 | ETU30 |
| -6 dB | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 |
| -7 dB | 1 | 3 | 4 | 1 | 3 | 4 | 1 | 3 | 4 | 1 | 3 | 4 |
| -8 dB | 1 | 3 | 4 | 1 | 3 | 5 | 1 | 3 | 5 | 1 | 3 | 5 |

Table 7. One-shot detection rate (Case 4, SINR = -6dB)

| SCS | AWGN | EPA5 | ETU30 |
| --- | --- | --- | --- |
| 15 kHz | 1.00 | 0.69 | 0.59 |
| 30 kHz | 1.00 | 0.69 | 0.54 |
| 120 kHz | 1.00 | 0.66 | 0.27 |
| 240 kHz | 1.00 | 0.45 | 0.18 |

Table 8. The 90%-ile PSS/SSS acquisition time (Case 4, SINR = -6dB)

| SCS | AWGN | EPA5 | ETU30 |
| --- | --- | --- | --- |
| 15 kHz | 1 | 2 | 3 |
| 30 kHz | 1 | 2 | 4 |
| 120 kHz | 1 | 3 | 8 |
| 240 kHz | 1 | 5 | 12 |

- Discussion

Based on evaluation results shown in previous section, we could make following observations.

- There is no large difference between Four cases with different cell ID combinations

- Cell detection with larger SCS such as 120/240 kHz in 30 GHz carrier frequency needs more number of PSS/SSS samples to achieve reliable cell detection in fading channel due to short CP/symbol length compared with multipath delay.

- Required number of samples for cell detection could be reduced compared with LTE case since NR-PSS/SSS is based on longer sequence mapped to larger number of subcarriers compared with LTE-PSS/SSS.

Regarding evaluation assumption, there would be no need to evaluate the case of K = 4 since in cell detection UE has no idea on which SS block is actually transmitted, i.e., PSS/SSS detection relies only on single SS block in every SS burst set.  In addition, delay scaling for CDL/TDL should consider typical scenario such as UMi e.g. 30 ns for 30 GHz.

- Conclusion

In this contribution, we showed our initial evaluation results regarding PSS/SSS detection delay based on the agreed link level simulation assumptions. Based on the evaluation results, we presented our views on the requirements for cell identification requirements.

- Reference

- 3GPP RAN4 #NR-AH2, Chairman’s note, June 2017.

- 3GPP, R4-1706903, Huawei, “Link level simulation of PSS/SSS detection in NR,” June 2017.

- 3GPP, R4-1708278, Huawei, HiSilicon, NTT DOCOMO, “Updated link level simulation assumption of cell detection in NR,” August 2017.