3GPP TSG-RAN WG4 Meeting #84 	R4-1708074

# Berlin, Germany, 21st-25th Aug 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 4575 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce test for sustained downlink data rate with active V2X Sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_V2X-Perf |  |  |  |  |  |  | Date: |  | 2017-08-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN4#83, RAN4 group agreed to introduce test to verify the concurrent WAN and V2X communication operation. Currently, there is no such test case in current specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test case to verify the concurrent WAN and V2X communication operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE performance are not verified when WAN and V2X communication operation are concurrent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------------Beginning of Change ----------------------------------------------------------

## 14.5 Maximum PSSCH processing capability and Sustained downlink data rate

The purpose of this test is to verify the maximum number of Sidelink processes and the maximum number of bits per TTI supported by the UE and further to verify the downlink data rate is not impacted when Sidelink resource are also configured.  The test parameters are in Table 14.5-1. Cell 1 is the serving cell and UE 1 is the transmitter of V2X sidelink Communication. The test UE is expected to receive all PSSCH and PDSCH transmissions simultaneously.

In each test, both PDSCH performance and PSSCH performance will be verified simultaneously. For PSSCH, the test cases apply to bandwidth as specified in Table 14.5-4. For PDSCH, the test cases apply to UE categories and bandwidth combinations with maximum aggregated bandwidth as specified in Table 14.5-2. The minimum requirements for PSSCH are specified in Table 14.5-4 and the minimum requirements for the downlink data rate are specified in Table 14.5-3. The TB success rate in the cellular link shall be sustained during at least 300 frames.

Table 14.5-1: Test parameters for sustained downlink data rate (FDD 64QAM) and maximum PSSCH processing capability

| Parameter |  | Unit |  |
| --- | --- | --- | --- |
| Communication resource pool configuration Note 5 |  |  | As specified in Table A.9-1 (Configuration #1-V2X) |
| Active cell(s) |  |  | Cell 1 (PCell) |
| Cell 1 | Test parameters |  | As specified in clause 8.7.1: Table 8.7.1-1 and Test 1, 2, 3A , 3, 4, 6 in Table 8.7.1-2 |
| Active Sidelink UE(s) |  |  | Sidelink UE 1 |
| Sidelink UE 1 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSSCH RB allocation |  | Fully allocated |
|  | Synchronization |  | GNSS or GNSS-equivalent |
|  | Time offset (NOTE 1) | s | 0 |
|  | Frequency offset (NOTE 2) | Hz | 0 |
|  | Propagation Channel |  | Static propagation condition |
|  | Antenna configuration |  | 1x2 Low |
|  | at antenna port | dBm/15kHz | -85 |
| NOTE 1: Time offset of Sidelink UE receive signal with respect to Cell 1 downlink timing at the tested UE. NOTE 2: Frequency offset of Sidelink UE with respect to Cell 1 uplink frequency. |  |  |  |

Table 14.5-2: Test cases for sustained data rate

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) | Cat. 1 | Cat. 2 | Cat. 3 | Cat. 4 | Cat. 6,7 | Cat. 9,10 | Cat 11, 12 | DL Cat. 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | DL Cat. 11,12 |  |
| Single carrier | 10 | 1 | 2 | 3A | 3A | 3A | 3A | 3A | 3A |
|  | 20 | - | - | 3 | 4 | 6 | 6 | 6 | 6 |

Table 14.5-3: Minimum requirements (FDD 64QAM) for PDSCH

| Test | Bandwidth (MHz) | Number of bits of a DL-SCH transport block received within a TTI | Measurement channel | Reference value |
| --- | --- | --- | --- | --- |
|  |  |  |  | PDSCH TB success rate (%) |
| 1 | 10 | 10296 | R.31-1 FDD (NOTE 2) | 95 |
| 2 | 10 | 25456 | R.31-2 FDD (NOTE 2) | 95 |
| 3 | 20 | 51024 | R.31-3 FDD | 95 |
| 3A | 10 | 36696 (NOTE 1) | R.31-3A FDD (NOTE 2) | 85 |
| 4 | 20 | 75376 (Note 2) | R.31-4 FDD | 85 |
| 6 | 20 | 75376 (Note 2) | R.31-4 FDD | 85 |
| NOTE 1: 35160 bits for sub-frame 5. NOTE 2: 71112 bits for sub-frame 5. |  |  |  |  |

Table 14.5-4: Minimum performances for PSSCH

| Test | Bandwidth | PSSCH Reference channel | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |
| 1 | 20 MHz | TBD | AWGN | 10 | TBD |
| 2 | 10 MHz | TBD | AWGN |  | TBD |

-----------------------------------------------------------End of Change ----------------------------------------------------------