3GPP TSG-RAN WG4 Meeting #84	R4-1708506

# Berlin, Germany, 21 – 25, August, 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 5144 | rev |  | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR PSCell Addition and Release Delay for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2017-08-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | NR PSCell addition and release delay requirements need to be introduced in 36.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section for NR PSCell addition and release delay requirements is added using LTE PSCell addition and release delay for DC section as baseline. Values to be agreed later. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 7.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 7.X	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity

### 7.X.1	Introduction

This section defines requirements for the delay within which the UE shall be able to configure an NR PSCell in E-UTRA – NR dual connectivity. The requirements are applicable to an E-UTRA – NR dual connectivity capable UE.

### 7.X.2	NR PSCell Addition Delay Requirement

The requirements in this section shall apply for the UE configured with only LTE PCell.

Upon receiving NR PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards NR PSCell no later than in subframe n+ Tconfig_ PSCell_EN-DC:

Where:

Tconfig_PSCell_EN-DC = [TBD] + Tactivation_time_EN-DC + [TBD] + TPCell_ DU_EN-DC + TPSCell_ DU_EN-DC

Tactivation_time_EN-DC is the NR PSCell activation delay. If the NR PSCell is known, then Tactivation_time is [TBD]. If the PSCell is unknown, then Tactivation_time_EN-DC is [TBD] provided the NR PSCell can be successfully detected on the first attempt.

TPCell_ DU_EN-DC is the delay uncertainty due to PCell PRACH preamble transmission. TPCell_ DU_EN-DC is up to 20ms if PSCell activation is interrupted by a PCell PRACH preamble transmission, otherwise it is 0.

TPSCell_ DU_EN-DC is the delay uncertainty in acquiring the first available PRACH occasion in the NR PSCell. TPSCell_ DU_EN-DC is up to [TBD].

NR PSCell is known if it has been meeting the following conditions:

During the last [5] seconds before the reception of the NR PSCell configuration command:

-	the UE has sent a valid measurement report for the NR PSCell being configured and

-	the PSCell being configured remains detectable according to the cell identification conditions specified in section 8.8,

-	PSCell being configured also remains detectable during the NR PSCell configuration delay according to the cell identification conditions specified in section 8.8.

otherwise it is unknown. The PCell interruption specified in section 7.12 is allowed only during the RRC reconfiguration procedure [2].

The NR PSCell addition delay specified in this section can be extended if SRS carrier based switching occurs during the NR PSCell addition procedure.

### 7.X.3	NR PSCell Release Delay Requirement

The requirements in this section shall apply for a UE configured with LTE PCell and one NR PSCell.

Upon receiving NR PSCell release in subframe n, the UE shall accomplish the release actions specified in [2] no later than in subframe n+20.

The PCell interruption specified in section 7.12 is allowed only during the RRC reconfiguration procedure [2].

The NR PSCell release delay specified in this section can be extended if SRS carrier based switching occurs during the NR PSCell release procedure.