3GPP TSG-RAN WG4 Meeting #84	R4-1708798

Berlin, Germany, 21 - 25 Aug, 2017

Agenda item:	8.2.2

Source:	Dish Network

Title:	TP for TR 36.715-02-01: MSD for CA_66A-70A

Document for:	Approval

# 1	Introduction

This contribution provides a TP to capture MSD for CA_66A-70A. Difference to previous contribution [1] is that potential support of also B2+B66 is considered.

# 2	Discussion

The remaining open issue for CA_66A-70A is MSD. ΔTIB,c and ΔRIB,c  were agreed during R14, but because this combination is moved to Rel15 we we just transfer the old agreements to current release.

The frequency arrangements are shown below.

B66: UL 1710-1780MHz, DL 2110-2200MHz

B70: UL 1695-1710MHz, DL 1995-2020MHz

B2: UL 1850-1910MHz, DL 1930-1990MHz

The uplinks for bands 66 and 70 are adjacent with no gap, thus a triplexer instead of a quadplexer is used to combine these bands.

B2 is just in the gap between B66/B70 UL and B70 DL hence not helping the situation at all.

### 2.1 CA_66A-70A only

This is the typical way of defining requirements for CA combinations in 3GPP.

We contacted several filter vendors to get triplexer data. The data is collected in tables below. Please note that some of the data is in typical conditions (TYP) and some is worst case (WC). The difference to our earlier contribution [1] is added worst case data from one of the filter vendors and including worst case data from Qualcomm paper [2].

Table 1 Isolation for CA_66A-70A

We note that two of the vendors provided information for both conditions and two provided information for worst case only. We have calculated averages for typical conditions and worst case conditions separately. These averages are not directly comparable with each other because they use different data mixes.

The worst case averages show all isolation values are at least 52.75dB. According to our analysis this should be sufficient together with the assumptions below to warrant no MSD is needed for B66, B70, and B66+B70 CA.

The following additional assumptions were used in our analysis:

RX IP2					50dB

Phase noise				-156dBc/Hz

B66/B70 FE TX IL	        5.3dB

B66 RX FE IL			  4.9dB

B70 RX FE IL			  4.0dB

PA noise at RX 			-128dBm/Hz

Conclusion 1: No MSD requirement is needed for B66, B70, and B66+B70 CA when UE supports CA_66A-70A only

### 2.2 CA_66A-70A and CA_2A-66A supported by the same UE

In this option, we consider the impacts when both of these CA combinations are supported by the same UE. We note that this is very untypical approach but because CA_2A-66A is typically supported in US UE’s the implications of supporting CA_66A-70A as well are considered.

We envision two high level implementation options.

a) Single CA filter covering B2, B66, and B70

b) B2+B66 quadplexer and B66+B70 triplexer in parallel

Pentaplexer implementation for option a is too challenging because there would be only 5MHz gap between B2 and B70 DL. Instead, a special quadplexer for option a is considered with the following characteristics. We note that also this is very challenging frequency arrangement.

Filter 1: B66+B70 UL

Filter 2: B2 UL

Filter 3: B2 and B70 DL

Filter 4: B66 DL

While this component might provide enough performance to aggregate these bands the concern is the impact to B2+B66 performance; at least performance defined in 3GPP specs should be maintained. In practice the performance comparison should be done against standalone B2+B66 quadplexers.

We got data from one filter vendor to verify the impact to B2+B66 CA in case B2+B66 quadplexer is modified to accommodate B70 as well. The isolations are above sufficient 3GPP limits, roughly 60dB or so for B2+B66+B70 quadplexer. In addition, the max IL in ETC for B2 and B66 is virtually the same or even better compared with same vendors B2+B66 product. Thus it can be concluded that B70 can be accommodated in B2+B66 quadplexer without degrading B2+B66 performance in CA using at least this particular component. Anyways, B2 Out-of-band blocking requirements might require reconsideration because B2 would attenuation from RF filter only 30MHz outside B2 upper edge.

Maybe the biggest drawback in this option is big impact to B70 RX IL compared to B66+B70 triplexer. According to filter manufacturer the ETC IL for B70 RX can be up to 2.4dB larger compared to B70 RX IL in triplexer. However, this increased IL exists only at the upper edge of B70 RX i.e at SCC region only. The MSD is 0dB even accounting increased B70 RX IL because the isolations in this component are good.

The additional assumptions were used in our analysis:

RX IP2					50dB

Phase noise				-156dBc/Hz

TX (x-band) isolation       61dB

RX (x-band) isolation	  62dB

B66/B70 FE TX IL	        4.7dB

B66 RX FE IL			  3.9dB

B70 RX FE IL			  6.3dB

PA noise at RX 			-128dBm/Hz

Option b employs duplicate B66 filters in main branch which has not been generally the practice in 3GPP. However, to make the evaluation we look at this option as well. Basically, we need to avoid situation that same band would need more than one LNA input. Thus some modifications into B66 paths are needed to route duplicate B66 filters into one LNA input.

The implication of having two filters for B66 is that one needs to have one extra PA switch throw on TX side and one SPDT switch before LNA on RX side to route signals from PA and to combine B66 RX into one LNA input. In some implementations, the SPDT could be considered as an additional one for the main branch.

Additional PA switch throw impact is negligible in terms of extra IL. On RX side, the SPDT before LNA has net impact of less than 0.5dB because in diversity path there is no need for such a switch. In general, there are some practical limits in the amount of switch throws and duplexers that can be accommodated in RF FE.

With these additional loss assumptions, option b does not change MSD analysis results in section 2.1.

Conclusion 2: If UE supports both CA_2A-66A and CA_66A-70A, at least one feasible implementation option is to have dedicated CA component for both combinations.

Conclusion 3: For that option, MSD, TIB, and RIB are the same as in case when UE supports only CA_66A-70A

Given that the baseline approach in 3GPP has been to define requirements for each CA combinations separately and due to negligible impact to results if support of other combinations were assumed, the requirements for CA_66A-70A should be defined based on B66+B70 triplexer only.

Conclusion 4: MSD for CA_66A-70A is derived based on B66+B70 triplexer, i.e. No MSD requirement is needed for B66, B70, and B66+B70 CA when UE supports CA_66A-70A

Please note that the same MSD conclusions hold for other higher class B66+B70 CA cases.

The TP below is mostly copied from R14 TR. Only the Reference Sensitivity section is new.

# 3	TP

## ----- Start of Text proposal -----

## 5.X	CA_66A-70A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_66-70 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  |

Table 5.X.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_66A-70A

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_66A-70A | 66 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  | 70 |  |  | Yes | Yes | Yes |  |  |  |

### 5.X.2	UE co-existence studies

UE harmonics are shown in table below.

Table 5.X.2-1 UE harmonics

| UE UL frequencies | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| Frequency [MHz] | 1710 | 1780 | 1695 | 1710 |
| 2nd harmonic [MHz] | 3420 | 3560 | 3390 | 3420 |
| 3rd harmonic [MHz] | 5130 | 5340 | 5085 | 5130 |
| 4th harmonic [MHz] | 6840 | 7120 | 6780 | 6840 |
| 5th harmonic [MHz] | 8550 | 8900 | 8475 | 8550 |

There are no harmonic issues for UE.

### 5.X.3	ΔTIB,c and ΔRIB,c values

Triplexer data is collected in table below.

Table 5.X.3-1 Insertion loss for CA_66A-70A

The average worst case B66 IL is 3.3dB for TX and 2.7dB for RX. The average worst case B70 IL is 3.3dB for TX and 2.1dB for RX. The difference to respective single band duplexer IL is <1dB for TX and <0.5dB for RX. Thus, the generic 3GPP framework values for can be used for both bands.

For the UE which supports CA_66A-70A, the ΔTIB,c and ΔRIB,c  are defined for applicable bands in Table 5.X.3-2 and 5.X.3-3 respectively.

Table 5.X.3-2: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_66A-70A | 66 | 0.5 |
|  | 70 | 0.5 |

Table 5.X.3-3: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_66A-70A | 66 | 0 |
|  | 70 | 0 |

### 5.X.4	REFSENS requirements

Isolation values are collected in table below.

Table 5.X.4-1 Isolation for CA_66A-70A

We note that one of the vendors provided information for both conditions, one provided isolation for typical only, and one provided information for worst case only. We have calculated averages for typical conditions and worst case conditions separately. These averages are not directly comparable with each other because they use different data mixes.

TX isolation needs to be above 50dB to avoid MSD for this band while 50dB RX isolation is sufficient. Both the typical and worst case averages show all isolation values are at least 52.75dB. According to our analysis this should be sufficient with the assumptions below to warrant no MSD is needed for B66, B70, and B66+B70 CA.

The following additional assumptions were used in our analysis:

RX IP2					50dB

Phase noise				-156dBc/Hz

B66/B70 FE TX IL	        5.3dB

B66 RX FE IL			  4.9dB

B70 RX FE IL			  4.0dB

PA noise at RX 			-128dBm/Hz

Conclusion: No MSD requirement is needed for B66, B70, and B66+B70 CA when UE supports CA_66A-70A

The impact of UE supporting both B2+B66 and B66+B70 was also evaluated.

It was found out that one option is to have parallel B2+B66 quadplexer and B66+B70 triplexer. The outcome of the study would not change, i.e no MSD is needed. UE RF front-end would be a more complex with B66 duplicated across two components.

Another option would be to modify B2+B66 quadplexer filters to accommodate B70 as well. In this case, the quadplexer would in practice support B2, B66, B70 and any inter-band CA of these. B2 out-of-band blocking capabilities in this option would be worse making it less attractive because the quadplexer would offer attenuation for B2 RX starting 30MHz away from B2 RX upper edge. One quadplexer using an advanced was found to have no impact to the outcome of the study i.e no MSD is needed while another quadplexer using a more conventional technology was found to be unfeasible due to excessive insertion losses.

----- End of Text proposal -----

# References

[1] R4-175241, “TP for TR 36.714-02-01: MSD for CA_66A-70A”, DISH Network, 3GPP  RAN4#83, Hangzhou, China, May 2017

[2] R4-1705680, “Filter considerations for CA_66-70”, Qualcomm Incorporated, 3GPP RAN4#83, Hangzhou, China, May2017