3GPP TSG-RAN WG4 Meeting #84	R4-1708495

Berlin, Germany, 21st – 25th August, 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	Draft CR to TS 37.105 – Subclauses 9.7.2 Occupied bandwidth and 9.7.3 ACLR

Agenda item:	8.28.2

Document for:	Approval

# 1	Introduction

In this document, we present a revised version of the draft text for Sections 9.7.2 OTA Occupied bandwidth and 9.7.3 OTA Adjacent Channel Leakage power Ratio of TS 37.105 [1] based on the comments captured in the RAN4 #84 AAS ad hoc agenda and minutes.

Further constructive feedback is encouraged.

# 2	Discussion

# 3	Conclusions

In this document, we have revised the draft text for Section 9.7.2 OTA occupied bandwidth and Section 9.7.3 OTA ACLR according to comments captured in the RAN4 #84 AAS ad hoc agenda and minutes.

# 4         References

- R4-1708495, Draft CR to TS 37.105 – Subclauses 9.7.2 Occupied bandwidth and 9.7.3 ACLR, Nokia, Nokia Shanghai Bell

# 5        Text proposal

TS 37.105

--------------Start of text proposal-------------

## 3.1	Definitions

{Unchanged definitions omitted}

directional requirement: requirement which is applied in to a specific direction inside a defined coverage range, which only applies to a single direction at a time.

{ Unchanged sections omitted}

# 9        Radiated transmitter characteristics

{ Unchanged sections omitted}

### 9.7.2	OTA occupied bandwidth

#### 9.7.2.1	General

The OTA occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. See also recommendation ITU-R SM.328 [17].

The value of /2 shall be taken as 0.5%.

The OTA occupied bandwidth requirement applies during the transmitter ON period for a single transmitted carrier. The minimum requirement below may be applied regionally. There may also be regional requirements to declare the OTA occupied bandwidth according to the definition in the present clause.

The OTA occupied bandwidth is defined as a directional requirement and shall be met in the manufacturer’s declared OTA coverage range at the RIB.

#### 9.7.2.2	Minimum requirement for MSR operation

For AAS BS in MSR operation, the minimum requirement for OTA occupied bandwidth is the same as that stated in 3GPP TS 37.104 [9], subclause 6.6.3.

#### 9.7.2.3	Minimum requirement for single RAT UTRA operation

For AAS BS in single RAT UTRA operation FDD, the minimum requirement for OTA occupied bandwidth is the same as that stated in 3GPP TS 25.104 [6] subclause 6.6.1.

For AAS BS in single RAT UTRA operation TDD 1,28Mcps option, the minimum requirement for OTA occupied bandwidth is the same as that stated in 3GPP TS 25.105 [7], subclause 6.6.1.

#### 9.7.2.4	Minimum requirement for single RAT E-UTRA operation

For AAS BS in single RAT E-UTRA operation, the minimum requirement for OTA occupied bandwidth is the same as that stated in 3GPP TS 36.104 [8], subclause 6.6.1.

### 9.7.3	OTA Adjacent Channel Leakage power Ratio

#### 9.7.3.1      General

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

#### 9.7.3.2      Minimum requirement for MSR operation

For AAS BS in MSR operation using E-UTRA, the OTA ACLR limits for AAS BS are the same as those specified in 3GPP TS 37.104 [9] subclauses 6.6.4.1. The ACLR absolute limits in 3GPP TS 37.104 [9] subclauses 6.6.4.1 are replaced with the following:

- For E-UTRA Category A AAS BS of Wide Area BS class the OTA ACLR absolute limit of -4dBm/MHz shall apply,

- For E-UTRA Category B AAS BS Wide Area BS class the OTA ACLR absolute limit of -6dBm/MHz shall apply,

- For E-UTRA AAS BS of Medium Range BS class the OTA ACLR absolute limit of -16 dBm/MHz shall apply,

- For E-UTRA AAS BS of Local Area BS class the OTA ACLR absolute limit of -23dBm/MHz shall apply.

The OTA ACLR limit or the ACLR absolute limit of AAS BS, whichever is less stringent, shall apply outside the Base Station RF Bandwidth or Radio Bandwidth.

For AAS BS in MSR operation using UTRA FDD, the minimum requirement for OTA ACLR are the same as those specified in 3GPP TS 25.104 [6], subclause 6.6.2.2, where the ACLR absolute limit is replaced with the following:

- For UTRA FDD Category A AAS BS of Wide Area BS class the OTA ACLR absolute limit of -7dBm/MHz shall apply,

- For UTRA FDD Category B AAS BS Wide Area BS class the OTA ACLR absolute limit of -9dBm/MHz shall apply,

- For E-UTRA FDD AAS BS of Medium Range BS class the OTA ACLR absolute limit of -19 dBm/MHz shall apply,

- For E-UTRA FDD AAS BS of Local Area BS class the OTA ACLR absolute limit of -26dBm/MHz shall apply.

The OTA ACLR limit or the ACLR absolute limit of AAS BS, whichever less stringent, shall apply outside the Base Station RF Bandwidth or Radio Bandwidth.

For AAS BS in MSR operation using UTRA TDD 1,28 Mcps operation, the minimum requirement for OTA ACLR are the same as those specified in 3GPP TS 25.105 [7], subclause 6.6.2.2.1.2, and applies outside the Base Station RF Bandwidth or Radio Bandwidth.

For a RIB supporting operation in non-contiguous spectrum, the OTA ACLR requirement also applies for the first adjacent channel inside any sub-block gap with a gap size Wgap ≥ 15MHz. The OTA ACLR requirement for the second adjacent channel applies inside any sub-block gap with a gap size Wgap ≥ 20 MHz.

OTA CACLR requirements apply in sub-block gaps as defined in 3GPP TS 37.104 [9], subclause 6.6.4.4. Either the OTA CACLR limit or the OTA ACLR absolute limit of AAS BS shall apply, whichever is less stringent.

For a multi-band RIB, the OTA ACLR requirement also applies for the first adjacent channel inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 15MHz. The OTA ACLR requirement for the second adjacent channel applies inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 20 MHz.

OTA CACLR requirements apply in Inter RF Bandwidth gaps as defined in 3GPP TS 37.104 [9], subclause 6.6.4.4. Either the OTA CACLR limit or the OTA ACLR absolute limit of AAS BS shall apply, whichever is less stringent.

#### 9.7.3.3     Minimum requirement for single RAT UTRA operation

For AAS BS in single RAT UTRA operation FDD, the OTA ACLR minimum requirements are the same as those specified in 3GPP TS 25.104 [6], subclauses 6.6.2.2.1, where the ACLR absolute limit is replaced with the following:

- For UTRA FDD Category A AAS BS of Wide Area BS class the OTA ACLR absolute limit of -7dBm/MHz shall apply,

- For UTRA FDD Category B AAS BS Wide Area BS class the OTA ACLR absolute limit of -9dBm/MHz shall apply

- For E-UTRA FDD AAS BS of Medium Range BS class the OTA ACLR absolute limit of -19 dBm/MHz shall apply,

- For E-UTRA FDD AAS BS of Local Area BS class the OTA ACLR absolute limit of -26dBm/MHz shall apply.

The OTA ACLR limits or the absolute ACLR limits apply whichever is less stringent.

For AAS BS in single RAT UTRA operation FDD, the OTA CACLR limits are the same as those specified in 3GPP TS 25.104 [6], subclauses 6.6.2.2.2. The ACLR absolute limit of AAS BS is replaced with the following:

- For UTRA FDD Category A AAS BS of Wide Area BS class the OTA ACLR absolute limit of -7dBm/MHz shall apply,

- For UTRA FDD Category B AAS BS Wide Area BS class the OTA ACLR absolute limit of -9dBm/MHz shall apply

- For E-UTRA FDD AAS BS of Medium Range BS class the OTA ACLR absolute limit of -19 dBm/MHz shall apply,

- For E-UTRA FDD AAS BS of Local Area BS class the OTA ACLR absolute limit of -26dBm/MHz shall apply.

The OTA CACLR limit or the OTA ACLR absolute limit of AAS BS shall apply, whichever is less stringent.

For AAS BS in single RAT UTRA operation TDD 1,28 Mcps option, the AAS BS minimum requirements are the same as those specified in 3GPP TS 25.105 [7], subclause 6.6.2.2.

#### 9.7.3.4      Minimum requirement for single RAT E-UTRA operation

For AAS BS in single RAT E-UTRA operation, the OTA ACLR and OTA CACLR limits are the same as those specified in 3GPP TS 36.104 [8], subclauses 6.6.2.1 and 6.6.2.2. The ACLR absolute limits in 3GPP TS 36.104 [8] subclauses 6.6.2.1 and 6.6.2.2 are replaced with the following:

- For E-UTRA Category A AAS BS of Wide Area BS class the ACLR absolute limit of -4dBm/MHz shall apply,

- For E-UTRA Category B AAS BS Wide Area BS class the ACLR absolute limit of -6dBm/MHz shall apply,

- For E-UTRA AAS BS of Medium Range BS class the ACLR absolute limit of -16 dBm/MHz shall apply,

- For E-UTRA AAS BS of Local Area BS class the ACLR absolute limit of -23dBm/MHz shall apply.

The OTA ACLR (CACLR) limit or the ACLR absolute limit of AAS BS shall apply, whichever is less stringent.

--------------end of text proposal-------------