3GPP TSG-RAN WG4 #84 Meeting                                R4-1707187

Berlin, Germany, 21 - 25 August, 2017

Agenda item:	9.3.3

Source: 	ZTE

Title: 	NR spectrum utilization of single numerology case for 28GHz

Document for:	Approval

Introduction

In the NR study item phase, RAN4 agreed to enhance the NR spectrum utilization above 90%, and for NR in-band requirements when RAN4 defines 5G NR requirements RAN4 should ensure sufficiently good spectral efficiency in both single and mixed numerology cases. These have been captured in TR 38.803 [1].

For spectrum utilization, there are two main types for implementation mostly discussed, windowing and filtering. It has been proven in [2] [3] that filtering is too complex for implementation and windowing type is preferred to be used for obtaining RB numbers for different combinations of channel BW and SCS. [4] analyzed filtering and windowing and concludes that windowing type is the only suitable approach for isolating different numerologies because filtering would imply a static assignment of RBs to each numerology. Therefore windowing type is preferred to be used for obtaining RB numbers for different combinations of channel BW and SCS.

In RAN4 NR#2 meeting in this June R4-1706929 WF on Spectral Utilization was approved for mmWave as below [5]:

In this contribution, based on simulation results from BS side we present spectrum utilization numbers for ~28GHz in single numerology case. The final single set spectrum utilization numbers for Rel-15 should be determined by checking the simulation results from both BS and UE sides and further discussions and clear RAN4 agreements on the open issues pointed out in this contribution.

NR BS spectrum utilization of single numerology case for ~28 GHz

For ~28GHz spectrum utilization simulations, the assumed mmWave SEM is from [9] as below. The mmWave SEM is still under discussion [11] and the spectrum utilization numbers should be finalized after the mmWave SEM has been specified.

Other evaluation assumptions (for ~28GHz):

- Waveform: 			windowing type of waveform FB-OFDM option 2 described in [7] [8].

- BS DL EVM:  		<8% (LTE 64QAM EVM requirement).

- ACLR:				28 dBc for ~28GHz [10].

- BS DL output power:   34.5 dBm for all channel BWs.

- BS PA model: 		28GHz CMOS GMP model from [1]. Please see the Appendix.

In the simulations, 6dB back off and simple fixed phase compensation.

- DPD: 				No DPD.

- Channel raster:		RB size, asymmetrical guard band size.

The simulation results from BS side for 28GHz are summarized in Table 2 below. Please note that if the BS DL output power is higher than 34.5dBm, even with 6dB back off and simple fixed phase compensation the NRB will be smaller than the numbers in Table 1 otherwise it will not meet the SEM requirements.

Table 1: Transmission bandwidth configuration NRB for NR frequency bands around 28GHz for PDSCH/PUSCH single numerology case

| Channel bandwidth BWChannel [MHz] |  | 50 | 100 | 150 | 200 | 400 |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission bandwidth configuration NRB | SCS=60 KHz | 65 | 131 | 197 | 263 | N.A |
|  | SCS=120 KHz | 31 | 65 | 97 | 131 | 263 |

Some RAN4 requirements have impact to spectrum utilization and they have not been finalized, e.g. EVM, how to place the channel RBs [12], etc. Different companies might use different assumptions in the spectrum utilization simulations so it’s better that RAN4 has some common assumptions for the evaluation to align the simulation results. We have the following proposal:

Proposal 1: The open issues below should be discussed and clarified/specified in RAN4 first before the single set spectrum utilization numbers for mmWave could be finalized:

- BS DL EVM: should be specified for single numerology case first.

- ACLR: The absolute limits for ACLR values corresponding to different BS classes are FFS at this moment according to [10].

- BS output power: what’s the assumed BS output power for each channel BW in the evaluation?

- BS PA model: Rapp model could not model some PA behaviors such as the memory effects and IMD5 for larger channel BWs. These behaviors have negative impacts to EVM and ACLR. Which BS PA model should be used in the evaluation for around 28GHz and 50+ GHz ?

- Relevant BS RX and UE requirements.

- RB placement, and asymmetrical guard band size at the edges of the channel BW.

Conclusions

Based on the discussion in this contribution, we have the following Proposal:

Proposal 1: The open issues below should be discussed and clarified/specified in RAN4 first before the single set spectrum utilization numbers for mmWave could be finalized:

- BS DL EVM: should be specified for single numerology case first.

- ACLR: The absolute limits for ACLR values corresponding to different BS classes are FFS at this moment according to [10].

- BS output power: what’s the assumed BS output power for each channel BW in the evaluation?

- BS PA model: Rapp model could not model some PA behaviors such as the memory effects and IMD5 for larger channel BWs. These behaviors have negative impacts to EVM and ACLR. Which BS PA model should be used in the evaluation for around 28GHz and 50+ GHz?

- Relevant BS RX and UE requirements.

- RB placement, and asymmetrical guard band size at the edges of the channel BW.

# References

- RP-170439, TR 38.803 v2.0.0 on Study on New Radio Access Technology: RF and co-existence aspects, March 2017.

- R4-1701793, Feasible Spectral Utilization for NR, Qualcomm, February 2017.

- R4-1703267, Filter Complexity analysis and trade-offs, Qualcomm , April 2017.

- R4-1703083, On spectrum utilization and considerations for filtering and windowing, Ericsson, April 2017.

- R4-1706929 WF on spectrum utilization, June 2017.

- 3GPP TS 36.104 v14.2.0, E-UTRA Base Station (BS) radio transmission and reception, December 2016.

- R1-1609778, Further discussions on FB-OFDM, ZTE, ZTE Microelectronics, October, 2016.

- R1-1609779, Further evaluations on FB-OFDM, ZTE, ZTE Microelectronics, October, 2016.

- R4-1706073,	mmWave SEM, May 2017.

- R4-1706954 WF on BS ACLR ACS mmWave, June 2017.

- R4-1706964 WF on SEM mmWave, June 2017.

- R4-1706981 WF on RF channel raster for sub-6GHz and mmWave bands, June 2017.

Appendix

28GHz CMOS GMP model from [1]:

The third model is based on advanced circuit simulation of a CMOS PA research prototype. The model has been derived with input and output data at a sample rate of 2.281 GHz and a signal bandwidth of 400MHz.

The corresponding GMP model has -41.7dB NMSE and is defined by:

akl;k∈[0,1,…,7]:

l=1:

[-0.0109821+0.00313982i; -0.00397658-0.0427409i; -0.171194+0.151692i; 0.879844-0.0235651i;
-1.97684-0.862044i; 2.32524+1.99694i; -1.34472-1.77602i; 0.289959+0.559338i]

l=0:

[0.473465+0.860276i; -0.953417+0.640666i; 1.9899-2.3847i; 7.5417+6.38381i;
-64.8415-60.8762i; 159.01+189.579i; -167.466-225.579i; 65.4247+92.5967i]

l=-1:

[0.0164844+0.00671299i; -0.0198519+0.177212i; 0.669594-0.543745i; -2.98038-0.279477i;
6.6717+4.50511i; -8.26935-9.04627i; 5.42365+7.52782i; -1.47259-2.32623i]

bklm;k∈[0,1,…,7]:

l=1, m=0:

[-0.000292543-0.0150556i; -0.122202-0.283752i; 2.56792+4.68957i; -18.4244-34.2816i;
66.3648+126.766i; -124.066-239.871i; 115.273+220.218i; -42.1527-77.6225i]

l=-1, m=0:

[0.0163452+0.00969618i; -0.281971-0.188069i; 3.35025+3.60649i; -24.5434-31.1539i;
87.5451+124.093i; -157.821-243.086i; 139.85+227.416i; -48.6255-81.0794i]

Figure A.1-5 and Figure A.1-6 show the gain and phase characteristics of the GMP and static model using the same OFDM signal that was used for model estimation.

Figure A.1-5 Gain characteristics of 28GHz CMOS PA, (blue) GMP model (red) static model.

Figure A.1-6 Phase characteristics of 28GHz CMOS PA, (blue) GMP model (red) static model.