3GPP TSG-RAN WG4 Meeting #84 	R4-1707404

Berlin, Germany, 21 – 25 August, 2017

Agenda item:	6.2.4.2

Source: 	Intel Corporation

Title: 	PDSCH demodulation results for Rel-13 non-BL UE with 2Rx and 4Rx in FDD

Document for:	Discussion

- Introduction

In RAN4#83 meeting, the demodulation performance of Rel-13 non-BL UE with 2Rx and 4Rx was discussed. A WF [1] was agreed and companies were encouraged to provide simulation results to define new demodulation requirements for non-BL UE supporting coverage enhancement with 2Rx/4Rx. In this contribution, we first provide our FDD simulation results for non-BL UE with 2Rx/4Rx under ideal cases, i.e. no impairment is considered. We then share our view on the SNR level and/or repetition level for different test scenarios.

- Simulation assumptions and results

In [1], it was agreed to re-use the PDSCH simulation assumptions of Cat.M1 in TS 36.101 Section 8.11.1 for simulations of non-BL UEs with 2Rx/4Rx, except the repetition number where it applies.

  - Non-BL UE in TM6 with CE Mode A

For TM6 single-layer spatial multiplexing tests, it considers CE mode A with Repetition Number = 1 and Frequency Hopping disabled. And the simulation assumptions are summarised in Table 1 below:

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 10MHz 16QAM 1/2 | R.79 FDD | OP.2 FDD | EPA5 | 2x1 Low | 70% | 8.6 |
| 2 | 10MHz 16QAM 1/2 | R.79 FDD | OP.2 FDD | EPA5 | 2x2 Low | 70% | [ ] |
| 3 | 10MHz 16QAM 1/2 | R.79 FDD | OP.2 FDD | EPA5 | 2x4 Low | 70% | [ ] |

Table 1. Simulation assumption for Non-BL UE in TM6 with CE Mode A

Fig.1 below shows the throughput simulation results comparing non-BL UEs with 2Rx/4Rx with Cat.M1 UE.

Fig.1 Normalized Throughput for non-BL UEs and Cat.M1 UE, TM6, FDD, Repetition = 1

Observation 1: 2Rx has about 4dB gain against 1Rx; and 4Rx has about 3dB gain against 2Rx.

Proposal 1: The TM6 FDD reference SNR is about 3dB for 2Rx, and is about -0.5dB for 4Rx.

  - Non-BL UE in TM9 with CE Mode A

For TM9 single-layer spatial multiplexing tests, it considers CE mode A with Repetition Number = 8 for Cat.M1 and Frequency Hopping enabled. And the simulation assumptions are summarised in Table 2 below:

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 10MHz QPSK 1/3 | R.80 FDD | OP.2 FDD | EPA5 | 2x1 Low | 70 | -2.0 |
| 2 | 10MHz QPSK 1/3 | R.80 FDD | OP.2 FDD | EPA5 | 2x2 Low | 70 | [ ] |
| 3 | 10MHz QPSK 1/3 | R.80 FDD | OP.2 FDD | EPA5 | 2x4 Low | 70 | [ ] |

Table 2. Simulation assumption for Non-BL UE in TM9 with CE Mode A

Fig.2 below shows the throughput simulation results comparing 2Rx non-BL UE with Cat.M1 UE.

Fig.2 Normalized Throughput for 2Rx non-BL UE and Cat.M1 UE, TM9, FDD

Observation 2: For 2Rx non-BL UE, it can achieve similar operation SNR range to 1Rx UE by reducing the repetition number from 8 to 4.

Observation 3: To achieve the -6dB target SNR, Repetition Number = 8 is needed for 2Rx non-BL UE.

Proposal 2: Choose repetition number = 4 for 2Rx non-BL UE if to keep similar SNR level to 1Rx UE; otherwise, choose repetition number = 8 for 2Rx non-BL UE to meet the -6dB SNR target.

Fig.3 below shows the throughput simulation results comparing 4Rx non-BL UE with Cat.M1 UE.

Fig.3 Normalized Throughput for 2Rx non-BL UE and Cat.M1 UE, TM9, FDD

Observation 4: For 4Rx non-BL UE, it can achieve similar operation SNR range to 1Rx UE by reducing the repetition number to 2.

Observation 5: To achieve the -6dB target SNR, Repetition Number = 4 is needed for 4Rx non-BL UE.

Proposal 3: Choose repetition number = 2 for 4Rx non-BL UE if to keep similar SNR level to 1Rx UE; otherwise, choose repetition number = 4 for 4Rx non-BL UE to meet the -6dB SNR target.

  - Non-BL UE in TM2 with CE Mode A and CE ModeB

For TM2 transmit diversity tests, it first considers CE ModeB with Repetition Number = 64 for Cat.M1 and Frequency Hopping enabled. Then, it considers CE ModeA with Repetition Number = 1 and Frequency Hopping disabled. The simulation assumptions are summarised in Table 3 below:

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| CE ModeB 1 | 10MHz QPSK 1/10 | R.81 FDD | OP.2 FDD | ETU1 | 2x1 Low | 70 | -13.5 |
| CE ModeB 2 | 10MHz QPSK 1/10 | R.81 FDD | OP.2 FDD | ETU1 | 2x2 Low | 70 | [ ] |
| CE ModeB 3 | 10MHz QPSK 1/10 | R.81 FDD | OP.2 FDD | ETU1 | 2x4 Low | 70 | [ ] |
| CE ModeA 1 | 10MHz 16QAM 1/2 | R.79 FDD | OP.2 FDD | EPA5 | 2x1 Low | 70 | 9.4 |
| CE ModeA 2 | 10MHz 16QAM 1/2 | R.79 FDD | OP.2 FDD | EPA5 | 2x2 Low | 70 | [ ] |
| CE ModeA 3 | 10MHz 16QAM 1/2 | R.79 FDD | OP.2 FDD | EPA5 | 2x4 Low | 70 | [ ] |

Fig.4 below shows the throughput simulation results comparing 2Rx/4Rx non-BL UE with Cat.M1 UE in CE ModeB.

Fig.4 Normalized Throughput for 2Rx/4Rx non-BL UEs and Cat.M1 UE, TM2, FDD, CE ModeB

Observation 6: For 1Rx UE in CE ModeB with Repetition Number = 64, 2Rx UE requires 1/2 and 4Rx UE requires 1/4 of 64 repetition to achieve similar SNR performance to 1Rx UE, as well as achieve -15dB SNR target.

Proposal 4: Choose repetition number = 32 for 2Rx non-BL UE; and repetition number = 16 for 4Rx non-BL UE.

Fig.5 below shows the throughput simulation results comparing 2Rx/4Rx non-BL UE with Cat.M1 UE in CE ModeA and Repetition Number = 1.

Fig.5 Normalized Throughput for 2Rx/4Rx non-BL UEs and Cat.M1 UE, TM2, FDD, CE ModeA

Observation 7: 2Rx has about 4dB gain against 1Rx; and 4Rx has about 3dB gain against 2Rx.

Proposal 5: For the TM2, CE ModeA, FDD, reference SNR is about 5dB for 2Rx, and is about 2dB for 4Rx.

- Conclusion

In this contribution, we first provide our FDD simulation results for non-BL UE with 2Rx/4Rx under ideal cases, i.e. no impairment is considered. We then share our view on the SNR level and/or repetition level for different test scenarios.

Proposal 1: The TM6 FDD reference SNR is about 3dB for 2Rx, and is about -0.5dB for 4Rx.

Proposal 2: Choose repetition number = 4 for 2Rx non-BL UE if to keep similar SNR level to 1Rx UE; otherwise choose repetition number =8 for 2Rx non-BL UE to meet the -6dB SNR target.

Proposal 3: Choose repetition number = 2 for 4Rx non-BL UE if to keep similar SNR level to 1Rx UE; otherwise, choose repetition number =4 for 4Rx non-BL UE to meet the -6dB SNR target.

Proposal 4: Choose repetition number = 32 for 2Rx non-BL UE; and repetition number = 16 for 4Rx non-BL UE.

Proposal 5: For the TM2, CE ModeA, FDD, reference SNR is about 5dB for 2Rx, and is about 2dB for 4Rx.

- Reference

[1] R4-1706196, “WF on requirements for Rel-13 Non-BL UE”, Huawei, HiSilicon, Ericsson, Intel Corporation, Qualcomm, May 2017.