3GPP TSG-RAN WG4 Meeting #84	R4-1707473

Berlin, Germany, 21 - 25 Aug, 2017

Agenda item:	9.3.1.1

Source:	Dish Network

Title:	NR minimum BW and SS block SCS considerations

Document for:	Approval

# 1	Introduction

This contribution provides considerations on NR minimum channel BW and NR SS block SCS. We have found this issue is somewhat unclear when several RAN1 and RAN4 discussions and/or decisions are combined. 5MHz has been agreed as applicable BW for many bands in RAN4 while for instance NR/LTE DL co-existence requires 30kHz SS SCS according to RAN1.

# 2	Discussion

In previous RAN4 meeting, WF on band specific UE channel bandwidths was agreed [1], including 5MHz for many LTE re-farming bands. This allows operators who currently have 5Mhz spectrum block deployed in LTE to re-farm to NR or operators who have 5MHz in possession but not deployed to go directly to NR.

RAN1 has made a bunch of agreements on NR SS mapping recently [2]. Table 1 below illustrates related numerologies.

| SS SCS | SS block BW | Minimum channel BW |
| --- | --- | --- |
| 15kHz | 4.32MHz | 5MHz |
| 30kHz | 8.64MHz | 10MHz |

Table 1 Synchronization Signal block numerologies

In previous meeting, there were contributions in both RAN4 [3] and RAN1 [4] that addressed impacts of NR/LTE DL co-existence. The key takeaway is that without any additional measures, NR/LTE co-existence requires 30kHz SS SCS to map NR SS and LTE CRS in a way that they don’t overlap in time domain.

In short, this would mean RAN4 should specify more than one SS SCS at least for each LTE re-farming bands where NR/LTE DL co-existence is used. RAN4 should discuss whether this is feasible and depending on the outcome send an LS to RAN1.This LS should be sent to RAN1 during this meeting, because there is not too much time to study this and do potential specification work before the December plenary.

In future, when multiple NR use scenarios such as eMBB and URLLC are used on same band, having more than one SS SCS or some alternative method(s) need to be addressed both in RAN1 and RAN4. At least one SS mapping defined in RAN1 is using 30kHz SS SCS [2]. If that is the only SS SCS for URLLC then it is not clear how URLLC can be used with 5MHz BW.

The following points are for approval:

Proposal1: For NR frequency bands below 6GHz where 5MHz CH BW is defined and there is an operator interest in LTE-NR DL sharing, RAN4 needs to discuss the following options:

Option 1: Multiple SS block SCS is allowed per band

Option 2: Send an LS to RAN1 for further solution

Proposal2: If option 1 is agreed, RAN4 shall send an LS to inform RAN1

Proposal3: RAN1 should be asked to clarify usage of eMBB and URLLC within frequency bands where 5MHz is an applicable bandwidth

# 3	Conclusions

Considerations for NR minimum bandwidth and SS SCS are provided. The following proposals are made.

Proposal1: For NR frequency bands below 6GHz where 5MHz CH BW is defined and there is an operator interest in LTE-NR DL sharing, RAN4 needs to discuss the following options:

Option 1: Multiple SS block SCS is allowed per band

Option 2: Send an LS to RAN1 for further solution

Proposal2: If option 1 is agreed, RAN4 shall send an LS to inform RAN1

Proposal3: RAN1 should be asked to clarify usage of eMBB and URLLC within frequency bands where 5MHz is an applicable bandwidth

# 4	References

[1] R4-1706982, “WF on band specific UE channel bandwidth”, NTT Docomo

[2] R1 chairman notes RAN1#AH_NR2

[3] R4-1706501 “Min. system bandwidth and SS Block SCS”, AT&T

[4] R1-1710412 “NR SS block and burst set composition”, AT&T