3GPP TSG-RAN WG4 #84	R4- 1707474

Berlin, Germany, 21 – 25 August, 2017

Source:	Ericsson

Title:	Draft TS 37.113 Specification text for Section 8.2

Agenda item:	8.28.3.6

Document for:	Discussion

# 1.	Introduction

In this contribution a text proposal is presented for radiated emissions section 8.2 of TS 37.113 [1] specific for OTA AAS BS and we would like to encourage feedback from other companies on the proposed text.

An alternative approach on how to specify the EMC requirements for OTA AAS BS may be to directly add requirements to TS 37.114 [2], which currently serves as mainly a list of references to single-RAT and MSR EMC specifications. If the approach in this text proposal is chosen, only some minor changes to 37.114 in e.g. definitions and test conditions will be needed.

Some terminology has been added in this text proposal. These changes should be aligned with agreements regarding terminology in RAN4-#84 meeting.

# 2.	Discussion

Some aspects to consider when specifying the EMC radiated emissions requirements for OTA AAS BS are the following:

- The EMC radiated emissions from the enclosure port can not be separated from the transmitter spurious emissions for an OTA AAS BS

- The OTA spurious emission requirements will be specified in subclause 9.7.6 of 37.105 [3]

- The OTA spurius emission conformance requirements will be specified in 37.145-2 [4]

- RAN4 has agreed to specify one single test for the total OTA spurious emission requirement with one set of combined limits.

- RAN4 has agreed not to duplicate requirements in the RF and EMC specifications.

The bullets above have been considered when writing this draft specification text 37.113 to include requirements for OTA AAS BS. It can be seen below in the text proposal that the intension of this proposal is to specify a new subclause 8.2.2 specifically for OTA AAS BS, where the RF core and conformance requirements are referred. The intension is to not specify any EMC radiated emission requirements individually in 37.113 for OTA AAS BS.

# 4.	References

3GPP TS 37.113 v14.1.0

3GPP TS 37.114 v14.1.0

3GPP TS 37.105 v14.0.0

3GPP TS 37.145-2 v14.0.0

[Start of Text Proposal]

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [x] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

active antenna system base station: BS system which combines an antenna array with an transceiver unit array. An AAS BS may include a radio distribution network

Ancillary equipment: Equipment (apparatus), used in connection with a receiver, transmitter or transceiver is considered as an ancillary equipment (apparatus) if:

-	the equipment is intended for use in conjunction with a receiver, transmitter or transceiver to provide additional operational and/or control features to the radio equipment, (e.g. to extend control to another position or location); and

-	the equipment cannot be used on a stand-alone basis to provide user functions independently of a receiver, transmitter or transceiver; and

-	the receiver, transmitter or transceiver to which it is connected, is capable of providing some intended operation such as transmitting and/or receiving without the ancillary equipment (i.e. it is not a sub-unit of the main equipment essential to the main equipment basic functions).

Base Station equipment: Radio and/or ancillary equipment intended for operation at a fixed location and powered directly or indirectly (e.g. via an AC/DC converter or power supply) by AC mains network, or an extended local DC mains network.

Base Station RF bandwidth: The bandwidth in which a Base Station transmits and/or receives multiple carriers and/or RATs simultaneously

Base Station RF bandwidth edge: The frequency of one of the edges of the Base Station RF bandwidth

Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Continuous phenomena (continuous disturbance): Electromagnetic disturbance, the effects of which on a particular device or equipment cannot be resolved into a succession of distinct effects (IEC 60050-161 [14]).

Lower RF bandwidth edge:  The frequency of the lower edge of the Base Station RF bandwidth, used as a frequency reference point for transmitter and receiver requirements

Maximum Base Station RF bandwidth: The maximum RF bandwidth supported by a BS within an operating band.

Maximum throughput: The maximum achievable throughput for a reference measurement channel.

MB-MSR Base Station: MSR Base Station characterized by the ability of its transmitter and/or receiver to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different non-overlapping operating band than the other carrier(s).

MSR Base station: Base Station characterized by the ability of its receiver and transmitter to process two or more carriers in common active RF components simultaneously in a declared RF bandwidth, where at least one carrier is of a different RAT than the other carrier(s).

NOTE:	A Base Station where receiver or transmitter processes carriers of different RATs simultaneously, but not through common active RF components, is not an MSR BS according to the above definition. Such a BS is in the present specification referred to as “other BS supporting more than one RAT”.

NB-IoT In-band operation: NB-IoT is operating in-band when it utilizes the resource block(s) within a normal E-UTRA carrier.

NB-IoT guard band operation: NB-IoT is operating in guard band when it utilizes the unused resource block(s) within a E-UTRA carrier’s guard-band.

NB-IoT standalone operation: NB-IoT is operating standalone when it utilizes its own spectrum, for example the spectrum currently being used by GERAN systems as a replacement of one or more GSM carriers, as well as scattered spectrum for potential IoT deployment.

OTA AAS BS: AAS BS which has a radiated RF interface only

Radio communications equipment : Telecommunications equipment which includes one or more transmitters and/or receivers and/or parts thereof for use in a fixed, mobile or portable application. It can be operated with ancillary equipment but if so, is not dependent on it for basic functionality.

Radio equipment: Equipment which contains Radio digital unit and Radio unit.

Radio digital unit: Equipment which contains base band and functionality for controlling Radio unit.

Radio unit: Equipment which contains transmitter and/or receiver.

Port: A particular interface, of the specified equipment (apparatus), with the electromagnetic environment. For example, any connection point on an equipment intended for connection of cables to or from that equipment is considered as a port (see Figure 3.1.1).

Receiver exclusion band: The receiver exclusion band is the band of frequencies over which no tests of radiated immunity of a receiver are made. The exclusion band for receivers is expressed relative to the base station receive band.

Signal and control: Port which carries information or control signals, excluding antenna ports.

Telecommunication port: Ports which are intended to be connected to telecommunication networks (e.g. public switched telecommunication networks, integrated services digital networks), local area networks (e.g. Ethernet, Token Ring) and similar networks.

Throughput: The number of payload bits successfully received per second for a reference measurement channel in a specified reference condition.

Transient phenomena: Pertaining to or designating a phenomena or a quantity which varies between two consecutive steady states during a time interval short compared with the time-scale of interest (IEC 60050-161 [14]).

Transmitter exclusion band: The transmitter exclusion band is the band of frequencies over which no tests of radiated immunity of a transmitter are made. The exclusion band for transmitters is expressed relative to the carrier frequencies used (the carrier frequencies of the base stations activated transmitter(s).)

Upper RF bandwidth edge: The frequency of the upper edge of the Base Station RF bandwidth, used as a frequency reference point for transmitter and receiver requirements

Figure 3.1-1: Examples of ports

Figure 3.1-2:  BS with single enclosure solution

Enclosure Port

Apparatus

AC power port

DC power port

Earth port

Signal/control port

Telecommunication port

Integrated antenna array

Figure 3.1-3:  BS with multiple enclosure solution

Figure 3.1-4:  OTA AAS BS with integrated antenna array

{unchanged sections omitted}

### 8.2.2	Radiated emission for OTA AAS Base Stations

This  subclause is applicable to OTA AAS Base stations.

Radiated emissions from the enclosure port can for an OTA AAS BS not be separated from the RF transmitter spurious emissions. For OTA AAS BS a combined conformance requirement is specified taking into account spurious emissions from the enclosure port as well as the transmitter.

Transmitter spurious emission core requirements are specified in TS 37.105, whereas conformance requirements for the combined emissions requirement are specified in TS 37.145.

#### 8.2.2.1	Definition

The tests referred to in this subclause assesses the ability of OTA AAS BS to limit unwanted emission from the enclosure port as part of a combined test assessing the total radiated spurious emissions from an OTA AAS BS.

Note: Conformance with clause XX of TS 37.145 [X] means conformance with the total radiated spurious emission limit which has been derived as a TRP sum of transmitter spurious emission limits and radiated emission limits from the enclosure port of OTA AAS BS (Table 8.2.1.3-1).

#### 8.2.2.2	Test method

The test method in subclause X.X.X of 3GPP TS 37.145 [X] shall apply.

#### 8.2.2.3	Limits

The frequency boundary and reference bandwidths for the detailed transitions of the limits between the requirements for out of band emissions and spurious emissions are based on ITU-R Recommendations SM.329 [10] and SM.1539 [23].

The OTA AAS BS shall meet the limits below, when these are added to the transmitter spurious emission limits of TS 37.105 as specified in clause XX of TS 37.145 [X].:

Table 8.2.1.3-1: Limits for radiated emissions from BS

| Frequency range | Minimum requirement (TRP)/Reference Bandwidth |
| --- | --- |
| 30 MHz f <1000 MHz | -33.85 dBm/100 kHz |
| 1 GHz f <12,75 GHz | -27.85 dBm/ 1MHz |
| FBW RF,low - 10 MHz < f < FBW RF,high + 10 MHz (Note 1) | Not defined |
| NOTE 1: For BS capable of multi-band operation, the frequency ranges relating to the RF bandwidths of all supported bands apply. |  |

Key:

FBW RF,high:		Upper RF bandwidth edge

FBW RF,low :		Lower RF bandwidth edge

#### 8.2.2.4	Interpretation of the measurement results

The interpretation of the results recorded in a test report for the total radiated spurious emissions from an OTA AAS BS described in subclause XX of 3GPP TS 37.145 [X] shall be according to subclause XY of 3GPP TS 37.145 [X],

.