3GPP TSG-RAN WG4 Meeting #84	R4-1707681

Berlin, Germany, 21 – 25 August 2017

Agenda Item:	7.15.2.2

Source:	Ericsson

Title:	Simulation result for eFD-MIMO class A PMI reporting test

Document for:	Discussion

# 1	Introduction

RAN4#83 discussed the test setup and simulation assumption for the eFD-MIMO Class A PMI reporting test [1][2] and agreed with the following way forward:

- Test 1 Single PMI test case: EPA5Hz, PUSCH 3-1 mode, CDM4, (O1, O2) = (8,4), (N1, N2) = (3,4)

  - Antenna Ports:

    - 24 ports (N1,N2) =  (3,4)

  - Test 1-a: without CSI-RS density reduction

  - Test 1-b: with CSI-RS density reduction

    - d =1/3, Comb-offset list: {0,1,2}

  - Pending on UE capability for supporting CSI-RS density reduction, pick up one of above test case to pass

- Test 2 Multiple PMI test case (32 ports): EVA5Hz, PUSCH 1-2 mode, CDM8, (O1, O2) = (8,4), (N1, N2) = (4,4), d=1

- MCS and Rank

  - Single PMI test case:

    - Option1: 16QAM ½ Rank2

    - Option2: 64QAM ½ Rank1

  - Multiple PMI test case

    - Option 1: 16QAM ½ Rank2

    - Option 2: 64QAM ½ Rank2

- Reusing existing values for parameters α1, α2, β and γ for high spatial correlation

In this contribution, we continue the discussion on the test setup based on our simulation result.

# 2	Simulation results

## 2.1	Single PMI test

Figure 1 shows the simulation results for follow PMI and random PMI with 64QAM rank 1 and 16QAM rank 2. For each simulation, we show the case without the CSI-RS frequency domain density reduction and with the CSI-RS frequency domain density reduction with 1/3. For CSI-RS density reduction, we show the case the offset = 0. Note that it is observed that there is no difference among the offset values 0, 1 or 2.

It is observed that no CSI-RS density reduction gives better follow PMI performance than 1/3 CSI-RS density reduction in lower SNR region. We think it is because the dense CSI-RS gives better PMI estimation.

Figure 1	Simulation results for single PMI test with PUSCH 3-1.

Figure 2 shows the throughput ratio between the followed PMI and random PMI, and Table 1 summarizes the throughput ratio between the follow PMI and random PMI at SNR test points where the 60%/70%/80%/90% of maximum throughput is achieved with the follow PMI. If we choose the lower SNR test points such as 60% or 70%, the throughput ratio becomes around 10.0 or more. Considering the existing PMI test requirement, we prefer to set the test points so that the expected gamma is around 3.0. We therefore propose to choose 16QAM rank2 for MCS and SNR test point is chosen so that 90% of the maximum throughput is achieved with the follow PMI. Then the expected gamma is 4.3. We don’t have strong opinion on enabling CSI-RS density reduction or not because there is no performance difference especially for 16QAM rank 2.

Proposal 1: Set 16QAM rank 2 for single PMI test.

Proposal 2: For single PMI test, SNR is chosen so that the 80% or 90% of the maximum throughput is achieved with the follow PMI.

Figure 2	Comparison of throughput ratio of followed PMI and random PMI.

Table 1	Summary of single PMI throughput gain at SNR test points where 60%/70%/80%/90% of maximum throughput is achieved with follow-PMI.

|  | 64QAM rank1 |  |  |  | 16QAM rank2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | No CSI-RS density reduction |  | CSI-RS density reduction 1/3 |  | No CSI-RS density reduction |  | CSI-RS density reduction 1/3 |  |
|  | SNR [dB] | Gamma | SNR [dB] | Gamma | SNR [dB] | Gamma | SNR [dB] | Gamma |
| 60% | -1.7 | 14.8 | -0.9 | 11.6 | 0.2 | 13.3 | 0.5 | 12.0 |
| 70% | -0.8 | 14.1 | -0.1 | 10.1 | 1.7 | 9.6 | 1.7 | 9.3 |
| 80% | 0.3 | 11.0 | 0.9 | 9.2 | 3.4 | 6.8 | 3.3 | 6.6 |
| 90% | 1.9 | 7.6 | 2.1 | 7.0 | 6.2 | 4.3 | 5.9 | 4.3 |

## 2.2	Multiple PMI test

Figure 3 shows the simulation results for follow PMI and random PMI with 16QAM rank 2 and 64QAM rank 2.

Figure 3	Simulation results for multiple PMI test with PUSCH 1-2.

Figure 4 shows the throughput ratio between the followed PMI and random PMI, and Table 2 summarizes the throughput ratio between the follow PMI and random PMI at SNR test points where the 60%/70%/80%/90% of the maximum throughput is achieved with the follow PMI. It is observed from the table that the gamma for 16QAM is very high (9.0) even if we set SNR achieving 90% of maximum throughput. For 64QAM, the gamma values are not so high compared with 16QAM. For example, our simulation shows SNR=13.2dB to achieve 90% of maximum throughput although the expected gamma is 5.5. We therefore propose to adopt 64QAN rank2 for the multiple PMI reporting test and SNR test point is chosen so that the 90% of maximum throughput is achieved with follow PMI.

Proposal 3: Set 64QAM rank 2 for single PMI test.

Proposal 4: For multiple PMI test, SNR is chosen so that the 90% of the maximum throughput is achieved with the follow PMI.

Figure 4	Comparison of throughput ratio of followed PMI and random PMI.

Table 2	Summary of multiple PMI throughput gain at SNR test points where 60%/70%/80%/90% of maximum throughput is achieved with follow-PMI.

|  | 16QAM rank2 |  | 64QAM rank2 |  |
| --- | --- | --- | --- | --- |
|  | SNR [dB] | Gamma | SNR [dB] | Gamma |
| 60% | -1.5 | 44.8 | 5.3 | 19.9 |
| 70% | 0.0 | 26.3 | 7.4 | 12.6 |
| 80% | 1.8 | 17.4 | 9.4 | 9.1 |
| 90% | 4.9 | 9.0 | 13.2 | 5.5 |

# 3	Conclusion

Proposal 1: Set 16QAM rank 2 for single PMI test.

Proposal 2: For single PMI test, SNR is chosen so that the 80% or 90% of the maximum throughput is achieved with the follow PMI.

Proposal 3: Set 64QAM rank 2 for single PMI test.

Proposal 4: For multiple PMI test, SNR is chosen so that the 90% of the maximum throughput is achieved with the follow PMI.

# 4	References

R4-1706194, “Way forward on eFD-MIMO performance requirements”, Samsung, Qualcomm, Huawei, Ericsson.

R4-1706268, “Simulation assumption for eFD-MIMO performance”, Samsung.

# Appendix

## Simulation setup (based on [2])

| Parameter | Unit | Test 1-a (Single PMI test) -24 ports | Test 1-b (Single PMI test) -24 ports with CSI-RS density reduction | Test 2 (Multiple PMI test) -32 ports |
| --- | --- | --- | --- | --- |
| Bandwidth | MHz | 10 | 10 | 10 |
| Transmission mode |  | 9 | 9 | 9 |
| Propagation channel |  | EPA5 | EPA5 | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | 50 | 50 | 6 |
| Correlation and antenna configuration |  | 2D High XP 24 x 2 (N1,N2,P) = (3,4,2) | 2D High XP 24 x 2 (N1,N2,P) = (3,4,2) | 2D High XP 32 x 2 (N1,N2,P) =(4,4,2) |
| Beamforming model |  | [Annex B.4.3] | [Annex B.4.3] | [Annex B.4.3] |
| Cell-specific reference signals |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  | Antenna ports | Antenna ports | Antenna ports |
|  |  | 15,…,38 | 15,…,38 | 15,…,46 |
| Number of CSI-RS ports |  | 24 | 24 | 32 |
| CDM Type |  | CDM4 | CDM4 | CDM8 |
| CSI-RS periodicity and subframe offset TCSI-RS / ICSI-RS |  | 5/1 | 5/1 | 5/1 |
| NZP-CSI-RS-Configuration-List |  | {0,1,2} | {0,1,2} | {0,1,2,3} |
| FrequencyDensityNonPrecoded |  | 1 | 1/3 | 1/3 |
| NZP-TransmissionCombListNonprecoded |  | NA | {0,1,2} | NA |
| NZP-TransmissionCombNonprecoded |  | 0 | 0 | 1 |
| eMIMO-Type |  | Class A | Class A | Class A |
| codebookConfig-N1 |  | 3 | 3 | 4 |
| codebookConfig-N2 |  | 4 | 4 | 4 |
| codebook-Over-Sampling-RateConfig-O1 |  | 8 | 8 | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  | 4 | 4 | 4 |
| Codebook-Config |  | 1,2,3,4 | 1,2,3,4 | 1,2,3,4 |
| codebookSubsetRestriction-1 |  | 0x02/01 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF | 0x02/01 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF | 0x02/01 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  | FFS | FFS | FFS |
| Reporting mode |  | PUSCH 3-1 | PUSCH 3-1 | PUSCH 1-2 |
| Reporting interval | ms | 5 | 5 | 5 |
| PMI delay | ms | 8 | 8 | 8 |
| Measurement channel & Rank |  | Option1: 16QAM ½ Rank2 Option2: 64QAM ½ Rank1 | Option1: 16QAM ½ Rank2 Option2: 64QAM ½ Rank1 | Option1: 16QAM ½ Rank2 Option2: 64QAM ½ Rank2 |
| Max number of HARQ transmissions |  | 4 | 4 | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} | {0,1,2,3} | {0,1,2,3} |