3GPP TSG-RAN4 Meeting #84	R4-1707800

# Berlin, DE, 21st – 25th August 2017

Source: 	Qualcomm Incorporated

Title: 	A-MPR for CAT-M2

Agenda item:	7.9.2

Document for:	Approval

# Introduction

In [1] A-MPR for some of the NS cases was left as TBD. In this paper, some of these values have been proposed.

# Discussion

In general, it is our view that the existing LTE A-MPR values need to be applied for CAT-M2 as well. Hence the below table proposes the same.A-

Proposal 1:

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Narrowband bandwidth | Resources Blocks (NRB) |  | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.5-1 | 1.4, 3, 5 | Table 5.6-1 |  | N/A |
| NS_03 | 6.6.2.2.1 | 2, 4 | 3,5 | Table 6.2.4-1 |  |  |
| NS_04 | 6.6.2.2.2 | 41 | 5 | Table 6.2.4-1 |  |  |
| NS_05 | 6.6.3.3.1 | 1 | [TBD] | [TBD] |  | [TBD] |
| NS_06 | 6.6.2.2.3 | 12, 13 | 1.4, 3, 5 | Table 5.6-1 |  | N/A |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 |  | [TBD] |  |  |
| NS_08 | 6.6.3.3.3 | 19 | 1.4, 3, 5 | Table 5.6-1 |  | N/A |
| NS_09 | 6.6.3.3.4 | 21 | 1.4, 3, 5 | Table 5.6-1 |  | N/A |
| NS_10 |  | 20 | 1.4, 3, 5 | Table 5.6-1 |  | N/A |
| NS_12 | 6.6.3.3.5 | 26 | 1.4, 3, 5 | Table 6.2.4-1 |  |  |
| NS_13 | 6.6.3.3.6 | 26 | 5 | Table 6.2.4-1 |  |  |
| NS_14 | 6.6.3.3.7 | 26 | N/A | Table 5.6-1 | N/A |  |
| NS_15 | 6.6.3.3.8 | 26 | 1.4, 3, 5 | Table 5.6-1 |  |  |
| NS_16 | 6.6.3.3.9 | 27 | 3, 5 | Table 6.2.4-1 |  |  |
| NS_17 | 6.6.3.3.10 | 28 | 5 | Table 6.2.4-1 |  |  |
| NS_18 | 6.6.3.3.11 | 28 | 5 | Table 6.2.4-1 |  |  |
| NS_32 | - | - |  | - |  | - |

Proposal 2:

For B13, A-MPR values for BWs less than 10 MHz need to be further studied. The current table 6.2.4-1, lists the AMPR needed for 10MHz only.

# Conclusion

In this paper, A-MPR needed for CAT-M2 have been proposed. B13 AMPR for BWs less than 10 MHz need to be studied further.

# References

[1] CR for CAT-M2 REFSENS, MPR and adding note for protection band 5, 3GPP TSG-RAN WG1 Meeting #83, Hangzhou, China, 15-19 May 2017