3GPP TSG-RAN WG4 Meeting #84		R4-1707813

Berlin, Germany, 21- 25 August 2017

Source:	CATT

Agenda item:	7.14.3

Title:	Further evaluation of PDSCH demodulation performance on Enhanced SU-MIMO IM

Document for:	Discussion

Introduction

In RAN #73 meeting, the “LTE Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements” WI was initiated [1].

In the previous RAN4#82bis meeting, further agreements on the PDSCH target scenarios were made and summarized in [2].

- Rank 2 SU-MIMO test cases Keep TC#1 with 16QAM

And it is proposed to provide simulation results to identify target scenarios for requirements definition and confirm reference receiver feasibility.

- Rank 4 SU-MIMO test cases Select one test case from TM4/TM9 in next meeting Companies are encouraged to further check results

In this contribution, we provide RANK4 PDSCH demodulation simulation results based on the agreed simulation assumptions provided in [3]. The detailed simulation assumptions are attached in the Annex.

Simulation assumptions

In the simulation summary [3], the agreed PDSCH test cases for further evaluation are proposed as shown in Table 1.

Table 1.  Test cases for further evaluation

| Test scenario | Rank | Descriptions of other parameters |
| --- | --- | --- |
| TS #1 | Rank 4 | TM4 16QAM 0.57 EPA5 4x4 XPOL Medium A |
| TS #2 |  | TM9 16QAM 0.57 EPA5 4x4 XPOL Medium A |

Reference receivers are used for the evaluation:

- Receiver #1: L-MMSE(baseline)

- Receiver #2: R-ML

Simulation results for MIMO Rank-4 scenarios

Test Scenario #1:

TM4 16QAM 0.57 EPA5 4x4 XPOL Medium A

FDD mode                                                                                   TDD mode

Figure 1. demodulation performance of Test Scenario#1 TM4-16QAM

Test Scenario #2:

TM9 16QAM 0.57 EPA5 4x4 XPOL Medium A

FDD    mode                                                                                  TDD mode

Figure 2. demodulation performance of Test Scenario#2 TM9-16QAM

From the above simulation results of the colliding scenarios, the performance gain of the R-ML receiver to the LMMSE receiver is shown in table 2.

Table 2.  the performance gain(dB) of the SNRs @ x% of the maximum throughput

| Test Scenario | LMMSE(dB)@70% max. throughput | R-ML(dB) @70% max. throughput | Gain over LMMSE(dB) @70% max. throughput | LMMSE(dB)@85% max. throughput | R-ML(dB) @85% max. throughput | Gain over LMMSE(dB) @85% max. throughput |
| --- | --- | --- | --- | --- | --- | --- |
| TS #1 TM4 (FDD mode) | 14.16 | 12.96 | 1.2 | 16.92 | 14.74 | 2.18 |
| TS #1 TM4 (TDD mode) | 14.06 | 13.33 | 0.73 | 16.93 | 15.08 | 1.85 |
| TS #2 TM9 (FDD mode) | 19.02 | 16.2 | 2.82 | 22.08 | 17.67 | 4.41 |
| TS #2 TM9 (TDD mode) | 17.88 | 15.54 | 2.34 | 20.7 | 17.13 | 3.57 |

Observation 1: For TM9 with 16QAM scenario test case, the R-ML receiver can provide testable gain under the XPOL Medium A channel condition.

Observation 2: For TM4 with 16QAM scenario test case, it is feasible to get a testable performance gain using the 85% metric.

Proposal: Specify the PDSCH RANK4 demodulation performance requirement with the TM9 16QAM scenario.

Conclusions

In this contribution, we provide our simulation results for PDSCH demodulation further evaluation scenarios of  enhanced SU-MIMO IM . And according to the simulation results, the observations are given as below:

Observation 1: For TM9 with 16QAM scenario, the R-ML receiver can provide testable gain under the XPOL Medium A channel condition.

Observation 2: For TM4 with 16QAM scenario test case, it is feasible to get a testable performance gain using the 85% metric.

Proposal: Specify the PDSCH RANK4 demodulation performance requirement with the TM9 16QAM scenario.

# References

- RP-161877 “New Work Item Proposal: Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements for LTE”, Intel Corporation, Huawei, LGE, RAN #73, September 2016

- R4-1706168  “WF on enhanced SU-MIMO receiver performance requirements”, Huawei, HiSilicon, RAN4 #83, May 2017

- R4-1704942  “Summary of simulation results for enhanced SU-MIMO”, LGE, RAN4 #83, May 2017

# Annex-simulation assumptions

- Common test parameters (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Duplex mode |  | FDD |
| Number of control OFDM symbols |  | 2 |
| Number of HARQ processes per component carrier | Processes | 8 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK and 16QAM |
|  |  | {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH | OFDM symbols | 2 for 10 MHz |
| PMI |  | PUSCH 3-1 for TM4/TM9 |
|  |  | Follow wideband PMI feedback for TM4/TM9 |
| CSI-RS ports |  | port 15, 16 for 2 Tx antennas port 15,…18 for 4 Tx antennas (port 0,1 for CRS) |
| CSI-RS periodicity and subframe offset TCSI-RS / ΔCSI-RS (for TM9) | Subframe | 5 / 2 |
| CSI reference signal configuration (for TM9) |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap (for TM9) |  | 3 / 0010000000000000 |
| Cyclic Prefix |  | Normal |
| Cell_ID |  | 0 |
| Tx EVM |  | 6% for 16QAM and 64QAM; |
|  |  | 3% for 256QAM |

- Common test parameters (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Duplex mode |  | TDD |
| Uplink-Downlink Configuration |  | 1 |
| Special Subframe Configuration |  | 4 |
| Number of control OFDM symbols |  | 2 |
| Number of HARQ processes per component carrier | Processes | 7 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK and 16QAM |
|  |  | {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH | OFDM symbols | 2 for 10 MHz |
| PMI |  | PUSCH 3-1 for TM4/TM9 |
|  |  | Follow wideband PMI feedback for TM4/TM9 |
| CSI-RS ports |  | port 15, 16 for 2 Tx antennas port 15,…18 for 4 Tx antennas (port 0,1 for CRS) |
| CSI-RS periodicity and subframe offset TCSI-RS / ΔCSI-RS (for TM9) | Subframe | 5 / 4 |
| CSI reference signal configuration (for TM9) |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap (for TM9) |  | 3 / 0000000000000000 |
| Cyclic Prefix |  | Normal |
| Cell_ID |  | 0 |
| Tx EVM |  | 6% for 16QAM and 64QAM; |
|  |  | 3% for 256QAM |
| Reference receiver |  | R-ML, interesting companies are encouraged to provide full ML results |

- Antenna correlation for evaluation：

|  | Antenna Correlation |
| --- | --- |
| AC #1 | ULA Low correlation |
| AC #2 | ULA Medium correlation (α=0.3, β=0.9) |
| AC #3 | ULA Medium A correlation (α=0.3, β=0.3874) |
| AC #4 | XPOL Medium A correlation (α=0.3, β=0.6, γ=0.2) |

- FRC of test cases for performance requirements:

|  |  | Test case for performance requirements |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| FRC for FDD |  | Rank2 |  | Rank3 | Rank4 |  |
|  |  | TC#1 | TC#2 | TC#3 | TS#1 | TS#2 |
| Parameter | Unit | TM4 16QAM | TM9 16QAM | TM3 16QAM | TM4 16QAM | TM9 16QAM |
| Reference channel |  |  |  |  |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 | 10 |
| Allocated resource blocks (Note 3) |  | 50 | 50 | 50 | 50 | 50 |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 | 9 | 9 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 1/2 | 1/2 | 0.57 |
| Information Bit Payload |  |  |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 12960 | 11448 | 12960(CW0) 25456(CW1) | 25456 | 25456 |
| For Sub-Frames 2,3,7,8 | Bits | 12960 | 11448 | 12960(CW0) 25456(CW1) | 25456 | 25456 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 12960 | 9528 | 11448(CW0) 22920(CW1) | 22920 | 21384 |
| Number of Code Blocks (Note 4) |  |  |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Code blocks | 3 | 2 | 3(CW0) 5(CW1) | 5 | 5 |
| For Sub-Frames 2,3,7,8 | Code blocks | 3 | 2 | 3(CW0) 6(CW1) | 5 | 5 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 3 | 2 | 2(CW0) 4(CW1) | 4 | 4 |
| Binary Channel Bits |  |  |  |  |  |  |
| For Sub-Frames 1,6 | Bits | 26400 | 24000 | 25600(CW0) 51200(CW1) | 51200 | 43200 |
| For Sub-Frames 4,9 | Bits | 26400 | 24000 | 25600(CW0) 51200(CW1) | 51200 | 43200 |
| For Sub-Frames 2,7 | Bits | 26400 | 23600 | 25600(CW0) 51200(CW1) | 51200 | 41600 |
| For Sub-Frames 3,8 | Bits | 26400 | 23200 | 25600(CW0) 51200(CW1) | 51200 | 41600 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 24768 | 19680 | 24064(CW0) 48128(CW1) | 48128 | 35424 |
| Max. Throughput averaged over 1 frame | Mbps | 11.664 | 10.1112 | 11.513(CW0) 22.657(CW1) | 22.657 | 22.503 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211. Note 3: For TM9, 50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit).. |

| FRC for TDD |  | Rank2 |  | Rank3 | Rank4 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | TC#1-TDD | TC#2-TDD | TC#3-TDD | TS#1 | TS#2 |
| Parameter | Unit | TM4 16QAM | TM9 16QAM | TM3 16QAM | TM4 16QAM | TM9 16QAM |
| Reference channel |  |  |  |  |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 5 | 50 | 50 | 50 |
| Uplink-Downlink Configuration (Note 3) |  | 1 | 1 | 1 | 1 | 1 |
| Special Subframe Configuration (Note 3) |  | 4 | 4 | 4 | 4 | 4 |
| Allocated subframes per Radio Frame (D+S) |  | 2+2 | 3+2 | 2+2 | 2+2 | 3+2 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 1/2 | 0.5 | 0.57 |
| Information Bit Payload |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 12960 | 11448 | 12960(CW0) 25456(CW1) | 25456 | 25456 |
| For Sub-Frames 1,6 |  | 9528 | 7736 | 9528(CW0) 19080(CW1) | 19080 | 16992 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | 9528 | N/A | N/A | 21384 |
| Number of Code Blocks (Notes 4) |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  | 3 | 2 | 3(CW0) 5(CW1) | 5 | 5 |
| For Sub-Frames 1,6 |  | 2 | 2 | 2(CW0) 4(CW1) | 4 | 3 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | N/A | 2 | N/A | N/A | 4 |
| Binary Channel Bits |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 26400 | 23600 | 25600(CW0) 51200(CW1) | 51200 | 40000 |
| For Sub-Frames 1,6 |  | 21312 | 15744 | 20512(CW0) 41024(CW1) | 41024 | 27552 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |  |
| For Sub-Frame 0 | Bits | N/A | 19680 | N/A | N/A | 35424 |
| Max. Throughput averaged over 1 frame | Mbps | 4.498 | 4.79 | 4.498(CW0) 8.907(CW1) | 8.907 | 10.628 |
| UE Category |  | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 5 | ≥ 5 |