3GPP TSG-RAN4 Meeting #84 	R4- 1708456

Berlin, Germany; 21 – 25 August 2017

Source: 	Huawei, Hisilicon

Title: 	TP for TR 36.753: UE reference sensitivity power level

Agenda Item:	8.14.1

Document for:	Approval

# Introduction

A work item [1] was agreed to define a new TDD band covering 1427 – 1518 MHz.

This contribution is to provide discussion and text proposal on UE reference sensitivity power level.

# Discussion

## Background

The frequency band 1427-1518 MHz was identified for IMT in WRC-15, named as L-band.

Based on the approval at the 3GPP RAN4 meeting#83 in Hangzhou, China, May 15 to 19 2017, the TDD L-Band arrangement is aligned on the SDL L-Band arrangement. Thus, there is only a TDD L-band challenging arrangements divided in two bands, as follow:

Figure 1 - TDD Arrangement (a)

## UE reference sensitivity power level

In the TR 36.892, there is the paragraph 8.2:

The 1.4GHz band is a TDD band without simultaneous Rx/Tx hence no Rx/Tx isolation requirement is needed. The bandwidth is 20MHz and BW/F0 equals to 1.37% which will not bring difficulty for implementation. The reported insertion loss values from different filter vendors are shown in Table 8.2-1. The proposed reference sensitivity is shown in Table 8.2-2.

Table 8.2-1: IL values for

|  | Typ. IL (dB) | ETC. IL (dB) |
| --- | --- | --- |
| Vendor 1 | 1.2 | 1.9 |
| Vendor 2 | 1.6 | 2.1 |

Table 8.2-2: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 45 |  |  | -100 | -97 | -95.2 | -94 | TDD |

And also the contribution “R4-1703068” [7]:

Table 1: Insertion Loss

|  | Typ. IL (dB) | Note |
| --- | --- | --- |
| Vendor 1 | 1.37 |  |
| Vendor 2 | 2.4 | with 2016 technologies |
|  | 1.6 | with 2018 technologies |

The band plan for TDD L band and SDL L band is the same. Hence it is foreseen the receiver filter performance will be similar. The insertion loss presented in [7] is similar as that for band 45. It is proposed that REFSENS in Table 2 defined for band 50 and 51.

Table 2: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 50 | - | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 51 |  | -102.2 | -100 |  |  |  |  |

# Conclusion

It is proposed to approve the text proposal for TR 36.753.

# Text proposal for TR 36.753

<Start of TP>

## 8.x    UE sensitivity for E-UTRA

The band plan for TDD L band and SDL L band is the same. Hence it is foreseen the receiver filter performance and reference sensitivity will be similar.

The reference sensitivity is specified in Table 8.3-1 for Band 50 and Band 51.

Table 8.3-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 50 | - | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 51 |  | -102.2 | -100 |  |  |  |  |

<End of TP>

# References

[1] RP-162557 “New E-UTRAN L-band plan 1427-1518MHz”, Etisalat, Huawei, HiSilicon

[2] R4-1704099_WF on UE and BS requirements in FDD L-band

[3] ECC Report 263, “Adjacent band compatibility studies between IMT operating in the frequency band 1492-1518 MHz and the MSS operating in the frequency band 1518-1525 MHz”;

[4] ITU-R Report M.2292 “Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analyses”

[5] R4-1704098- Band plan discussion 1 5 GHz TDD

[6] TR 36.892 - 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Introduction of 1447-1467MHz Band for TD-LTE in China

[7] R4-1703068 - UE reference sensitivity