3GPP TSG-RAN4 Meeting #84 	R4-1708458

Berlin, Germany; 21 – 25 August 2017

Source: 	Huawei, Hisilicon

Title: 	TP for TR 36.753 Channel numbering of B50 &B51

Agenda Item:	8.14.1

Document for:	Approval

# Introduction

A work item [1] was agreed to define a new TDD band covering 1427 – 1518 MHz.

This contribution is to provide the TP about the channel numbering of B50 & B51.

# Text proposal for TR 36.753

<Unchanged sections omitted>

# 10	Channel numbering for E-UTRA, MSR

For 1427-1517MHz Band the E-UTRA frequency band is defined as in Table 10-1.

Table 10-1 E-UTRA frequency bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 50 | 1432 MHz | – | 1517 MHz | 1432 MHz | – | 1517 MHz | TDD |
| 51 | 1427 MHz | – | 1432 MHz | 1427 MHz | – | 1432 MHz | TDD |

EARFCN for 1427-1517MHz Band is defined as in Table 10-2.

Table 10-2: EARFCN allocated for E-UTRA Band 50 & 51

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low [MHz] | NOffs-DL | Range of NDL | FUL_low [MHz] | NOffs-UL | Range of NUL |
| 50 | 1432 | 58240 | 58240 - 59089 | 1432 | 58240 | 58240 - 59089 |
| 51 | 1427 | 59090 | 59090 - 59139 | 1427 | 59090 | 59090 - 59139 |

<Unchanged sections omitted>

# Conclusion

Proposal 1: Add this text above to TDD LBand TR 36.753.

.