3GPP TSG-RAN WG4 Meeting #84	R4-1708742

Berlin, Germany, 21 – 25 August, 2017

Source:	Sprint

Title:	TP for Rel-15 Intra-band 36.715-00-00 for CA_5DL_41F_2UL_41C_BCS0

Agenda item:	8.1.2

Document for:	Approval

# 1. Introduction

This contribution provides a TP for TR 36.715-00-00 to include the requirements for the Intra-band CA combination CA_5DL_41F_2UL_41C_BCS0 as defined in RP-170962 [1].

# 2. Text Proposal

##### ---Start of changes---

## 6.x	CA_5DL_41F_2UL_41C_BCS0

### 6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  |  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_41F | CA_41C | 10, 15, 20 |  | 15, 20 |  | 20 | 20 | 20 | 100 | 0 |

### 6.x.2		Co-existence studies

Co-existence studies are not required since this combination is similar to existing combination CA_41E/41C.

### 6.x.3		∆TIB and ∆RIB values

TIB and RIB does not need to be defined for this combination.

### 6.x.4		REFSENS requirements

REFSENS requirements do not need to be defined for this combination.

### 6.x.x		Blocking requirements

In-band blocking and out-of-band blocking need to be included in  TS 36.101 according to Table 6.x.x-1 and Table 6.x.x-2.

Table 6.x.x-1: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_41F | FInterferer (Range) | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |

Table 6.x.x-2: Out of band blocking

| CA configuration | Parameter | Units | Frequency |
| --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_41F | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for this CA configuration for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |

##### ---End of changes---

# Reference

[1]	 RP-170962, “New WID: LTE Advanced intra-band CA Rel-15 for xDL/yUL including contiguous and non-contiguous spectrum”, RAN#76, Ericsson