3GPP TSG-RAN WG4 Meeting #84bis	R4-1710793

Dubrovnik, Croatia, 9 – 13 October 2017

Agenda Item:	6.2.4.1

Source:	Ericsson

Title:	Simulation results of PDSCH for Rel-14 BL/CE UE

Document for:	Discussion

# 1	Introduction

RAN4#84 agreed with the repetition numbers for PDSCH demodulation requirements for UE DL Cat-M2 as follows [1]:

- Repetition number for CE Mode A: 8

- Repetition number for CE Mode B: 32

Companies are encouraged to provide the ideal simulation result assuming 50 Hz frequency offset for CE Mode A and 100 Hz for CE Mode B.

This contribution discusses the repetition number for PDSCH CE Mode B and necessary MPDDCH repetition number used for the PDSCH demodulation requirements.

# 2	Simulation results

## 2.1	CE Mode A

Figure 1 shows the PDSCH simulation results for CE Mode A for FDD and TDD. The simulation parameter for PDSCH is given in [3].

Figure 1	PDSCH relative throughput for Cat-M2 CE Mode A (FDD and TDD).

## 2.2	CE Mode B

Figure 2 show the PDSCH simulation results for CE Mode B for FDD and TDD. The simulation parameter for PDSCH is given in [3].

Figure 2	PDSCH relative throughput for Cat-M2 CE Mode B (FDD and TDD).

## 2.3	Summary

Table 1 summarizes the simulation results for Cat-M2 PDSCH. This table also provides the impairment results used to specify the Cat-M2 PDSCH demodulation requirements.

Table 1	Summary of simulation results for Cat-M2 PDSCH.

| CE Mode | Duplex mode | Ideal results | Impairment results |
| --- | --- | --- | --- |
| CE Mode A | FDD | -8.0 | -6.0 |
|  | TDD | -8.4 | -6.4 |
| CE Mode B | FDD | -15.5 | -13.5 |
|  | TDD | -15.5 | -13.5 |

# 3	Frequency hopping interval parameters

In the simulation assumption [3], we assumed

- Interval-FDD = 8ms for CE Mode A FDD

- Interval-TDD = 10ms for CE Mode A TDD

- Interval-FDD = 16ms for CE Mode B FDD

- Interval-TDD = 20ms for CE Mode B TDD

We have agreed to set 8 repetitions for Cat-M2 CE Mode A FDD/TDD test cases with 2 wideband frequency hopping. However, the existing interval setting cannot verify the frequency hopping for PDSCH. We therefore to set the interval parameter as follows. Note we can keep the interval parameters for CE Mode B.

Proposal 1: Set the frequency hopping interval parameters as follows.

|  | CE Mode A | CE Mode B |
| --- | --- | --- |
| interval-FDD | 4 | 16 |
| interval-TDD | 5 | 20 |

# 4	MPDCCH parameter for PDSCH test

For UE Cat-M2 PDSCH with coverage enhancement, we should consider the MPDCCH repetition number to avoid MPDDCH decoding failure. Figure 3 and Figure 4 is the copy from our previous contribution [2]. The parameter for MPDCCH is same as TS36.101 8.11.2 except aggregation level. In MPDCCH demodulation requirement for CE Mode A, the AL is set to 16, but for PDSCH demodulation, MPDCCH AL is set to 24 to achieve higher decoding reliability. From Table 1, the required SNR for PDSCH to achieve 70% of maximum throughput is -8.0dB and -8.4dB for CE Mode A FDD and CE Mode A TDD. From the Figure 3, MPDCCH repetition can be set to 16 considering 3dB power boosting for both FDD and TDD.

Figure 3	PDSCH relative throughput and MPDCCH BLER for Cat-M2 CE Mode A [2].

For CE Mode B, from Table 1, the required SNR for PDSCH to achieve 70% of maximum throughput is -15.5dB and -15.5dB for CE Mode A FDD and CE Mode A TDD, respectively. From Figure 4, MPDCCH repetition can be set to 64 for FDD and 32 for TDD considering 3dB power boosting.

Figure 4	PDSCH relative throughput and MPDCCH BLER for Cat-M2 CE Mode B [2].

Proposal 2: Set the MPDCCH repetition as shown in the table below during PDSCH test. Also set AL24 and apply 3dB power boosting for MPDDCH transmission.

|  | CE Mode A | CE Mode B |
| --- | --- | --- |
| MPDDCH repetitions for FDD | 16 | 64 |
| MPDDCH repetitions for TDD | 16 | 32 |

# 5	Conclusion

Proposal 1: Set the frequency hopping interval parameters as follows.

|  | CE Mode A | CE Mode B |
| --- | --- | --- |
| interval-FDD | 4 | 16 |
| interval-TDD | 5 | 20 |

Proposal 2: Set the MPDCCH repetition as shown in the table below during PDSCH test. Also set AL24 and apply 3dB power boosting for MPDDCH transmission.

|  | CE Mode A | CE Mode B |
| --- | --- | --- |
| MPDDCH repetitions for FDD | 16 | 64 |
| MPDDCH repetitions for TDD | 16 | 32 |

# 6	References

R4-1708702, “Way forward on UE demodulation requirement for Rel-14 FeMTC”, Ericsson, Intel, Qualcomm, Samsung.

R4-1707671, “PDSCH demodulation requirements for UE DL Cat-M2”, Ericsson.

R4-1704179, “Simulation assumption of FeMTC for UE demodulation requirements”, Ericsson, Huawei, HiSilicon, Intel, Qualcomm.