3GPP TSG RAN WG4 Meeting #84Bis	R4-1710121

Dubrovnik, Croatia, 09 - 13 October, 2017

3GPP TSG RAN WG1 Meeting #90		R1-1715335

Czechia, 21th – 25th August 2017

Title:	Corrections on antenna switching

# Response to:

# Release:	Rel-13

Work Item:	LTE_CA_TDD_FDD-Core

Source:	RAN1

To:	RAN2, RAN4

# Contact Person:

# Name:	Alberto Rico

E-mail Address:	albertor@qti.qualcomm.com

Attachments:	R1-1715336 Corrections for antenna switching

# 1. Overall Description:

RAN1 discussed the issue of antenna selection for the case of carrier aggregation. RAN1 agreed in principle to the attached CR (pending details on RAN2 signaling), and reached the following agreements:

Agreements:

- For a UE that supports Tx antenna selection in TDD but not in FDD, the UE accepts the RRC message configuring Tx antenna selection in TDD but not in FDD, but does not expect to receive a DCI indicating UE port 1 for the UL carrier not supporting Tx antenna selection.

- For each band combination, signal which bands support Tx antenna selection. For each of the bands that support Tx antenna selection:

- For each uplink band, signal all the bands for which UL switches together (thus same port has to be enforced)

- For each uplink band, signal all the bands for which DL switches together (thus introducing a ‘glitch’ in the DL reception)

# 2. Actions to RAN2, RAN4:

RAN1 respectfully requests RAN2:

- For agreement 1, to amend the text “EUTRAN configures the same value for all serving cells” in the field description of ue-TransmitAntennaSelection in TS36.331.

- For agreement 2, to implement the above signalling from Rel-13.

RAN1 respectfully requests RAN4 to take the above information into account, and assess whether changes to the RAN4 specifications are needed to incorporate the notion of glitches in the above context.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #90-Bis	Oct. 9-13,  2017 		      Prague (CZ)

TSG RAN WG1 Meeting #91	Nov. 27-Dec. 01,  2017 	      Reno, NV (USA)