3GPP TSG-RAN WG4 Meeting #84-Bis	R4-1710430
Dubrovnik, Croatia, 09 – 13 October, 2017

Agenda item:	9.4.3.2.2

Source:	Intel Corporation, Apple, Samsung

Title:	On Spherical Coverage: EIRP CDF data for mm-Wave

Document for:	Approval

# 1	Introduction

During the RAN #75 meeting the study item on New Radio access technology [1] was finalized with its outcome captured in TR38.803 [2]. During RAN #76 the New Radio Work Item was initiated [3]. With the agreement to define and test RF requirements for NR mm-wave over the air (OTA), the discussion of how to define UE power class and represent the UE’s spherical coverage has progressed over several agreements.

This paper focuses on summarizing the CDF contributions presented by companies in the previous two meetings. The observed trends in the reported results help highlight the need to align on the framework and simulation assumptions used in CDF calculations.

# 2	Discussion

## 2.1	Background

The existing RAN4 agreements on UE power class definition include the NR SI outcome in TR38.803 [2], the WF from the WF from RAN4 #83 [5], and the WF from RAN4 #84 [6]. The latter narrows down a three CDF percentile ranges.

From RAN4 #83 [5]:

- Both TRP and EIRP are considered for power class definition

  - EIRP can be peak, boresight or %-tile or minimum value

  - TRP can be max or min and max

- Max allowed EIRP for all UEs is 43 dBm for regulatory reasons

- Companies are encouraged to provide input

  - Feasible definition and values for spherical coverage

From RAN4 #84 [6]:

Agreement:

To agree on Power class definition, the following candidates are to be considered

- Peak EIRP over sphere with minus tolerance

- EIRP at CDF percentiles with minus tolerance

  - [5-20]%

  - [50]%

  - [80-90]%

In last month’s RAN4 AH #3 meeting [7], a way forward for power class requirements for EIRP and spherical coverage was approved. Below are the main points on spherical coverage.

As stipulated above, more aspects of real-product implementations need to be included in simulation analysis. These aspects significantly impact the attained EIRP results, and thus should be agreed on as framework for CDF calculations. We will discuss a summary of the reported CDF results and their specific assumptions, in hopes to align these results across companies. This is needed before any agreements on EIRP mask and percentile points can be made.

## 2.2	Survey of previously presented results

Several parameters are relevant to CDF simulations. Environment conditions, such as enclosing material, affect final antenna performance. The number of arrays in the UE and their specific placement is also important. The table below summarizes the results and simulation assumptions of contributions presented in RAN4 #84 [8-14].

Table 1: Reported results for RAN4 #84

|  | Source |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Intel | MediaTek | Qualcomm | LG | Huawei | Sony |  | Docomo |
| # antenna arrays in DUT | FFS | no data | no data | no data | no data | UE1=2 | UE2=3 | 2 |
| enclosure around antennas | no data | no data | case loss | cover loss | cover loss | case loss |  | yes |
| materials considered | no data | no data | no data | no data | glass/plastic | no data4 |  | metal/plastic |
| PA output [dBm] | 14 | 9 | 14 | 173 | 14 | no data |  | 17-19 |
| spatial coverage of CDF | no data | no data | around the sphere | over all angles | no data | no data |  | no data |
| 50 percentile value [dBm] | 26.62 | 181 | 17.1-28.92 | 20.7 | not listed | 26.5 | 27.5 | 26-30 |
| Notes: 1 Separate classes for fixed and mobile, value reflects mobile 4 Relates to TRP (rather than conducted) 2 Range for worst to best case scenario based on losses 3 Max conducted power |  |  |  |  |  |  |  |  |

Starting with the number of arrays found in the UE, only Sony and DOCOMO provide any information. Whether the antennas are enclosed is not explicitly stated in most papers, though is often found in a loss parameter (cover or case loss) or implementation (those listed as “no data”). However, this does not mean it was included in the simulation. The specific materials used in the simulations are not typically discussed either. Similarly, whenever materials are mentioned, it is in context of their presence in a loss parameter, not necessarily if they were used in the simulation environment.

From the results in the Table 1, we see significant variability in the PA output power, which is then reflected in the final CDF profile. For power class definition, details such as fixed or mobile, dual or single polarization, impact the results and thus must be reported. Also, for the majority of the contributions, how the CDF spatial coverage was approached is not discussed. The final results will change if focus is only on the main beamwidth, as opposed to the entire sphere. Regardless, the overall trend observed for the 50%-tile point is around 26 dBm.

Observation 1: To avoid greater variability in reported results, PA output power should be reported and agreed upon.

Observation 2: All CDF simulations should assume similar parameters. Alignment on these parameters is vital for proper comparison across reported results.

Table 2: Reported results for RAN4 AH #3

|  | Source |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Intel | MediaTek | Qualcomm | LG | Huawei | Sony |  | Docomo |
| # antenna arrays in DUT | FFS | 1 | no data | 3 | no data | UE1=2 | UE2=3 | 2 |
| enclosure around antennas | no data | case loss | plastic loss | case loss | cover loss | case loss |  | case loss |
| material considered | no data | no data | Plastic | no data | plastic/glass | no data 2 |  | no data |
| PA output [dBm] | 14 | 9 1 | 14 | 17 | 13-14 | no data |  | 13-14 |
| spatial coverage of CDF | conical ±60° | no data | no data | over sphere | no data | no data |  | no data |
| 50 percentile value [dBm] | 23 | 23 | 23.7 | 24.8 | other points listed (20,90) | 22 | 25 | 23-27 |
| Notes: 1 Pout per element 2 Relates to TRP (rather than conducted) |  |  |  |  |  |  |  |  |

Table 2 presents a summary of the results reported in RAN4 AH#3 [15-21]. Similar to what is shown on table 1, not much is disclosed in terms of the number of arrays used, enclosing material and spatial coverage. It is worth noting that, overall, the observed trend is that the 50%-tile value has decreased. For most, this highlights the addition of more loss parameters and further detailing their granularity. Given the lack of important environment information, undisclosed assumptions and unspecified spatial coverage in simulations, it is hard to discuss and agree on which CDF percentile points are useful for real implementations. Therefore, a framework should be considered first. A clearer understanding of the implementation and environment setup will help companies align on all parameters before deciding on CDF points.

Proposal 1: In order to correctly compare results across companies, a framework to align on CDF simulation assumptions is needed.

Table 3 below proposes a list of parameters which may potentially define the framework.

Table 3: Parameters for EIRP CDF alignment

| Parameter | Value |
| --- | --- |
| DUT description | information UE enclosure and array placements is encouraged to be reported |
| CDF type | measured or simulated |
| Maximum EIRP (dBm) | the 100% percentile of the CDF should correspond to the company’s peak EIRP proposal or should match the agreed max EIRP in RAN4 (if available) |
| Spatial coverage CDF | over all angles (elevation & azimuth) |
| Spherical surface grid | Companies are encouraged to describe the spherical surface grid that was used to enable comparisons [10]. |
| 50 percentile value (dBm) | companies are encouraged to share the 50% value; sharing other values is not precluded |

Proposal 2: Agreement on the framework parameters is necessary to determine whether companies can achieve sufficient alignment in their assumptions to derive a spatial coverage EIRP CDF requirement. Additional parameters, and editing or improvement of the existing parameters is encouraged by the authors.

Proposal 3: As a fallback option, if a framework cannot be agreed, the power class requirement should focus on defining the peak EIRP value and consider deriving the spatial coverage EIRP CDF requirement from measurements once sufficient numbers of commercially-available UEs can be used.

# 3	Conclusions

This paper presented a summary of previously reported power class parameters and CDF values. We discussed our views on how to approach defining the framework to align assumptions for spherical coverage simulations needed to define power class for mm-wave. The following observations and proposals have been made:

Observation 1: To avoid greater variability in reported results, PA output power should be reported and agreed upon.

Observation 2: All CDF simulations should assume similar parameters. Alignment on these parameters is vital for proper comparison across reported results.

Proposal 1: In order to correctly compare results across companies, a framework to align on CDF simulation assumptions is needed.

Proposal 2: Agreement on the framework parameters is necessary to determine whether companies can achieve sufficient alignment in their assumptions to derive a spatial coverage EIRP CDF requirement. Additional parameters or improvement of the existing parameters is encouraged by the authors.

Proposal 3: As a fallback option, if a framework cannot be agreed, the power class requirement should focus on defining the peak EIRP value and consider deriving the spatial coverage EIRP CDF requirement from measurements once sufficient numbers of commercially-available UEs can be used.

# 4	References

- RP-161596, “Study on New Radio Access Technology,” NTT DOCOMO Inc., 3GPP RAN #73, September 2016

- TR 38.803, “Study on New Radio Access Technology RF and co-existence aspects,” Version 2.0.0, 3GPP RAN #75, March 2017

- RP-170855, “Work Item on New Radio (NR) Access Technology,” NTT DOCOMO Inc., RAN #75, March 2017

- R4-1704362, “WF on mm-wave UE power class,” Qualcomm, Ericsson, Sony, Skyworks, 3GPP RAN4 #82bis, April 2017

- R4-1706322, “Way Forward on mmW power class,” Qualcomm, Ericsson, Sony, 3GPP RAN4 #83, May 2017

- R4-1708914, “WF on Power Class and CDF,” NTT DOCOMO, 3GPP RAN #84, August 2017

- R4-1710081, “WF on Power Class for mm-Wave,” Qualcomm, Intel, LGE, 3GPP NR AH#3, September 2017

- R4-1708618, “UE Power Class for mm-wave,” Intel Corp., 3GPP RAN4 #84, August 2017

- R4-1707818, “mmW UE power class,” MediaTek, 3GPP RAN4 #84, August 2017

- R4-1708609, “mmW Output Power,” Qualcomm, 3GPP RAN4 #84, August 2017

- R4-1708162, “NR UE power class for mmWave UE,” LG, 3GPP RAN4 #84, August 2017

- R4-1707825, “Preliminary smartphone EIRP performance evaluation,”  Huawei, HiSilicon, RAN4 #84, Aug. 2017

- R4-1707331, “UE Power class for mmWave 28GHz,” Sony, 3GPP RAN4 #84, August 2017

- R4-1707567, “EIRP assumption for mmWave,” NTT DOCOMO Inc., 3GPP RAN4 #84, August 2017

- R4-1709403, “On Spherical Coverage: EIRP CDF data for mm-wave,” Intel Corp., 3GPP NR AH#3, Sept. 2017

- R4-1709445, “mmWave UE power class for 28 GHz band,” MediaTek, 3GPP NR AH#3, September 2017

- R4-1709835, “Output power values for mmW,” Qualcomm, 3GPP NR AH#3, September 2017

- R4-1709575, “NR UE power class for mmWave,” LG, 3GPP NR AH#3, September 2017

- R4-1709539, “mmWave power class and power control,” Huawei, HiSilicon, 3GPP NR AH#3, September 2017

- R4-1709750, “UE Power class for mmWave 28GHz,” Sony, 3GPP NR AH#3, September 2017

- R4-1709394, “EIRP CDF for mmWave UE,” NTT DOCOMO Inc., 3GPP NR AH#3, September 2017