3GPP TSG-RAN WG4 Meeting #84bis	R4-1710847

Dubrovnik, Croatia, 9 – 13 October 2017

Source:	Ericsson, AT&T

Title:	TP to TR 36.715-03-01: CA_3DL_2A-14A-66A_1UL_BCS0

Agenda item:	8.3.2

Document for:	Approval

# 1. Introduction

This contribution provides a TP for TR 36.715-03-01 to include the requirements for the 3DL/1UL CA combination CA_3DL_2A-14A-66A_1UL_BCS0 as defined in RP-171749 [1].

# 2. Text Proposal for TR 36.715-03-01

##### ---Start of changes---

## 5.x	CA_3DL_2A-14A-66A_1UL_BCS0

### 5.x.1	Operating bands for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-14-66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 5.x.2	Channel bandwidths per operating band for CA

Table 5.x.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_2A-14A-66A | 2 |  |  |  | Yes |  | Yes | Yes | Yes | 50 | 0 |
|  | 14 |  |  |  |  | Yes | Yes |  |  |  |  |
|  | 66 |  |  |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.x.3	Co-existence studies

Table 5.x.3-3 summarizes frequency ranges where harmonics occur due to Band 2, Band 5 and Band 46 CA with 1 UL.

Table 5.x.3-3: Impact of 1 UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 |
| 14 | 788 | 798 | 758 | 768 | 1576 | 1596 | 2364 | 2394 |
| 66 | 1710 | 1780 | 2210 | 2200 | 3420 | 3560 | 5130 | 5340 |

It can be seen from Table 5.x.3-3 for CA_2A-14A-66A that there is no harmonic interference from uplink bands towards downlink bands.

### 5.x.4	ΔTIB,c and ΔRIB,c values

Since Band 66 is close to Band 4 and Band 14 is close to Band 13, the ΔTIB,c values are derived from existing combination CA_2A-4A-13A and can be seen in table 5.x.4-1.

Table 5.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-14A-66A | 2 | 0.5 |
|  | 14 | 0.5 |
|  | 66 | 0.3 |

Since Band 66 is close to Band 4 and Band 14 is close to Band 13, the ΔRIB,c values are derived from existing combination CA_2A-4A-13A and can be seen in table 5.x.4-2.

Table 5.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2A-14A-66A | 2 | 0.3 |
|  | 14 | 0.3 |
|  | 66 | 0 |

### 5.x.5	 MSD

No MSD values need to be defined for this combination.

---End of changes---

# Reference

[1]	RP-171749, “Revised WID: LTE Advanced inter-band CA Rel-15 for 3DL/1UL”, RAN#77, Huawei, HiSilicon