3GPP TSG-RAN WG4 #84b	R4-1710865

# Dubrovnik, Croatia, 9th-13thOctober, 2017

Title:	Reply LS to R1-1709244 LS on the Power Splitting across Different TTI Lengths in UL

Release:	15

Work Item:	LTE_sTTIandPT

Source:	RAN4

To:	RAN1

Cc:	-

Contact Person:

#### Name:	Dominique Everaere

E-mail Address:	dominique.everaere@ericsson.com

1. Overall Description:

RAN4 thanks RAN1 for the LS on the Power Splitting across Different TTI Lengths in UL.

Note that RAN4 has similar discussion when considering Pcmax specification for CA and sTTI.

To guaranty phase continuity over a certain Tx time window, UE shall not change its power transmission settings over that Tx time window.

For intra-band CA with different length TTIs, phase continuity cannot be maintained with the mechanisms available today irrespective of the TTIs belonging to the same or different PUCCH groups. It’s not possible then to guarantee phase continuity and constant power in any of the proposed scenarios.

However, intra-band CA among TTIs of same length and TAG can be supported and phase continuity for each of the TTI could be maintained.

For inter-band CA with different TTI lengths, with proper evaluation of  Pc,max  ,specifying proper dropping and power reservation rules, it would be possible to maintain constant power and guarantee phase continuity in the proposed scenarios (1ms TTI and sTTI in different PUCCH groups, different sTTI lengths in different PUCCH group and 1ms TTI and sTTI in different CCs within a PUCCH group).

2. Actions:

ACTION: 	RAN4 respectfully asks RAN1 to consider above feedback on the design of shortened TTI feature.

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN4 Meeting #85     27 Nov – 1 Dec 2017	Reno, USA

TSG-RAN4 Meeting #86     22 Jan – 26 Jan 2018	Athens, Greece